WHATS-UP.DOC Release 1.40 Page 1

WHATS-UP (c) Joe Kasser, G3ZCZ, 1991-1996

_______

Joe Kasser G3ZCZ/W3      ____|__     |                  (tm)

POB 3419              --|       |    |-------------------

Silver Spring, Md. 20918        |   ____|__  |  Association of

Voice (301) 593 6136          |  |       |_|  Shareware

Data BBS (301) 593 9067          |__|   o   |    Professionals

Internet jkasser@capaccess.org  -----|   |   |---------------------

|___|___|    MEMBER

The program is distributed as a Shareware product. You may freely

copy and share the product for non commercial use, with your

friends, classmates, associates and radio hams. If you decide to use

the product, you are asked to become a registered user by completing

the registration form and sending it, and $35.00 (drawn on a US Bank

please) or equivalent in foreign currency to the author. Mastercard

and Visa are accepted if more convenient.

Upon receipt of your registration, you will receive one free update

disk, telephone and mail (electronic and regular) support. Please do

not use packet radio mail for commercial messages, or even those

that may be construed as such by individual SYSOPS through whose

BBSs the messages may pass.

This product may not be sold or distributed with another product

without the express written permission of Joe Kasser, G3ZCZ. Joe

Kasser, G3ZCZ will only support unmodified copies of this software.

Your comments and suggestions for changes are however welcome. If

you are the first to suggest a change that is implemented, you will

be sent a complimentary copy of the disk with the change

incorporated.

European Radio Amateur users may register (24.00 Pounds Sterling,

VAT) and obtain support from Terry Dansey at ReadyCrest Ltd., PO Box

75, Chatham, Kent, ME5 9DL, England. ReadyCrest Ltd., accepts credit

cards (Access, Visa, MasterCard and Eurocard). Telephones: Voice 44

(0)634-687168, FAX 44 (0)634-687178, Data (BBS) 44 (0)634-200931.

Potential Commercial and Educational Institution Users please

contact Joe Kasser directly for modifications and/or details of Site

licensing.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 2

Table of Contents

1.0 Introduction .......................................1

1.1 Capabilities of WHATS-UP ..........................3

1.2 Program Requirements ..............................4

1.3 Packet Link Quality Measurements ..................5

1.4 Copying Non-packet Telemetry ......................5

1.5 Obtaining Updates .................................5

1.6 Limit Checking ....................................6

1.7 Radio Control .....................................6

1.8 Using Different Terminal Units and TNCs ...........6

1.8.1 PK-232 .....................................6

1.8.2 KAM ........................................6

1.8.3 MFJ-1278 ...................................7

1.8.4 TNC1, TNC2 and PK-88 .......................7

1.8.5 PK-900 .....................................7

1.8.6 DSP-232 ....................................7

1.9 Upgraders Note Incompatibility ....................7

1.10 Logging Notes ....................................7

1.11 SAREX/MIR automatic connect/digipeat mode ........8

1.12 Self contained CW terminal .......................8

1.13 Parallel port Interface ..........................8

1.14 Equipping yourself for OSCAR ....................10

2.0 Customization .....................................12

2.1 Bringing WHATS-UP up for the First Time ..........12

2.2 Editing the Configuration File ...................12

2.3 Starting the program .............................13

2.3.1 Default ...................................13

2.3.2 User Chosen spacecraft ....................13

2.3.3 Custom Mode ...............................14

2.4 Screen Areas .....................................14

2.5 Setting Up Customized Display Pages ..............15

2.5.1 Customizing Analog Telemetry Channel

Displays .....15

2.5.2 Customizing Status Telemetry Channel

Displays .....16

2.5.3 Customizing Packet Header Displays ........16

3.0 Modes .............................................17

3.1 Standby Mode .....................................17

3.2 Interactive Mode .................................17

3.3 Real Time Mode ...................................17

3.4 Orbital Dynamics Mode ............................17

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 3

3.5 Playback Mode ....................................19

3.6 (Data) Extraction Mode ...........................19

3.7 Mutual Visibility Mode ...........................19

3.8 Audio Warnings and Orbit Data Displays ...........20

3.9 Autotrack ........................................20

4.0 Menus .............................................22

4.1 Function Keys ....................................22

4.1.1 FK 1 Capture to disk Toggle ...............22

4.1.2 FK 2 Type of display

Engineering Units/Raw Byte Toggle ..22

4.1.3 FK 3 Select display page ..................22

4.1.4 FK 4 Change Doppler frequency display .....23

4.1.5 FK 5 Display MH list ......................23

4.1.6 Alt-B send a 'Break' to the TNC ...........23

4.1.7 Alt-C connect to another packet station ...23

4.1.8 Alt-D disconnect from another

packet station .........23

4.1.9 Alt-F flush receiver buffer ...............23

4.1.10 Alt-I autotrack toggle ...................23

4.1.11 Alt-J jump to DOS (shell) ................23

4.1.12 Alt-P Printer on/off toggle ..............24

4.1.13 Alt-S Sound on/off toggle ................24

4.1.14 Alt-X Quit ...............................24

4.1.15 Alt += debug toggle ......................24

4.1.16 left arrow decreases playback speed ......24

4.1.17 right arrow increases playback speed .....24

4.1.18 Up arrow steps parallel port

radio memory up ........24

4.1.19 Down arrow steps parallel port

radio memory down ......24

4.2 Selections Menu ..................................24

4.2.1 Change Display Page .......................25

4.2.2 Edit Menu .................................25

4.2.3 Files Menu ................................25

4.2.4 Help Menu .................................25

4.2.5 Jump to DOS ...............................25

4.2.6 Modes Menu ................................25

4.2.7 Log Menu ..................................26

4.2.8 Orbits Menu ...............................26

4.2.9 Radio Menu ................................26

4.2.10 Spacecraft Menu ..........................26

4.2.11 TNC or PK232 Menu ........................26

4.2.12 Utilities Menu ...........................26

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 4

4.2.12 CW Menu ..................................26

4.2.14 Exit to DOS ..............................26

4.2.15 SAREX Menu ...............................26

4.3 Modes Menu .......................................26

4.3.1 Extract From Playback File ................27

4.3.2 Interactive Mode ..........................27

4.3.3 Orbital Parameters ........................27

4.3.4 Mutual Visibility .........................27

4.3.5 Playback Mode .............................27

4.3.6 Real Time Mode ............................27

4.4 Edit Menu ........................................27

4.4.1 Edit Doppler File .........................30

4.4.2 Any File ..................................30

4.4.3 Edit Keplerian Element File ...............30

4.4.4 Edit Spacecraft Configuration File ........30

4.4.5 Pick Capture-to-disk File .................30

4.4.6 Edit Capture-to-disk File .................30

4.4.7 Edit WHATS-UP.SYS .........................30

4.4.8 Two Files .................................30

4.4.9 Edit Doppler Channel File .................31

4.4.10 Edit Doppler Data File ...................31

4.4.11 Pick Spacecraft Configuration File .......31

4.4.12 Edit Today's Data ........................31

4.5 Files Menu .......................................31

4.5.1 Change Directory Path .....................31

4.5.2 Change Playback File ......................31

4.5.3 View Playback File ........................32

4.5.4 Show Spacecraft Capture-to-disk Files .....32

4.5.5 Show Data Files ...........................32

4.5.6 Show Files for 1 Spacecraft ...............32

4.6 Orbits Menu ......................................32

4.6.1 Pick AMSAT Format Element Set .............32

4.6.2 Edit Default Keplerian Element File .......33

4.6.3 Load Element File .........................33

4.6.4 Set Ref S/C (Mutual Visibility) ...........33

4.6.5 Pick NASA 2 Line Element Set ..............33

4.6.6 Show next pass ............................33

4.6.7 Set Ref = Sun (Mutual Visibility) .........34

4.6.8 View Spacecraft Orbit Elements ............34

4.6.9 Show/hide Sun Data ........................34

4.7 Radio Menu .......................................35

4.7.1 Turn Doppler Tracking ON/OFF ..............35

4.7.2 Set New Frequency .........................35

4.7.3 Change Doppler Interval ...................35

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 5

4.7.4 Set Radio Modulation ......................36

4.7.5 Read VFO A Frequency ......................36

4.7.6 Set Default Frequency .....................36

4.7.7 Select VFO A/B ............................36

4.7.10 Set Current Radio Memory .................36

4.7.11 Step Radio Memory Down ...................36

4.7.12 Set Radio Memory Scan Delay ..............36

4.7.13 Set Scan Radio Memory ON/OFF .............37

4.7.14 Select Radio Memory ......................37

4.7.15 Step Radio Memory Up .....................37

4.8 Spacecraft Menu ..................................37

4.8.1 Show Spacecraft Configuration File ........37

4.8.2 Default Spacecraft ........................37

4.8.3 picK Spacecraft ...........................37

4.8.4 Change Spacecraft .........................38

4.8.5 Pick Ops. Schedule ........................38

4.8.6 Show Ops. Schedule ........................38

4.9 TNC Menu .........................................38

4.9.1 UoSAT ASCII Beacon ........................39

4.9.2 Phase 3 RTTY Beacon .......................39

4.9.3 Set Morse Code (CW) .......................39

4.9.4 Fuji/MicroSat ASCII Packet ................39

4.9.4 Configure TNC .............................39

4.9.5 FM Packet .................................39

4.9.6 SARA ASCII Beacon .........................40

4.9.6 Select MFJ-1278 Radio Port

(if TNC is an MFJ 1278) ...............40

4.9.7 Select AO-13 PSK (if TNC is a DSP-2232) ...40

4.9.8 Select 9600 Baud Packet

(if TNC is a DSP-2232 or PK-900) ......40

4.10 Utilities Menu ..................................40

4.10.1 Change Directory Path ....................40

4.10.2 Enable/Disable RS-232 Port ...............40

4.10.3 Enable/Disable TNC Port

4.10.4 Turn MET Window ON/OFF ...................41

4.10.5 Show Space on Disk ............................41

4.10.6 Reset Header Counters ....................41

4.10.7 Show Defaults ............................41

4.10.8 Show Files ...............................41

4.10.9 Show Color Chart .........................42

4.10.10 Reconfigure WHATS-UP ....................42

4.11 Debug Menu ......................................42

4.11.1 CW Tone Test .............................42

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 6

4.11.2 Turn Debug OFF ...........................42

4.11.3 Set Frequency ............................42

4.11.4 Interrogate Radio ........................43

4.11.5 Show Defaults ............................43

4.11.6 Command Radio ............................43

4.11.7 Speak Frequency ..........................43

4.11.8 Identify Radio ...........................43

4.12 Log Menu ........................................43

4.12.1 Alt-A Append Entry .......................44

4.12.2 Alt-E Edit Log Entry .....................45

4.12.3 Alt-H Scan Log by Call ...................45

4.12.4 Pack Logbook .............................45

4.12.5 Alt-S Scan Log by Call ...................45

4.12.6 Alt-U Toggle Delete Mark .................45

4.12.7 Alt-X eXit Log ...........................46

4.12.8 Ins Toggle Insert Mode ...................46

4.12.9 End Show Last Page .......................46

4.12.10 Home Show First Page ....................46

4.12.11 PgUp Move Up One Page ...................46

4.12.12 PgDn Move Down One Page .................46

4.12.13 Up Arrow Move Up One Entry ..............46

4.12.14 Down Arrow Move Down One Entry ..........46

4.13 SAREX Menu ......................................46

4.13.1 Set Attack Mode ON/OFF ...................47

4.13.2 Change SAREX Call ........................47

4.13.3 Set Attack Mode Attack Mode [QCB] ........47

4.13.4 Reset SAREX Flags ........................48

4.13.5 Turn Zap R0MIR-1 ON/OFF ..................48

4.13.6 Single/Multiple User Connects ............48

4.14 CW Menu .........................................48

4.14.1 Sound Text ...............................49

4.14.2 Set CW Speed .............................49

4.14.3 Turn CW keyboard ON/OFF ..................49

4.14.4 Set CW Note ..............................50

4.14.5 Show CW Memories .........................50

4.14.6 Transmit Text ............................50

4.14.7 Slow CW Down .............................50

4.14.8 Speed CW Up ..............................50

4.14.9 Change CW Memory 1..10 ...................50

5.0 Orbital Elements ..................................51

5.1 Basics ...........................................51

5.2 Orbital Trajectories .............................51

5.3 Types of orbits ..................................52

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 7

5.3.1 Kepler's Laws .............................53

5.4 Orbital Elements ................................53

5.4.1 Angle of Inclination ......................54

5.4.2 Right Ascension of Ascending Node (RAAN) .......54

5.4.3 Eccentricity and Semimajor Axis ...........55

5.4.4 Decay Rate ................................56

5.4.5 Argument of Perigee .......................56

5.4.6 Epoch Time (of Ascending Node)

and Revolution Number .................56

5.4.7 The Mean Motion ...........................57

5.4.8 The Catalog Number ........................57

5.4.9 Mean Anomaly ..............................57

5.5 Anticipated Spacecraft Lifetimes .................57

5.5.1 Orbital Decay .............................57

5.5.2 Battery lifetime ..........................58

5.5.3 Radiation Damage ..........................58

6.0 The Spacecraft ....................................60

6.1 Receiving system components ......................61

6.1.1 Antennas ..................................61

6.1.2 Receivers .................................62

6.1.3 Terminal Units and Modems .................63

6.2 Receiving Signals from DOVE ......................64

6.3 Receiving Signals from UO-2 ......................65

6.4 Receiving Signals from AO-13 .....................65

6.5 Receiving PSK Modulated Signals in the 70cm Band .66

7.0 Decoding Spacecraft Telemetry .....................68

7.1 DOVE (DO-17) .....................................69

7.2 UoSAT-2 ..........................................72

7.3 AO-13 ............................................75

7.4 AO-16, WO-18 and LO-19 ...........................83

7.5 Fuji-OSCAR 20 ....................................91

8.0 Spacecraft No Longer Active ......................101

8.1 Fuji-OSCAR 12 ...................................101

8.2 SARA-OSCAR 23 ...................................104

8.2.1 The Primary Mission ......................105

8.2.2 The Secondary Mission ....................106

8.2.3 The Downlink .............................107

8.2.4 The Onboard Electronics ..................107

8.2.5 The Power System .........................108

8.2.6 Mechanical integration ...................108

8.2.7 Thermal Control ..........................109

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 8

8.2.8 Educational Opportunities ................109

8.3 ARSENE ..........................................110

8.3.1 Arsene Telemetry Equations ...............110

8.3.2 ARSENE Digital status telemetry ..........114

8.4 AO-21 (RM-1) ....................................115

9.0 File formats .....................................124

9.1 Configuration File (WHATS-UP.SYS) ...............124

9.1.1 Your callsign ............................125

9.1.2 Default spacecraft Name ..................125

9.1.3 Station Latitude .........................125

9.1.4 Station longitude ........................126

9.1.5 Station Altitude .........................126

9.1.6 Station minimum antenna elevation

for acquisition .................126

9.1.7 Default Kepler file ......................126

9.1.8 UTC offset ....................................126

9.1.9 Default directory path ...................126

9.1.10 Default extracted data file .............126

9.1.11 Default file name

with list of telemetry parameters

to extract file ......................126

9.1.12 TNC Type ................................127

9.1.13 Serial port to TNC ......................127

9.1.14 PC TNC Serial baud rate .................127

9.1.15 PC TNC port data bits ...................127

9.1.16 PC TNC port Stop bits ...................127

9.1.17 PC TNC Port parity bits .................127

9.1.18 Status (top) window color ...............127

9.1.19 Incoming window color ...................127

9.1.20 Outgoing window color ...................128

9.1.21 Prompt window color .....................128

9.1.22 Alarm window color ......................128

9.1.23 Bottom window color .....................128

9.1.24 Emphasis color ..........................128

9.1.25 Option color ............................128

9.1.26 Parameter changed color .................128

9.1.27 Parameter limit exceeded color ..........128

9.1.28 Orbit element window color ..............128

9.1.29 Orbit element window In range color .....128

9.1.30 Orbit element window early warning

color ...............128

9.1.31 Orbit element window next one up

color ...............128

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 9

9.1.32 Logbook window color ....................128

9.1.33 SAREX call display color in

Status window ...............128

9.1.34 Active color ............................128

9.1.35 Orbit element post pass color ...........128

9.1.36 Orbit alert dit time ....................128

9.1.37 Orbit alert note ........................128

9.1.38 Flag Sound ..............................128

9.1.39 Doppler display Flag ....................129

9.1.40 TNC Command to select modem for CW ......129

9.1.41 TNC Command to select modem for

UoSAT ASCII 1200 baud ......129

9.1.42 TNC Command to select modem for

1200 baud PSK ..............129

9.1.43 TNC Command to select modem for

400 baud AO-13 PSK .........129

9.1.44 TNC Command to select modem for

9600 baud packet ...........129

9.1.45 TNC Command to select modem for

1200 baud FM AFSK ..........129

9.1.46 Logbook file ............................129

9.1.47 QSO Logging flag ........................129

9.1.48 Minimum angle of the sun for darkness

at your QTH ...........129

9.1.49 TNC stream switch character .............130

9.1.50 Spacecraft Configuration File Linkages ..130

9.1.51 Marker and Comment line .................130

9.1.52 CW Memory contents ......................131

9.1.53 The marker line .........................131

9.1.54 TNC configuration commands ..............131

9.2 Spacecraft Parameter Files ......................131

9.2.1 Spacecraft ID ............................132

9.2.2 Spacecraft Suffix ........................132

9.2.3 Beacon Frequency .........................133

9.2.4 Doppler Measurement File .................133

9.2.5 Spacecraft Identification in

Keplerian Element File .....133

9.2.6 Doppler Measurement Sample Interval ......133

9.2.7 Initial Frequency Offset .................133

9.2.8 Autotrack flag ...........................133

9.2.9 Modulation ...............................134

9.2.10 Data Type ...............................134

9.2.11 Receiver Type ...........................134

9.2.12 Receiver Address ........................134

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 10

9.2.13 PC Serial port to Radio .................135

9.2.14 PC Radio port Serial baud rate ..........135

9.2.15 PC Radio port data bits .................135

9.2.16 PC TNC port Stop bits ...................135

9.2.17 PC Radio Port parity bits ...............135

9.2.18 Post pass delay .........................136

9.2.19 Station minimum usable pass time ........136

9.2.20 Early warning time (EWT) ................136

9.2.21 SAREX/Mir Callsign ......................136

9.2.22 SAREX/Mir Header Delay ..................136

9.2.23 SAREX/Mir Attack Mode ...................136

9.2.24 SAREX/Mir Beacon Text ...................137

9.2.25 LPT parallel port

radio memory parameters ..............137

9.2.26 Selected or default display page number .137

9.2.27 Page Definitions ........................137

9.2.28 Telemetry Parameter Configuration .......137

9.2.29 Digital Telemetry Status Channels .......144

9.2.30 Packet/Link Parameters ..................146

9.3 Telemetry Channel Extraction File ...............148

9.4 Extracted Telemetry Data File ...................149

9.5 Doppler File ....................................149

9.5.1 The Time .................................149

9.5.2 The Doppler Mark .........................150

9.5.3 The Frequency ............................150

9.5.4 The Doppler Shift ........................150

9.5.5 The Measured Shift .......................150

9.6 Kepler Element Files (*.TLE) ...............151

9.7 AMSAT Format Element File (*.AMS) ...............152

9.8 Spacecraft Operations File .................153

10.0 Glossary ........................................155

11.0 References and Further Reading ..................156

12.0 Change History ..................................157

13.0 Obtaining Further Information ...................158

14.0 Other Products by Joe Kasser, W3/G3ZCZ ..........159

14.1 PC-HAM 3.52 .....................................159

14.1.1 LOGBOOK..................................159

14.1.2 CONTEST .................................159

14.1.3 CQSS ....................................159

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 11

14.1.4 WHATSON .................................159

14.2 STARTREK The Computer Program ...................159

14.3.LAN-LINK 2.34 ...................................160

14.4 ELMER 1.00 ......................................161

14.5 BASIC PACKET RADIO ..............................162

15.0 How Shareware Works .............................164

Appendix 1 WHATS-UP 1.40 REGISTRATION FORM ...........166

Index ................................................167

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 1

1.0 Introduction

WHATS-UP is a ground station tool for communicating through and

monitoring the state of satellites.

WHATS-UP is a tool which provides you the Radio Amateur, or the

Educator, with the capability to perform experiments in spacecraft

orbital dynamics as well as monitoring the environment onboard

several of the Orbiting Satellites Carrying Amateur Radio (OSCAR)

during individual passes or over long periods of time.

WHATS-UP is a table driven program which uses configuration files to

allow maximum flexibility. If you have a Kenwood Radio, WHATS-UP

will monitor a default frequency, the when a spacecraft comes up

over the horizon, WHATS-UP will tune the Radio's VFO A to the

spacecraft beacon frequency (plus a presettable offset) and set the

receive mode for the satellite of interest. If the Radio responds

correctly, WHATS-UP will allow you to read back the VFO A frequency

during the pass and capture the data to a Doppler measurement file

for later import to a spreadsheet. At the end of the pass, WHATS-UP

will return the radio to the default frequency and communications

mode.

This version of the program is configured for a TNC-1, TNC-2, PK-88,

PK-900, DSP 2232, KAM and PK-232. Note, it will not allow you to

copy non packet telemetry if you don't have a multi-mode terminal

unit. It will intercept the Microsat Binary telemetry from AMSAT-

OSCAR 16 (PACSAT or AO-16), WEBER-OSCAR 18 (WEBERSAT or WO-18) and

LUSAT-OSCAR 19 (LUSAT or LO-19)) and display and process them as if

they were DOVE-OSCAR 17 (DO-17) ASCII telemetry. NOTE: AMSAT have

STILL NOT FINALIZED or PUBLISHED the Binary Telemetry Format for the

Microsats.

Spacecraft in the OSCAR series send back volumes of Telemetry daily

and apart from the odd Command station, few if any Radio Amateurs or

Educators seem to be doing anything with it. Radio Amateurs tend to

concentrate on the communications capabilities of the spacecraft and

ignore their telemetry completely. If they do listen to a beacon,

it's usually just to check that the transponder is on, heaven forbid

- to actually copy any data.

The telemetry can tell us a story. It can tell us what is happening

to both the spacecraft and its environment. As such it has a

tremendous educational potential which has remained just that - a

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 2

potential for at least the last eight years.

Before every satellite launch the equations and format for the

spacecraft telemetry are published by the builders. The telemetry

tells us about the health and welfare of the spacecraft itself, and

something about the payload. Spacecraft health and welfare

information tells us about the battery, solar cells and on board

computer status. Payload information can range from information

about transponder loading/utilization to data from instruments that

measure the environment of the space in and around the satellite.

Battery Telemetry is used by the command stations to determine when

the spacecraft can be used, and when the transponders should be shut

down. The number of individuals not associated with command stations

who have decoded spacecraft telemetry and published their findings

can be counted using the fingers of one hand.

Capturing, decoding and displaying telemetry from orbiting

spacecraft in real time, in the classroom, is an excellent way of

introducing space science to students. Signals from these spacecraft

are downlinked on frequencies that can be received on regular

vhf/uhf scanner radio receivers. WHATS-UP provides an introduction

as to how this can be done using readily available low cost

equipment. General topics discussed cover telemetry, the spacecraft

themselves. Groundstation hardware topics include receiving

antennas, radio receivers and modems. Software topics discussed

include the software used to track the spacecraft and the software

used to both decode and display the data in real time as well as

that for post pass analysis. Excluding the Personal Computer, a

simple telemetry capturing groundstation can be set up for less than

$500.00 in equipment costs.

There is no substitute for the excitement of hands-on experience in

awakening an interest in space. While OSCARs in the main, transmit

down to the ground (downlink) telemetry in morse code, several of

them also utilize standard computer literate digital data schemes

such as BAUDOT or ASCII codes. The thrill of receiving a signal from

space soon fades however if the data cannot be understood. Even

after the data has been decoded, watching the temperatures on-board

a spacecraft as it passes overhead is also of little interest, but,

what can be made interesting is receiving and capturing the data

over many days or even months and looking for trends and

relationships.

You can capture telemetry just by listening to a spacecraft and

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 3

copying morse code by ear writing it down with a pencil on a piece

of paper. All you need to do is listen on the correct frequency at

the right time and you will hear some signals. Write down what you

hear. Apart from the thrill of copying a signal from a satellite,

there's not much else to do with the data. While you can look up the

conversion equations and get a snapshot of what is happening, it

soon becomes tedious and only dedicated souls do it regularly.

When AMSAT were building the Phase 3A spacecraft, personal

computing was in its infancy. That was before the Apple 2 or the

TRS-80; that was the day of the home built S-100 machine, and the

AMSAT-GOLEM-80 Project. At that time, Karl Meinzer, DJ4ZC, had

written a native German high level computer language called IPS.

This language was very similar to Forth, but contained a multi-

tasking kernel. This language was programmed into the spacecraft

flight computer and also used in the ground command stations. Using

a computer on the spacecraft allowed AMSAT to provide telemetry that

could be captured by computer on the ground. While Phase 3A never

made it into orbit (the launch vehicle malfunctioned), subsequent

spacecraft continued to provide the same facility. Today UoSAT-2

(UO-2), AMSAT-OSCAR 13 (AO-13), UoSAT-OSCAR 14 (UO-14), AMSAT-OSCAR

16 (AO-16), DOVE-OSCAR 17 (DO-17), WEBER-OSCAR 19 (WO-18), LUSAT-

OSCAR 19 (LO-19), Fuji-OSCAR 20 (FO-20) and AMSAT-OSCAR 21 (AO-21)

are all sending back telemetry that you can capture with your PC and

amateur radio equipment. As most non radio amateurs do not

understand morse code, WHATS-UP concentrates on those spacecraft

which downlink computer compatible telemetry and can be received

with relatively simple equipment.

1.1 Capabilities of WHATS-UP

WHATS-UP contains the following features:

*    Self contained CW (keyboard only - you copy audio by ear)

satellite ground terminal with parallel port interface to Radio

up/down frequency control.

*    Automatic connect attempt and beaconing via MIR, SAREX or any

other satellite with appropriate TNC.

*    Automatic logging of Acquisition of Signal (AOS) in LAN-LINK

compatible logbook.

*    Display of spacecraft orbital elements and tracking data.

*    Will automatically set your Kenwood receiver to spacecraft

beacon frequency when the spacecraft comes above your horizon

and return it to the default frequency when the pass is over.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 4

*    Audio warning of AOS and loss of signal (LOS).

*    Real-time, Interactive and Playback modes.

*    Automatic Capture-to-disk of raw telemetry.

*    Extracts telemetry channel data to a database or spreadsheet

readable file for further analysis.

*    Link quality measurement.

*    Capability to display and print the raw telemetry as it is

received.

*    Up to 16 user configurable display pages (screens). You set the

position on the page (width of engineering unit field, and

number of decimal places) that a parameter is displayed at.

*    Wild card page (parameter shows up on all pages).

*    Selectable display of Engineering units or Hex byte for each

display page.

*    Display of raw packets (i.e. STATUS)

*    Color changes if a parameter value changed between successive

frames.

*    Audio and visual alarms if a telemetry value exceeds, falls

below or falls outside a preset limit value(s).

*    Dumb split screen terminal mode with user selectable number of

window rows.

*    Customizable colors, PC to TNC baud rate, data parity and stop

bits.

*    Default spacecraft configuration files.

*    UTC Time of day clock display (in HH:MM:SS format)

1.2 Program Requirements

IBM PC or clone with at least 256k memory.

A Radio receiver and a TNC with an RS-232 interface is only

required for real time data capture.

The program DOES require that the packetized telemetry be received,

and captured-to-disk with the packet header on a different line to

the contents of the packet (HEADERLINE ON). I also suggest that you

turn the date/time stamp on so that you will be able to playback

your data and extract selected values and their corresponding time

codes into a file that can be read into your spreadsheet program for

further trend analysis.

This version (the default) is set up to display all packets as wild

cards (i.e. will show up on all pages), and then display several

temperatures and solar cell array currents.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 5

By putting the correct parameters in the SPACECRAFT.CNF file, you

can set up any of the pages to display any of the telemetry channel

data in any row and column in that page.

1.3 Packet Link Quality Measurements

The packet link quality part allows the following to be done. You

can define which of the packets you want to display/count (If you

just want to count them and not display them, set the page value to

99). You can then view an incrementing count counter each time a

selected packet is received. For example, using DO-17, you can also

display the contents of the STATUS, WASH or BCRXMT packets in any

display page. This feature allows you to get an idea of how good

your receiving system is.

1.4 Copying Non-packet Telemetry

Packet telemetry, by definition is error free. The RTTY blocks from

AO-13 on the other hand may be received with errors. These errors

may take the form of garbled or missing characters. If your link is

bad, and such hits do occur, you should edit the capture-to-disk

file before you playback the telemetry and decode/display the data.

If you get bad data in real time, look at the raw data. You will be

able to see the quality of the link.

1.5 Obtaining Updates

It is anticipated that WHATS-UP is going to grow and incorporate

features for decoding and displaying data from other spacecraft.

Better Microsat binary telemetry decode and display capability will

be added when AMSAT announce that the format has stabilized, and

sufficient registered users express interest in having it. To stay

on the mailing list and receive an update as it is released,

register your copy, then send in a disk containing at least 1

Megabytes (zipped) of captured data from the spacecraft of your

choice. If you would like to exchange data with other educational

institutions or users so as to be able to analyze more data than you

can get on a single pass, indicate that fact and we will try and put

you in direct touch with others who are similarly inclined.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 6

1.6 Limit Checking

Limits are only checked for parameters being displayed (in

Engineering units). With this approach, you can set up different

pages for different on-board subsystems, you can also set up

different display pages of the same parameters for daylight,

darkness and terminator crossing passes, with different limit values

to draw your attention to changes.

1.7 Radio Control

Assuming you use a radio, when you first bring up WHATS-UP, it sets

the radio VFO A to the beacon frequency of the default spacecraft

configuration file (*.CNF). When a selected spacecraft is scheduled

to come above your horizon WHATS-UP will tune the radio to the

spacecraft's beacon frequency. When the pass is over, WHATS-UP will

return the radio to the default frequency. You can thus set up a

"dummy" configuration file for your local Packetcluster or voice

repeater and monitor that channel as long as the selected spacecraft

are below your horizon.

If a pass is taking place, and a second selected spacecraft comes

over your horizon, WHATS-UP will tune to its frequency UNLESS the

autotrack is disabled. See Section 3.9 for a description of the

autotrack feature.

1.8 Using Different Terminal Units and TNCs

This section briefly covers some of the differences between the

various TNCs on the market. To copy PSK signals from Fuji-OSCAR 20

and the microsats, you will need a PSK modem adapter for your TNC.

WHATS-UP has no way of checking that it is present.

1.8.1 PK-232

The PK-232 does not have a software selectable TNC radio port. You

have to use the switch on the panel. The PK-232 will receive signals

from UO-11 properly, only if the hardware modification described in

Section 7.2 of this document is performed.

1.8.2 KAM

The KAM cannot demodulate SARA's 300 baud ASCII or 1200 baud ASCII,

so signals from SARA and UO-11 cannot be copied. The KAM 2.85

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 7

firmware used to develop WHATS-UP assumes vhf packet on the vhf

port, and the non packet modes on the hf port. WHATS-UP 1.20 does

not let you switch ports on the KAM.

1.8.3 MFJ-1278

The MFJ-1278 cannot copy SARA's 300 baud ASCII or UO-11's 1200 baud

ASCII. WHATS-UP does not change the radio port. You set the radio

command in the WHATS-UP.SYS file. If you wish to change the radio

port, an option is provided in the Configure TNC Menu.

1.8.4 TNC1, TNC2 and PK-88

These TNCs only copy packet. The other modes will not work.

1.8.5 PK-900

This TNC is treated like a PK-232. If you have the optional 9600

baud modem you can try for the 9600 baud microsats.

1.8.6 DSP-232

This TNCs will give you all known modes. The current firmware does

not give you step track from the TNC to the Radio for PSK.

1.9 Upgraders Note Incompatibility

If you are upgrading from 1.30 or an earlier version the WHATS-

UP.SYS and SPACECRAFT.CNF files in 1.40 are not compatible.

1.10 Logging Notes

When WHATS-UP logs an acquisition, it places some temporal

information in the comments column. This information is, either:

*    A check mark, if AOS is as scheduled.

*    The number of minutes into the pass if AOS is late.

*    A '>' and the estimated time to AOS is acquisition occurred

when WHATS-UP computed the spacecraft as being below the

horizon.

This information is designed to help you update any estimates for

AOS in the event your element set is out of date.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 8

1.11 SAREX/MIR automatic connect/digipeat mode

This feature should be used with caution as it has the potential to

cause QRM to other stations on channel. As a safeguard, WHATS-UP

will turn its attack mode off if it someone connects and sends a

':QRT:' instruction.

1.12 Self contained CW terminal

WHATS-UP contains a CW terminal, controlling the radio keyer and

parallel port interface. You can connect the parallel port to the

radio up/down buttons via the simple circuit described in Section

1.13. With a display of the Orbit parameters, you can:

Transmit CW by typing the characters at the keyboard.

Copy CW by ear.

Control the VFO to compensate for Doppler shift using the

up/down arrows.

Log the contacts into the logbook.

And you don't need a TNC to do it. If you go on a cw satellite DX-

pedition, and are taking along a laptop or notebook computer, try

this feature. If you are staying home, try it in the shack.

1.13 Parallel port Interface

WHATS-UP contains a driver to interface the PC's parallel port to a

radio. It maybe used as described in Section 1.12, or as described

in this section.

The interface circuit is shown in Figure 1.13. It consists of a TTL

latching register type 75LS175 and optical isolators on the output

side. You also need a 5 volt supply for the 74LS175 which may be

obtained from any suitable source. Each optical isolator circuit is

identical, so only one is shown below. A serial LED with each

optical isolator provides visual indication that something is

happening. You must use a strobed interface to ensure reliable

control of each bit.

The parts may be obtained from Radio Shack or any similar source

and are not critical. Almost any optical isolator will do provided

it can handle the open key voltage from the radio. If you are using

a solid state radio which runs of a 12 Volt supply, then don't worry

about the interface.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 9

The port bit assignments are as follows:

Bit            Assignment

1             Key

2             Radio up

3             Radio down

4..8          Unassigned/unused

You can set the printer port assignment in the receiver address

line of the SPACECRAFT.CNF file (Section 9.2.12).

Figure 1.13 Simple parallel interface circuit.

+---------------+-----------------------< +5 volts

PC Printer     |               |

Port           |5V          RESISTOR (330 ohms)

ST       -----              |      -------

1 >--------|CK   |             |-|<|--|     |---------< key

DB0     |     |   keyboard    LED  |     |---------< Common

2 >--------|A  Qa|--------------------|_____|optical isolator

DB1     |     |   radio up                      4N33 etc.

3 >--------|B  Qb|---------------    similar isolation  -< Up

DB2     |     |   radio down         circuits

4 >--------|C  Qc|---------------                       -< Down

|     |

|     |Ground

|-----|-+

74LS175    |

25>----------------+--signal ground (also on 18..24).

Notes

1    Do not connect PC signal ground connect to radio common side.

If you do, there is not much point in using isolation circuits.

2    Decouple 5 Volt supply to PC Signal ground.

3    Ground all unused inputs of the 74LS175 to avoid spurious

effects due to transients.

4    Use similar optical isolator connections for the up and down

connections to the radio.

5    Optional, but useful. Wire a 470 ohm resistor in series with a

green LED across the 5Volts and ground.

6    Use red LEDs in series with the optical isolators.

7    Add pull-up resistors on PC interface lines and 74LS175 clear

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 10

and reset lines.

1.14 Equipping yourself for OSCAR

The OSCAR satellites use frequencies in the VHF and UHF bands not

normally covered by the usual short wave or HF transceivers. Also

since the spacecraft transponders receive on one band and transmit

on a second, you will need to talk at them on one band and listen to

them on a second band. That means, not only do they operate split

frequency, but they operate split frequency on different bands.

In order to work people via the OSCAR spacecraft, you will need

transmitting equipment for your uplink and receiving equipment for

the downlink. If you are already a serious VHF/UHF operator you will

have SSB/CW equipment for the VHF/UHF bands used by the spacecraft.

On the other hand if you are new to the satellite bands, you will

have to purchase or otherwise obtain the necessary equipment.

There are two types of OSCAR satellites in orbit at this time. The

Phase 2 spacecraft are in low orbits, and carry a Mode A transponder

with an uplink on the 2 Meter band, and a downlink on the 10 Meter

band as well as a transceiver for a different Mode. The Phase 3

spacecraft are in high orbits and carry a Mode B transponder with an

uplink on the 70 Cm band and a downlink on the 2 Meter band. If you

thus equip yourself for operation on the 2 Meter band, assuming that

you have HF capability, you will be able to monitor signals from

both types of satellites and may even be able to work Mode A.

Take a look at the advertisements for 2 Meter SSB/CW transceivers in

the magazines. They look nice, have useful specifications but the

price! Do you really want to spend that money on a band you are not

sure that you will be active on?. The VHF/UHF bands aren't like the

HF ones. The majority of your contacts will be ground wave line-of-

sight. That means that your maximum range under normal conditions

will be between 100 and 200 miles.

If you live in an area of high VHF/UHF activity you may find it

worth while, because you will always have someone to talk to and

when conditions are right, you will be able to work DX.

Another factor to consider is the distance between the operating

position and the antennas. Co-axial cable has significant losses at

VHF/UHF frequencies. Satellite links are usually marginal, that

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 11

means that you cannot afford any reduction in your uplink power or

in the strength of the downlink.

Consider an alternative. In the days before commercial multi mode

VHF/UHF equipment became available, many people got on the air using

transverters. A transverter is a piece of equipment that contains a

transmitting and a receiving converter. Equipment such as the old

"Europa B" and the Microwave Modules series which are still

available were used to good effect. The Microwave Modules and units

made by other manufacturers are still available as you can see by

looking at the advertisements in your favorite amateur radio

magazine.

The transverter must be driven by a transceiver. The output from the

transceiver down converter is fed to the regular station receiver.

The input to the transceiver up converter comes from a low power

point in the transceiver. The well known FT-101 series of HF

transceivers as well as a number of others were designed with

transverters in mind. They are readily available on the used

equipment market at reasonable prices. Look at those advertisements

again, compare the cost of a used FT-101 (or similar rig) and a

transverter with the cost of that multi mode VHF/UHF rig. Getting on

two meters SSB this way has a number of advantages as follows.

The FT-101 can be used as a back up HF rig in the event of a failure

in the prime station transceiver. The FT-101 can also be modified to

work the new WARC bands. It can thus be used in a stand alone manner

from all bands 160 through 10 Meters.

The transverter could even be mounted on the mast at the antenna,

thus potentially providing a significant reduction in line losses

bare).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 12

2.0 Customization

2.1 Bringing WHATS-UP up for the First Time

WHATS-UP comes configured to use Com1 in the PC as the serial port

between the PC and the PK-232, and with the serial port between the

PC and the Kenwood Radio DISABLED (Set to 0). WHATS-UP is also

configured to assume DOVE as the default spacecraft.

If you type 'WHATS-UP' followed by the 'Enter' key, the program

will load and start to do things. If the default serial ports are

correct for your system, you need do nothing further, other than

change the colors of the windows to your preference.

2.2 Editing the Configuration File

The configuration file is called WHATS-UP.SYS. You must edit it

using option 'S' in the Edit Menu or with an ASCII word processor

(in the non document mode) to set up the correct parameters on the

RS-232 link between your TNC and your PC. See Section 10 for details

of what parameter is on which line of the WHATS-UP.SYS file. If you

wish to activate the Radio features, you must select a Radio Port.

Before you make any changes, make sure that you do not try and edit

your original, always work from a copy.

These are the minimum set up items to change in the WHATS-UP.SYS

file. Refer to Section 9 for fuller details of what items are on

which lines. Bring up the Selections menu, the access the Edit Menu.

choose the option for editing the WHATS-UP.SYS file. As you enter

items on the lines listed below, delete the entries currently in

place.

Line 1: Enter your callsign instead of the default one. Use Capital

letters (Upper case). If you do not have a callsign, pick a

mnemonic that represents your school or organization. You

may use up to 10 characters. The callsign entered here,

will be shown at the top of the status window, next time

you load WHATS-UP, and will also (more important) be

appended to your capture-to-disk files to identify the

ground station which received the data.

Line 3: Enter the latitude of your location. In the southern

hemisphere, use a negative number.

Line 4: Enter the longitude of your station in degrees West of

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 13

Greenwich in the range 0 to 359.9.

Line 5: Enter the station altitude above sea level in meters. If

you don't know, don't change anything.

Line 10: Enter the difference between local time in your PC and

UTC. For example, eastern standard time is 5.

Line 12: Enter the name of the TNC (PK-232, PK-900, DSP-2232, TNC2,

KAM, MFJ1278, or PK-88).

Line 15: If you are not using Com1 as the serial port between the

PC and the TNC, enter the port number on this line.

Line 16: If you are not using 1200 baud between the PC and the TNC

enter the actual value on this line.

Line 20: If you have a compatible radio with RS-232 control

capability, enter the port number on this line (usually

Port 2).

Read Section 9.1 to see what else to change to make the PC

communicate with the TNC and the radio.

Save the file at this time. WHATS-UP will now work for you in its

minimal mode. Read the manual to learn about its capabilities and

use section 9 when you customize it to suit yourself.

2.3 Starting the program

You may start the program in three ways as follows.

2.3.1 Default

Type 'whats-up' and return (without the ' characters). This brings

the program up in the default mode. It will read the whats-up.sys

file to determine the spacecraft being monitored, and then prompt

you for the mode.

2.3.2 User Chosen spacecraft

Type 'whats-up spacecraft' and return (without the ' characters).

The program reads the spacecraft.sys file to load the parameters for

the Microsat of choice, and then prompts you for the mode. Examples

of the command are :

'WHATS-UP DOVE' or 'WHATS-UP Fuji20'

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 14

2.3.3 Custom Mode

Type 'whats-up spacecraft mode' and return (without the '

characters). This brings the program up in the custom mode. It will

read the whats-up.sys file to determine the spacecraft being

monitored, and then start up in the mode you set. Valid modes are

'p', 'i', 'o' and 'r'. Examples of the command are:

'WHATS-UP DOVE R' or 'WHATS-UP Fuji20 R'

If you place a command line like this in your autoexec.bat file,

should you be copying telemetry in an unattended manner and a power

failure take place, the system will boot up into the correct WHATS-

UP mode when power is restored.

When the program loads, the first thing it does is load the

Keplerian data from the default data file and compute the current

position of all the spacecraft. The following typical messages will

be send during this activity.

MIR Loaded

RS-10/11 Loaded

AO-13 Loaded

UO-14 Loaded

DO-17 Loaded

Computing Next Pass for AO-10

Computing Next Pass for UO-11

Computing Next Pass for MIR

Computing Next Pass for RS-10/11

Computing Next Pass for AO-13

Computing Next Pass for UO-14

Computing Next Pass for DO-17

If the default Keplerian data file is not present on the disk, the

orbital mode will be disabled. If the default Keplerian data file is

empty, WHATS-UP will abort with a Run-Time Error.

2.4 Screen Areas

The screen is divided into four window areas. The Status window

occupies the top line. The prompt window occupies the bottom line.

Raw data are always shown in the lower data area. This window also

shows any outgoing text that you may type at the keyboard or any

commands WHATS-UP sends to the TNC. The data area in the top half of

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 15

the screen contains the processed data (real time and playback), the

raw data (interactive) and the orbital data display.

2.5 Setting Up Customized Display Pages

The ability to customize a display page is a unique tool provided

by WHATS-UP. This tool allows you to group telemetry information so

you can see how the monitor points you are interested in behave, and

visually see the changes. You can locate information in any position

on the screen and in any color you wish. You can also perform limit

checking on the data, and have the "out-of-limit" show up in

different colors and generate audible alarms.

Setting up display pages is an orderly procedure using the

information contained in Section 9. When you want to customize the

displays, make a copy of the supplied spacecraft configuration file

and work with it. Look at the format of the file, read Section 9.2

to sure that you understand what is in the file. Use the editor to

adjust the items in the spacecraft configuration file. After editing

the file, reload the file to enter the changed parameters into

WHATS-UP to see the effects of your changes. Repeat this procedure

until you are satisfied.

First, from the utilities menu, view the colors and decide on the

color scheme for the window and data.

Lay out the display page on graph paper for a screen window of 13

rows by 78 columns.

Decide on the name of the page. If it is a new page enter the name

on a new line after the existing pages before the line that ends in

an '*' character. Do not delete the '* character. After you have

entered the page name, insert a comma character ',' and a number for

the default page color. An example from the DOVE.CNF file for such

an entry is shown below.

PAYLOAD TRANSMITTER/RECEIVERS, 30

2.5.1 Customizing Analog Telemetry Channel Displays

The next few lines contain the analog telemetry decoding display

information. Review Section 9.2 to see what has to be inserted in

each of the elements. The analog telemetry lines are terminated with

the following line, which must be present even if no data lines are

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 16

present.

'* end of telemetry'

2.5.2 Customizing Status Telemetry Channel Displays

Section 9.2.14 describes the meaning of each item. The status

telemetry lines are terminated with the following line, which must

be present even if no data lines are present.

'* end of status bytes'

2.5.3 Customizing Packet Header Displays

The packet header configuration lines contain the analog telemetry

decoding display information. Details of the meaning of each item

are given in section 9.2.15. The packet header lines are terminated

with the following line, which must be present even if no data lines

are present.

'* end of packet types'

If you want to change the colors that channels out of limit, or

channels in which the data changed between telemetry frames are

displayed in, you must change the entries in the WHATS-UP.SYS file.

207 is a respectable value for the color to display data for which

the limit has been exceeded (flashing white on red background).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 17

3.0 Modes

WHATS-UP is designed to operate in a number of modes as listed

below. Each mode has its own type of window display. To change

things or select features, touch the 'Esc' key to bring up the top

level menu. Menus in WHATS-UP are organized as a hierarchy, each

option will take you as deep as necessary to make your selection.

3.1 Standby Mode

The standby Mode presents you with the Modes Menu as described in

Section 4. This mode is activated if you bring up WHATS-UP without

specifying a mode in the command line.

3.2 Interactive Mode

The Interactive mode is a dumb terminal. You can use it to give

commands to the TNC. You should also use it to set the 'HEADERLINE

ON'. In this mode, you will see the raw packets on the channel. You

can also use this mode as a regular TNC program (If you do, you

ought to get your head examined, because LAN-LINK will do the job

much better). The capture-to-disk will turn on when the first packet

is copied, and will turn off two minutes after the last.

3.3 Real Time Mode

The Real-time mode converts and displays engineering data. You can

display up to 16 (configured by you) pages of information.

Information that changes between successive frames, is shown in a

different color. Information that has exceeded a preset (by you)

limit is shown in an alarm color (default: blinking red). The

capture-to-disk will turn on when the first packet is copied, and

will turn off two minutes after the last.

3.4 Orbital Dynamics Mode

The orbital dynamics mode gives you a display of the positions of

the spacecraft with respect to your location. If the spacecraft are

out of range, then you see time related information. If the

spacecraft are in range then you also see position related

information. The time that the spacecraft comes above your horizon

is when you acquire signals. This time is known as Acquisition of

Signals (AOS) time. You lose signals when the spacecraft drops below

your horizon. This time is known as Loss of Signals (LOS). WHATS-UP

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 18

allows you to define an early warning time (EWT) (in minutes) before

AOS. The period of time between AOS and LOS is known as the pass.

Orbit positional information is only as accurate as your PC clock

and the reference Keplerian data supplied by AMSAT and other

sources. You can expect an accuracy of within a minute or two if

your PC clock is correct. WHATS-UP only provides time displays

accurate to within a minute or two. Its not worth going for greater

accuracy in most cases.

The following information is only displayed for spacecraft in

range.

AZ Azimuth    - Pointing angle to spacecraft along horizon (0 =

North, 90 = East).

EL Elevation  - Pointing angle to spacecraft from horizon to

zenith.

RANGE         Distance between your station and the spacecraft. An

Up arrow next to it indicates that the altitude is

increasing, a down arrow, that it is decreasing.

DPLR          Doppler shift on spacecraft's beacon signal.

A typical example of part of the display is shown below.

#�##�#S/C#�##�##�##�##�#WNDOW#�##�#AOS#�##�##�#LOS#�##�#PASS#�##�##�##�#MA#�##�##�##�##�#ALT#�##�##�##�#AZ#�##�##�##�##�##�#EL#�##�##�#RANGE

UO-11    02:05 00:08 00:19 00:11  143     681

AO-13    04:51       02:54 08:01  122   37723  47.92  24.91

=DO-17    03:32 01:35 01:45 00:10  206*    792

The first column provides information about the automatic

sequencer.

A check mark indicates that the spacecraft is the selected one for

data to be captured.

An equals sign indicates that WHATS-UP is configured to configure

itself to tune to its beacon signal and capture data at AOS for

that spacecraft.

A "less than equals" sign indicates that WHATS-UP is configured to

configure itself to tune to its beacon signal and capture data

at EWT for that spacecraft.

The display is color coded as follows.

Spacecraft out of range  displayed in the default window color.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 19

Spacecraft in range are displayed in the 'in range' color.

Spacecraft within EWT are is displayed up in the EXT color. The

spacecraft designator for the next spacecraft to come in range is

displayed in the 'next one up' color. These colors may be configured

to your preference in the WHATS-UP.SYS file.

3.5 Playback Mode

The Playback mode allows you to play back captured telemetry with 4

speeds (speedy, slow, slower and snail's pace).

3.6 (Data) Extraction Mode

In this mode, data is extracted from a playback file into a file

that can be read into a spreadsheet. If you answer the prompt for

the default file with a non-existent filename, WHATS-UP will prompt

you for individual channel numbers. To terminate the sequence and

begin the extract mode, touch the 'Enter' key without entering a

channel number. Note: Start and stop times are text string matches.

3.7 Mutual Visibility Mode

This mode provides a display of the mutual visibility between a

spacecraft and all the others displayed in the window. A typical

section of a display is shown below.

===S/C=WNDOW==AOS===LOS==PASS===MA====ALT=LAT====LONG==FTPRNT=D-SSP

#�#MIR   00:48 22:46 22:54 00:08  66#�#   395 -46.25 287.01  4378 11259

AO-10 02:42 00:39 07:09 06:29 217#�# 19206 -13.89 343.58 16834 15543

AO-13             22:02 08:28 189#�# 30799 -39.77  68.49 17841  7504

=AO-21 04:29 02:26 02:31 00:04 158#�#  1003  -5.67 155.89  6729  2139

RS-12 00:14 22:11 22:23 00:11 245#�#   963 -24.46 111.93  6608  3646

The information displayed in the mutual visibility mode window are

as listed below.

WNDOW    Time till AOS (if out of range, or time till LOS if in

range.

AOS      Time of AOS. This item is not displayed if the spacecraft

is in range.

LOS      Time of LOS.

PASS     The amount of time that the pass will last. If the

spacecraft is in range, then this item displays the

minutes remaining till LOS.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 20

MA       Mean Anomaly, or position of the spacecraft in its orbit

(see Section 5).

If the spacecraft have mutual visibility, a check mark '#�#'

character will be displayed following the MA value.

ALT      The altitude of the spacecraft above sea level. An Up

arrow next to it indicates that the altitude is

increasing, a down arrow, that it is decreasing.

LAT      The latitude of the point on the surface directly below

the spacecraft (sub-satellite point).

LONG     The longitude of the point on the surface directly below

the spacecraft (sub-satellite point).

FTPRINT  The diameter of the footprint of the satellite on the

surface of the earth.

D-SSP    The distance between the sub-satellite point of the

satellite and the one for which mutual visibility has been

calculated.

This mode is useful if you want to determine the possibility of a

double hop communications link through two spacecraft, or want to

see when Mir can work the Space Shuttle.

3.8 Audio Warnings and Orbit Data Displays

WHATS-UP provides audible warnings in morse code of AOS, LOS and

EWT. Each warning consists of a letter followed by the spacecraft

designator. An 'A' prefixes AOS, an 'L' prefixes LOS and a 'Q'

prefixes EWT.

WHATS-UP also shows orbit data associated with the selected

spacecraft in the bottom window. This data which duplicates the line

shown in the orbital mode, is there to be used in the real-time and

interactive modes.

3.9 Autotrack

An autotrack flag is present in each spacecraft configuration file.

The autotrack flag allows you to customize the way WHATS-UP switches

radio frequencies and TNC modes. If the autotrack is disabled,

WHATS-UP will not switch until after LOS of the satellite currently

tracking. If the autotrack flag is enabled, WHATS-UP will switch

radio frequency and TNC mode when a new spacecraft is present even

if it is currently tracking one. You use the autotrack flag to stop

WHATS-UP from switching away from a satellite in the middle of a

pass if a new one comes up over the horizon.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 21

If the autotrack flag a disabled, a flashing indicator will be seen

next to the spacecraft name in the status window.

WARNING

If you disable the autotrack flag in the default configuration file,

or manually load a spacecraft configuration file with the autotrack

disabled, WHATS-UP will not change radio frequencies or TNC modes

until the next LOS.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 22

4.0 Menus

WHATS-UP provides various features organized in a hierarchy of

menus. To bring up the top menu, touch the 'Esc' key.

4.1 Function Keys

The following function keys are active in the operational modes

described in Section 3:

FK 1          Capture to disk Toggle

FK 2          Type of display Engineering Units/Raw Byte Toggle

FK 3          Select display page

FK 4          Change doppler frequency display

FK 5          Display packet Monitor Heard (MH) list

Alt-B         Send a 'Break' to the TNC Alt-C Connect to another

packet station

Alt-D         Disconnect from another packet station

Alt-F         Flush receiver buffer

Alt-I         Autotrack toggle

Alt-J         Jump to DOS (shell)

Alt-P         Printer on/off toggle

Alt-S         Sound on/off toggle

Alt-X         Quit Mode

Alt +=        Debug toggle

left arrow    Decreases playback speed

right arrow   Increases playback speed.

4.1.1 FK 1 Capture to disk Toggle

This function key turns the capture-to-disk ON if it is off, and

turns it OFF if it is on.

4.1.2 FK 2 Type of display Engineering Units/Raw Byte Toggle

This function key changes the real-time, playback telemetry display

between raw data and decoded engineering units.

4.1.3 FK 3 Select display page

This function key allows you to select a different customized data

display page.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 23

4.1.4 FK 4 Change Doppler frequency display

This function key allows you to change the Doppler frequency

display between the computed receive frequency and the difference

between the satellite beacon frequency and the computed receive

frequency.

4.1.5 FK 5 Display MH list

This function key configures WHATS-UP into the Interactive mode and

sends a "MH" command to the TNC.

4.1.6 Alt-B send a 'Break' to the TNC

This function key allows you to send a "break" character to the

TNC. You use it if you accidentally put the TNC in the transparent

mode.

4.1.7 Alt-C connect to another packet station

This function key prompts you to enter a callsign, then, if you are

in the packet communications mode, attempts to connect you to that

station.

4.1.8 Alt-D disconnect from another packet station

This function key allows you to disconnect a packet mode

connection.

4.1.9 Alt-F flush receiver buffer

This function key allows you to flush the receive buffer.

4.1.10 Alt-I autotrack toggle

This function key allows you to enable or disable the autotrack

feature. You use this key to override the default value in the

spacecraft configuration file.

4.1.11 Alt-J jump to DOS (shell)

This function key lets you jump into DOS for a while without

exiting from WHATS-UP.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 24

4.1.12 Alt-P Printer on/off toggle

This function key lets you turn the printer on and off. If the

printer is turned on, any incoming raw data will be echoed to the

printer as well as to the screen.

4.1.13 Alt-S Sound on/off toggle

This function key lets you turn the sound on and off.

4.1.14 Alt-X Quit

This function key quits the mode and allows you to terminate WHATS-

UP and return to DOS.

4.1.15 Alt += debug toggle

This function key lets you enable and disable the Debug Menu. If

the debug menu is enabled, an '*' will be displayed on the left hand

side of the status window.

4.1.16 left arrow decreases playback speed

This function key decreases playback speed in the Playback mode.

4.1.17 right arrow increases playback speed.

This function key increases playback speed in the Playback mode.

4.1.18 Up arrow steps parallel port radio memory up

This function key steps the parallel port radio memory up on

channel.

4.1.19 Down arrow steps parallel port radio memory up

This function key steps the parallel port radio memory up on

channel.

4.2 Selections Menu

This menu presents you with the following typical options.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 25

C Change Display Page

E Edit Menu

F Files Menu

H Help Menu

J Jump to DOS

L Log Menu

M Modes Menu

O Orbits Menu

R Radio Menu

S Spacecraft Menu

T PK232 Menu

U Utilities Menu

W CW Menu

X Exit to DOS

Z SAREX Menu

4.2.1 Change Display Page

This option allows you to change the display page for the Real-

time, Playback and Extraction Modes. It performs the same operation

as Function key 3 (F3) when those modes are active.

4.2.2 Edit Menu

This option brings up the Edit Menu.

4.2.3 Files Menu

This option brings up the Files Menu.

4.2.4 Help Menu

This option displays help information for the function keys.

4.2.5 Jump to DOS

This option allows you to jump into a DOS Shell. You return to

WHATS-UP by typing the DOS command 'EXIT'.

4.2.6 Modes Menu

This option brings up the Modes Menu.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 26

4.2.7 Log Menu

This option brings up the Log Menu.

4.2.8 Orbits Menu

This option brings up the Orbits Menu. This option is only

available if an orbit element file (*.AMS or *.TLE) is present.

4.2.9 Radio Menu

This option brings up the Radio Menu if a Kenwood Radio is present

and the interface is operational.

4.2.10 Spacecraft Menu

This option brings up the Spacecraft Menu.

4.2.11 TNC or PK232 Menu

This option brings up the TNC Menu.

4.2.12 Utilities Menu

This option brings up the Utilities Menu.

4.2.13 CW Menu

This option brings up the CW Menu.

4.2.14 Exit to DOS

This option allows you to terminate WHATS-UP and return to DOS.

4.2.15 SAREX Menu

This option brings up the SAREX Menu.

4.3 Modes Menu

When you bring up the Modes menu you will be prompted with the

following options.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 27

E Extract From Playback File

I Interactive Mode

O Orbital Parameters

M Mutual Visibility

P Playback Mode

R Real Time Mode

Type the letter associated with the option to perform it.

Each of the options are described below.

4.3.1 Extract From Playback File

This option activates the Extraction mode.

4.3.2 Interactive Mode

This option activates the Interactive mode.

4.3.3 Orbital Parameters

This option activates the Orbital mode. This option is only active

if Keplerian data files are in the defined directory path.

4.3.4 Mutual Visibility

This option activates the Mutual Visibility mode.

4.3.5 Playback Mode

This option activates the Playback mode.

4.3.6 Real Time Mode

This option activates the Real-time mode.

4.4 Edit Menu

WHATS-UP contains an ASCII text editor suitable for files less than

64k in size. It is built based on Borland's Turbo Pascal Editor

Toolbox and the commands are compatible with Sidekick and Wordstar.

A summary of the editor commands is shown below.

F10           Switch Windows (if more than one window open)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 28

Backspace     Delete left char

Del           Delete Char

Enter         New line

Ins           Toggle insert mode

PgUp          Move cursor up one page

PgDn          Move cursor down one page

^A            Move cursor left one word

^C            Move cursor down one page

^D            Move cursor right one character

^E            Move cursor up one line

^F            Move cursor right one word

^G            Delete one character

^H            Delete left one character

^I            Tab

^L            Find/replace next occurrence

^P            Insert a control character into the text

^M            New line

^N            Insert line

^R            Move cursor up one page

^S            Move cursor left one character

^T            Delete one word after cursor ^V Toggle insert mode

^W            Scroll up

^X            Move cursor down one line

^Y            Delete line at cursor

^Z            Scroll down

^K^B          Mark beginning of block

^K^C          Copy block to position of cursor

^K^D          Save file and exit edit ^K^H Hide block marker

^K^K          Mark end of block

^K^Q          Abandon file and exit edit

^K^R          Read file into screen

^K^T          Mark single word as block

^K^V          Move block to position of cursor

^K^W          Write block to disk file

^K^Y          Delete block

^K 1..9       Set marker 1 .. 9

^Q^A          Find text and replace

^Q^B          Move to beginning of block

^Q^C          Move to end of file

^Q^D          Move to right of line

^Q^E          Move to top of window

^Q^F          Find text

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 29

^Q^I          Toggle autoindent mode

^Q^K          Move to end of block

^Q^R          Move to top of file

^Q^S          Move to left of line

^Q^X          Move to bottom of window

^Q^Y          Delete to end of line

^Q 1..9       Jump to marker 1..9

The following options can be set in the Find/Find and Replace (^Qf

and ^Qa) operations.

#          locates #th occurrence

G          global replace

N          replace without Y/N question

U          ignore upper case/lower case

W          match whole words only

Notes

1 The ^ key in front of a character identifies the character as a

'control' character. To activate it, hold down the Control key

AND the character key.

2 Some of the commands require two keystrokes.

3 Use ^P to embed a control character in the text. For example, if

you are creating a file containing commands to be sent to the

TNC, to enter a control-C character into the file, use the ^P^C

sequence of keystrokes.

WHILE USING THE EDITOR, WHATS-UP can't receive and process

characters from the TNC. All other features ARE INHIBITED OR LOCKED

OUT.

The Edit menu allows you to call up the various files for editing

as shown by this typical display.

D DOVE.DOP

F Any File

K WHATS-UP.TLE

M c:dove.CNF

P Pick ***.D17 File

R c:910313.D17

S WHATS-UP.SYS

T Two Files

W Arrays

X whats-up.txt

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 30

Y Pick ***.CNF File

Z Today's Data

Type the letter corresponding to your choice.

4.4.1 Edit Doppler File

This option lets you edit the file containing Doppler measurements.

4.4.2 Any File

This option lets you edit any file.

4.4.3 Edit Keplerian Element File

This option lets you edit the default file containing Keplerian

data.

4.4.4 Edit Spacecraft Configuration File

This option lets you edit the file containing the spacecraft

configuration data, such as display pages, and automatic radio

tuning options.

4.4.5 Pick Capture-to-disk File

This option lets you pick a capture-to-disk file to be edited. When

you implement this option you will be presented with a list of

available files. Move the cursor down to the desired file and push

the 'Enter' key.

4.4.6 Edit Capture-to-disk File

This option lets you edit the file containing data captured during

the last pass.

4.4.7 Edit WHATS-UP.SYS

This option lets you edit the WHATS-UP.SYS file.

4.4.8 Two Files

This option lets you edit any two files.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 31

4.4.9 Edit Doppler Channel File

This option lets you edit the file containing the channel numbers

of the data to be extracted from the raw telemetry, processed and

written to disk.

4.4.10 Edit Doppler Data File

This option lets you edit the file containing extracted  data.

4.4.11 Pick Spacecraft Configuration File

This option lets you pick a spacecraft configuration file to be

edited. When you implement this option you will be presented with a

list of available files. Move the cursor down to the desired file

and push the 'Enter' key.

4.4.12 Edit Today's Data

This option lets you edit a file containing data captured today.

You will be prompted for the particular spacecraft.

4.5 Files Menu

This menu presents you with the following typical options.

A Change Directory Path

F Change Playback File

S Show Data Files

V View Playback File

Z Show *.D17 Files

1 Show Files for 1 Spacecraft

4.5.1 Change Directory Path

This option allows you to temporarily change the directory path to

the capture-to-disk files.

4.5.2 Change Playback File

This option allows you to change the playback file. To select a

file, move the cursor down to the desired file and push the 'Enter'

key. If you have more files than fit in the window, touch the 'PgDn'

key to display another window full.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 32

4.5.3 View Playback File

This option allows you to view the contents of the playback file.

4.5.4 Show Spacecraft Capture-to-disk Files

This option shows you the names of the capture-to-disk files for

the chosen spacecraft in the default directory path.

4.5.5 Show Data Files

This option shows you the names of ALL the capture-to-disk files in

the default directory path.

4.5.6 Show Files for 1 Spacecraft

This option shows you the names of the capture-to-disk files for a

particular chosen spacecraft in the default directory path. It will

prompt you for the filetype associated with the spacecraft (e.g.

O23, D17).

4.6 Orbits Menu

This menu presents you with the following typical options.

A Pick AMSAT Format Element Set

E Edit WHATS-UP.TLE

L Load Element File

M Set Ref S/C (Mutual Visibility)

N Pick NASA 2 Line Element Set

P Show next pass

S Set Ref = Sun (Mutual Visibility)

V View Spacecraft Orbit Elements

Z Show/hide Sun Data

4.6.1 Pick AMSAT Format Element Set

This option allows you to change the AMSAT Format Keplerian Element

file. To select a file, move the cursor down to the desired file and

push the 'Enter' key. If you have more files than fit in the window,

touch the 'PgDn' key to display another window full.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 33

4.6.2 Edit Default Keplerian Element File

This option lets you edit the default file containing Keplerian

data.

4.6.3 Load Element File

This option loads the data in the element file into WHATS-UP. When

you activate the option you will be prompted as follows.

Which Element File ? WHATS-UP.TLE

WHATS-UP will supply the default name, you may overwrite it to

supply the name of another file. You use this option to load a file

that is not located in the default directory.

4.6.4 Set Ref S/C (Mutual Visibility)

This option lets you set the spacecraft for which mutual visibility

will be calculated (in the mutual visibility mode). The default is

the Sun. To select a spacecraft, move the cursor down to the desired

name and push the 'Enter' key. If you have more spacecraft than fit

in the window, touch the 'PgDn' key to display another window full.

4.6.5 Pick NASA 2 Line Element Set

This option allows you to change the 2 Line Format Keplerian

Element file. To select a file, move the cursor down to the desired

file and push the 'Enter' key. If you have more files than fit in

the window, touch the 'PgDn' key to display another window full.

4.6.6 Show next pass

This option allows you to view the future passes of a spacecraft.

When you activate this option, you will be presented with a menu

window containing a list of spacecraft designators. Move the cursor

to the one of interest and touch the 'Enter' key to select it.

An * will be shown next to the MA column, if the spacecraft will be

in sunlight at the time. A + will be shown if the spacecraft will be

visible. Touch the space key to view a subsequent pass, touch the

'Esc' key to return to the spacecraft choice menu.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 34

4.6.7 Set Ref = Sun (Mutual Visibility)

This option lets you reset the Sun as the spacecraft for which

mutual visibility will be calculated (in the mutual visibility

mode).

4.6.8 View Spacecraft Orbit Elements

This option lets you view the data associated with the spacecraft.

When you activate this option, you will be presented with a menu

window containing a list of spacecraft designators. Move the cursor

to the one of interest and touch the 'Enter' key to select it.

If the default element file is in AMSAT format (*.AMS) the two line

display is not shown.

A typical display is shown below :-

1 20440U 90  5  E 91059.65616971  .00001077  00000-0  44042-3 0  2017

2 20440  98.6806 140.0431 0012003 123.1299 237.1040 14.29083383 57498

Catalog ID:               20440   Apogee:                 803.312

Element Set:                201   Perigee:                786.093

Epoch Year:                1991   Period:                  100.76

Epoch Day:           59.6561697   Semi Major Axis:       7172.862

Drag:                0.00001070

Inclination:            98.6806   Epoch Age:               22.263

RAAN:                  140.0431   Current Date:            81.919

Eccentricity:         0.0012003   Current Orbit #:           6067

Argument of Perigee:   123.1299

Mean Anomaly:          237.1040

Mean Motion:         14.2908338

Epoch Orbit #:             5749

4.6.9 Show/hide Sun Data

This option allows you to display the position of the Sun in the

last row of the orbit display window. It will replace any spacecraft

in that position. The option as an on/off toggle, i.e. push once to

turn on the display, push again to turn it off.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 35

4.7 Radio Menu

This menu is only present if you have a Kenwood Radio defined as

your Radio Receiver for the spacecraft or are using the printer

port. The Radio menu presents you with the following typical

options.

D Turn Doppler Tracking ON

F Set New Frequency

I Change Doppler Interval

M Set Radio Modulation

R Read VFO A Frequency

S Set Default Frequency

V Select VFO A/B

The Radio printer port menu presents you with the following typical

options.

C Set Current Radio Memory

D Step Radio Memory Down

E Set Radio Memory Scan Delay

K Set Scan Radio Memory ON

M Select Radio Memory

U Step Radio Memory Up

The following options are valid for a Radio.

4.7.1 Turn Doppler Tracking ON/OFF

This option lets you turn the Doppler tracking on and off.

4.7.2 Set New Frequency

This option lets you set a new (default) frequency into the radio

and tune to it.

4.7.3 Change Doppler Interval

This option lets you change the time interval (in minutes) between

successive samples of the radio VFO frequency.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 36

4.7.4 Set Radio Modulation

This option lets you change the modulation mode in the radio.

Options are FM, USB, LSB and CW.

4.7.5 Read VFO A Frequency

This option lets you read back the frequency that VFO A is tuned

to, and displays it in the status window.

4.7.6 Set Default Frequency

This option lets you set the radio back to the default frequency

and tune to it.

4.7.7 Select VFO A/B

This option lets you select between the two VFOs in the radio and

tune to it.

4.7.8 Spare - Not used

4.7.9 Spare - Not used

WARNING

There is no way for WHATS-UP to read back the memory positions from

the radio. WHATS-UP consequently assumes synchronization.

4.7.10 Set Current Radio Memory

This option lets you set the radio memory number into WHATS-UP to

synchronize the radio to the software. Set this to the same number

as the memory channel of the radio.

4.7.11 Step Radio Memory Down

This option lets you set the radio memory down one position.

4.7.12 Set Radio Memory Scan Delay

This option lets you change the dwell time the radio waits on each

channel when scanning through WHATS-UP.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 37

4.7.13 Set Scan Radio Memory ON/OFF

This option lets you start and stop scanning through the radio

memories using WHATS-UP to control the scan. When the scan is turned

off, you will be prompted to synchronize the channels (in case of a

slippage). If WHATS-UP is in the scanning mode when a satellite pass

occurs, WHATS-UP will select the appropriate memory channel and

remain on that channel until the pass is over. At that time, it will

resume scanning.

4.7.14 Select Radio Memory

This option lets you step to a memory. You will be prompted to for

the memory number. Once the memory is reached, you will be prompted

for a new number. Enter 0 to remain at the current position.

4.7.15 Step Radio Memory Up

This option lets you set the radio memory up one position.

4.8 Spacecraft Menu

This menu presents you with the following options.

C Show DOVE.CNF

D Default Spacecraft

K picK Spacecraft

M Change Spacecraft

P Pick Ops. Schedule

S Show Ops. Schedule

4.8.1 Show Spacecraft Configuration File

This option allows you to display the spacecraft configuration file

default settings.

4.8.2 Default Spacecraft

This option allows you to override the default spacecraft. WHATS-UP

will select this one when the current pass is over.

4.8.3 picK Spacecraft

This option allows you to choose another spacecraft from a list. To

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 38

select one, move the cursor down to the desired one and touch the

'Enter' key. For example, amongst the default files supplied are

DOVE.CNF and FUJI.CNF. To select the DOVE or the Fuji-20 spacecraft,

move the cursor down to the 'DOVE.CNF' or 'Fuji.CNF' line and touch

the 'Enter' key.

4.8.4 Change Spacecraft

This option allows you to choose another spacecraft. To select a

another one, enter the name of the spacecraft.sys file. For example,

amongst the default files supplied are DOVE.CNF and FUJI.CNF. To

select the DOVE or the Fuji-20 spacecraft, type 'DOVE' or 'Fuji'

4.8.5 Pick Ops. Schedule

This option allows you to pick an operations schedule. (A file with

the default extension of "OPS".) You would use it to look at the

schedule for a spacecraft other than the one currently selected.

4.8.6 Show Ops. Schedule

This option allows you to look at the schedule the spacecraft

currently selected.

4.9 TNC Menu

This menu presents you with the following typical options. The

actual ones will depend on which TNC you have.

A UoSAT ASCII Beacon

B Phase 3 RTTY Beacon

C Set Morse Code (CW)

M Fuji/MicroSat ASCII Packet

O Select AO-13 PSK

P 1200 Baud FM Packet

R Select MFJ 1278 Radio Port

S SARA ASCII Beacon

T Configure PK232

U 9600 Baud Packet

If you select an option that your TNC cannot perform, you will get

an error message.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 39

4.9.1 UoSAT ASCII Beacon

This option will configure the PK-232 to copy the UoSAT-2

telemetry. Note: you require a hardware change in the PK-232 to make

sense of the received data (Section 7.2). You should not use WHATS-

UP to capture AMSAT/UoSAT binary telemetry because WHATS-UP filters

the ^J and ^M (carriage return and line feed characters) from the

incoming datastream.

4.9.2 Phase 3 RTTY Beacon

This option will configure the Multi mode TNC to copy the UoSAT-

OSCAR 13 Baudot Beacon.

4.9.3 Set Morse Code (CW)

This option will configure the Multi mode TNC to copy morse code.

You use this if you want to copy morse code telemetry. Note the

decoding formats are not provided in this program, so you will have

to decode the telemetry in some other way, such as by hand or by

means of a spreadsheet.

4.9.4 Fuji/MicroSat ASCII Packet

This option will configure the Multi mode TNC to copy the Fuji

ASCII format PACKET telemetry. You should not use WHATS-UP to

capture AMSAT/UoSAT binary telemetry because WHATS-UP filters the ^J

and ^M (carriage return and line feed characters) from the incoming

datastream.

4.9.4 Configure TNC

This option configures the TNC to copy the UI packets transmitted

by the Packet spacecraft or any other configuration defined by the

parameters at the end of the *.CNF file.

4.9.5 FM Packet

This option will configure the Multi mode TNC to copy the DOVE

ASCII format PACKET telemetry. You should not use WHATS-UP to

capture AMSAT/UoSAT binary telemetry because WHATS-UP filters the ^J

and ^M (carriage return and line feed characters) from the incoming

datastream.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 40

4.9.6 SARA ASCII Beacon

This option will configure the PK-232 to copy the SARA ASCII format

300 baud binary telemetry. The option is left in WHATS-UP for

possible future use.

4.9.6 Select MFJ-1278 Radio Port (if TNC is an MFJ 1278)

This option lets you change the MFJ Radio port. WHATS-UP doesn't

change it at any other time.

4.9.7 Select AO-13 PSK (if TNC is a DSP-2232)

This option lets you select 400 baud PSK demodulation.

4.9.8 Select 9600 Baud Packet (if TNC is a DSP-2232 or PK-900)

This option lets you select 9600 baud FM packet demodulation.

4.10 Utilities Menu

This menu presents you with the following typical options.

A Change Directory Path

D Show Space on Disk

E Enable/Disable RS-232 Port to TNC

M Turn MET Window ON/OFF

R Reset Header Counters

S Show Defaults

Z Show Files

* Show Color Chart

! Reconfigure WHATS-UP

4.10.1 Change Directory Path

This option allows you to temporarily change the directory path to

the spacecraft capture-to-disk, configuration and schedule files.

4.10.2 Enable/Disable RS-232 Port

This option allows you to temporarily enable or disable the serial

port to the TNC. This feature is for the time you want to use WHATS-

UP in the Orbit Dynamics mode and another program to access the TNC.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 41

4.10.3 Enable/Disable TNC Port

This option lets you enable and disable the PC serial port to the

TNC. Use it to stop WHATS-UP taking control of the port in multi-

tasking environments, so you can run LAN-LINK in one window, and the

Orbit Dynamics display of WHATS-UP in another.

4.10.4 Turn MET Window ON/OFF

This option allows you to temporarily enable or disable the

(Mission Elapsed Time) MET display. The default MET value for each

spacecraft is stored in the spacecraft.CNF file. When you activate

the display you will be prompted to enter the time. Push the 'Enter'

key to accept the default, overwrite the default with any value in

the format YYYY:MM:DD:HH:mm:ss. For example, 06:38:00 on March 2,

1995 is 1995:03:02:06:38:00.

If you enter a '0' the timer will start from 0.

Use the MET counter to display the launch time for a SAREX mission,

or the last memory load time for DO-17 and other microsats.

4.10.5 Show Space on Disk

This option allows you to see how much space is left on a disk with

exiting from the program.

4.10.6 Reset Header Counters

This option applies to spacecraft transmitting packetized

telemetry. When activated, the option resets the packet counters to

zero. Use this before a pass to see how many packets of each type

are received during the pass.

4.10.7 Show Defaults

This option allows you to display the WHATS-UP default settings.

4.10.8 Show Files

This option lets you display the files in the default directory

path. Use this if WHATS-UP tells you that a file does not exist and

you are sure that it does.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 42

4.10.9 Show Color Chart

This option allows you to display the color combinations. Use this

to see what how the different color combinations appear on your

screen, note the numbers associated with each color, then exit from

the program and edit the WHATS-UP.SYS file using your editor in its

ASCII (non document)mode to change the colors to those you desire.

4.10.10 Reconfigure WHATS-UP

This option reloads the configurations from the WHATS-UP.SYS file.

Use it after editing the file to see the effect of your changes.

4.11 Debug Menu

This menu is only active when the debug flag is enabled (Alt +=

key). The menu presents you with the following typical options.

C Sound CW String

D Turn Debug OFF

F Set Frequency

I Interrogate Radio

S Show Defaults

T Command Radio

V Speak Frequency

W Identify Radio

4.11.1 CW Tone Test

This option asks you to enter some characters at the keyboard. When

you do so, it then sounds them off in morse code. You use this

option to adjust the speed of the morse code used in the AOS, LOS

and EWT warning signals.

4.11.2 Turn Debug OFF

This option lets you turn the debug flag off.

4.11.3 Set Frequency

This option lets you enter a frequency into the selected VFO.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 43

4.11.4 Interrogate Radio

This option lets you enter a manual command into the radio and see

the reply returned by the radio.

4.11.5 Show Defaults

This option allows you to display the WHATS-UP default settings.

4.11.6 Command Radio

This option lets you enter a manual command into the radio.

4.11.7 Speak Frequency

This option commands the voice module in the radio to announce the

frequency on which it is tuned.

4.11.8 Identify Radio

This option lets you enter the 'identify' command into the radio

and see the reply returned by the radio.

4.12 Log Menu

WHATS-UP automatic logging Packet mode Connects, and satellite

Acquisitions. The Log files are in dBASE 3 format and can be

processed by the LAN-LINK and DBASE Log book Package in PC-HAM for

indexed listings, tracking of DXCC and other AWARDS, etc. This is

ideal for special event stations and DX-peditions, for the whole QSL

process and further statistical analyses of their operation. The

structure of the logbook files are as shown below.

Field     Field Name       Type         Length

1              DATE      Character      8

2              TIME      Character      4

3              BAND      Character      3

4              CALL      Character      10

5              RX        Character      3

6              TX        Character      3

7              MODE      Character      4

8              POWER     Character      4

9              QSLSENT   Character      1

10             QSLRX     Character      1

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 44

11             COMMENTS  Character      20

The size of the COMMENTS field is a compromise. Each field is a

fixed size so that each log entry takes up a minimum of 62

characters worth of space on the disk even if all the character

space is unused. You may change the size of the field if you wish,

but to do that you will have to use dBASE. Note also, that if you do

make the comments field longer, the display will be screwed up.

For the logging feature to work, the logbook files must be present

on the disk. You may create a logbook by accessing the Log menu.

When you bring up the Logbook you will be shown the last screen or

page of log entries. The last entry will be highlighted. At this

time a number of function keys can be used, or you may depress the

"Escape" key to bring up the Log Help Menu. The following functions

are available.

A       Append Entry

E       Edit Log Entry

H       Scan for Log Entry

L       Pack Logbook

S       Scan for Call

U       Toggle Delete Mark

X       eXit Log Menu

Ins      Toggle Insert Mode

End      Show Last Page

Home     Show First Page

PgUp     Page Up

PgDn     Page Down

Up       One Entry

Down     One Entry

4.12.1 Alt-A Append Entry

This function key allows you to manually append an entry to the

log. You can also use this command to enter the odd SSB contact into

the logbook file. If you do, use the characters "SSB" or "FM" for

those voice modes to remain compatible with the rest of the DBASE

logbook package.

When appending or editing an entry, LAN-LINK will automatically

enter the date, time, callsign of the other station, band, mode and

QSL information in upper case.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 45

The comments field in the LAN-LINK logbook file is restricted to a

maximum of 20 characters. If you try and enter more than 20, the

surplus will be ignored.

4.12.2 Alt-E Edit Log Entry

This function key allows you to edit the highlighted entry. Move

the cursor with the arrow keys, and end the edit process by

depressing the "Escape" key. You may use the "QSL" entry to tag the

fact that you have written out or received a QSL card.

4.12.3 Alt-H Scan Log by Call

This function key allows you to scan the logbook by callsign prefix

for the callsign in the highlighted entry in the logbook. This

command displays the contents of the logbook file in a formatted

manner on the screen. It requests the callsign of the log entry to

be displayed. If you want to see entries for particular callsigns or

parts of a callsign, enter those callsigns or the front parts of the

callsign. Valid entries are G, G3, G3Z etc. If you want to see every

entry in the log, don't use this command, use the Page Up and Page

Down keys from the Main Log menu.

4.12.4 Pack Logbook

This function key allows you to remove the logbook entries flagged

for deletion.

4.12.5 Alt-S Scan Log by Call

This function key allows you to scan the logbook by callsign prefix

for the callsign entered with the Alt-C or Alt-E keys. This command

displays the contents of the logbook file in a formatted manner on

the screen. It requests the callsign of the log entry to be

displayed. If you want to see entries for particular callsigns or

parts of a callsign, enter those callsigns or the front parts of the

callsign. Valid entries are G, G3, G3Z etc. If you want to see every

entry in the log, don't use this command, use the Page Up and Page

Down keys from the Main Log menu.

4.12.6 Alt-U Toggle Delete Mark

This function key allows you to mark an entry for deletion, or

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 46

unmark it in case you change your mind. Entries marked for deletion

will be flagged by an "*" character next to the record number.

4.12.7 Alt-X eXit Log

This function key lets you exit from the Logbook.

4.12.8 Ins Toggle Insert Mode

This function key allows you to turn the insert mode ON and OFF for

the edit/append functions.

4.12.9 End Show Last Page

This function key moves you to the last callsign on the last page

of the logbook.

4.12.10 Home Show First Page

This function key moves you to the first callsign on the first page

of the logbook.

4.12.11 PgUp Move Up One Page

This function key moves the display up one page of the logbook.

4.12.12 PgDn Move Down One Page

This function key moves the display down one page of the logbook.

4.12.13 Up Arrow Move Up One Entry

This function key moves the highlight up one entry in the logbook.

4.12.14 Down Arrow Move Down One Entry

This function key moves the highlight down one entry in the

logbook.

4.13 SAREX Menu

This menu is designed for use with the Shuttle Amateur Radio

Experiment (SAREX), MIR, and the packet radio Microsats, or any

terrestrial station you want to connect with as soon as they appear

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 47

on frequency.

A Set Attack Mode ON/OFF

C Set SAREX Call

M Attack Mode [QCB]

R Reset SAREX Flags

Z Turn Zap R0MIR-1 ON/OFF

0 Multiple User Connects

4.13.1 Set Attack Mode ON/OFF

If the Attack Mode is set, this option will cause WHATS-UP to issue

a connect request/beacon to the SAREX Call whenever a packet sent to

or from it is heard. The mode, is cleared when the connect is made

(and does not retry out) or when the "A" option is selected a second

time. If this mode is enabled, the Alert/SAREX Call prefix shown in

the Status Window will indicate accordingly.

A happy face will be displayed in the status window before the call

once the connect has been achieved.

Be careful using this feature, as it has the potential to cause a

great deal of QRM. It can also be cleared by another station

connecting to you and telling you to ":QRT:".

In the SAREX configuration you are listening on one channel while

transmitting on an other. If this feature is used on a simplex

channel (everyone transmits and listens on the same channel) aimed

at a DX-pedition or the MIR space station, there is a potential

lockout mode, in which any one station sending a connect request to

the desired/SAREX call will trigger the other ones on frequency who

will in turn trigger the first. If this pile up situation occurs, I

hope the DX-station will QSY and leave the automatic stations to do

their thing. IN any event, if you use this feature and cause QRM,

anyone can connect to you with the callsign of the DX station (as a

pirate) and shut you down. You will then not get the coveted DX QSL.

4.13.2 Change SAREX Call

This option lets you temporarily change the SAREX call.

4.13.3 Set Attack Mode Attack Mode [QCB]

This option allows you to configure the SAREX Attack mode to:

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 48

Q   digipeat or call CQ via the SAREX station

C   connect to the SAREX Callsign

B   both at the same time

Use the CQ and beacon features to digipeat via MIR while attempting

a connect, and the connect alone for the Space Shuttle (SAREX). If

the Attack Mode is set, WHATS-UP will either try for a connect or

send an unproto packet containing the CQ text. If you want it to go

through the SAREX Callsign you must configure the UNPROTO parameter

in the TNC. For example, to attempt a digipeat via R2MIR, set

'UNPROTO CQ via R2MIR' from the command mode.

4.13.4 Reset SAREX Flags

This option lets you reset the SAREX Connect Status indicators in

the status window.

4.13.5 Turn Zap R0MIR-1 ON/OFF

This option turns the Zap feature ON and OFF.

4.13.6 Single/Multiple User Connects

This option configures the TNC for single or multiple users. Use

single when trying for a SAREX QSO and Multiple when beaconing

through MIR.

4.14 CW Menu

This menu provides you with a smart CW transmit terminal. The

keyboard will transmit letter, numbers and the following special

characters:

Keystroke Abbreviation   Meaning

*         SK        End of QSO

&         AS        Wait

+         AR        End of Message

(         KN        Go ahead designated station

=         BT        Break or pause

>         AA        New line

!         SN        Understand

%         KA        Attention

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 49

The following punctuation characters are supported

?   IMI       .   AAA        ,   MIM        -   DU

:   OS        ;   KR         )   KK         /   DN

"   AF        $   SX         '   WG         _   IQ

The CW menu presents you with the following typical options.

A Sound Text

D Set CW Speed

K Turn CW keyboard ON

N Set CW Note

S Show CW Memories

T Transmit Text

- Slow CW Down

+ Speed CW Up

1..0 Change CW Memory 1..10

WHATS-UP uses timing loops in software to produce the sounds and

key the transmitter. If you have time accessed multi-task switching

on your system, the sounds/key times will be wrong.

The menu options are as follows:

4.14.1 Sound Text

This option asks you to enter a line of text, then sounds it in

morse code. Use this option to test the audio note and speed.

4.14.2 Set CW Speed

This option allows you to change the default morse speed. The

higher the number you enter, the lower the speed. The number you

enter is the delay time of a single bit ("dot" or "space between

dots") in milliseconds.

4.14.3 Turn CW keyboard ON/OFF

This option allows you to turn the morse keyboard on and off. When

the CW keyboard is active:

the greek letter '#�#' will be seen flashing in the Status

window.

any letter or number typed at the keyboard will be output

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 50

to the parallel port keyer bit as well as to the TNC port.

4.14.4 Set CW Note

This option lets you set the audio cw tone used in the computer.

4.14.5 Show CW Memories

This option displays the contents of the CW memories as loaded from

the WHATS-UP.SYS file, or temporarily overridden by the "change

memory contents" option described in Section 4.14.9.

4.14.6 Transmit Text

This option asks you to enter a line of text, then transmits it in

morse code.

4.14.7 Slow CW Down

This option slows the CW rate by 5 milliseconds, each time it is

activated.

4.14.8 Speed CW Up

This option speeds the CW rate by 5 milliseconds, each time it is

activated.

4.14.9 Change CW Memory 1..10

When you enter a number between 1 and 9, or 0, this option asks you

to enter a line of text, and stores it in the CW memory associated

with the number. 0 is Memory number 10. The change is temporary and

will be lost when you terminate the program. For permanent changes,

edit the change into the appropriate line in the WHATS-UP.SYS file.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 51

5.0 Orbital Elements

5.1 Basics

As an object moves in space it is subject to gravity. The object

itself has mass and attracts other objects and is at the same time

attracted by the mass in the other objects. Sir Isaac Newton

formulated the law of gravity, which can be described in the

following manner.

All bodies attract each other with a force called gravitational

attraction. The strength of the mutual attraction between two bodies

is dependent on their masses, and the distance between the bodies.

In fact the closer together that the two bodies are, the greater is

the mutual attraction. Mathematically this can be expressed as the

gravitational attraction between two bodies is directly proportional

to their masses and is inversely proportional to the square of the

distance between them.

Planet Earth is an object moving in space and exerts a

gravitational force. It pulls anything close to it towards the

center of the Earth at an increasing speed. An increasing speed is

known as acceleration. The acceleration due to gravity at the

surface of the Earth is given the value of 1 Gravity (G).

5.2 Orbital Trajectories

If the Planet pulls everything towards itself, what keeps things in

orbit around it? For example, if you throw a rock up into the air,

gravity and air resistance (drag) slow it down and it falls back to

Earth. If you shoot a bullet towards the horizon it will travel much

further than the stone did, but will still fall to Earth (unless it

hits something first).

If you launch a rocket towards the horizon, the rocket will

accelerate as long as the fuel lasts. When the rocket fuel is

exhausted, the rocket will continue to travel in a straight line

until other forces alter its path. If the rocket is low enough, the

molecules of air or the atmospheric drag tends to slow it down. At

any height it will still be pulled back by gravity. The force of

gravity always acts towards the center of the Earth. On its own the

rocket will travel in a straight line. Gravity acting downwards will

curve the path of the rocket around the Earth.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 52

The force of gravity will pull the rocket away from its horizontal

path and cause it to fall in a downwards direction. Now the surface

of the Earth is also curved and also curves away in a downwards

direction. If speed of the rocket is such that the rate of descent

(due to the gravitational attraction of the Earth) is equal to the

curvature of the Earth, the rocket will always remain at the same

height , namely, it will be in a circular orbit. If the rocket does

not have enough speed it will gradually fall back to Earth, and if

it has more speed, it will tend to rise above the Earth.

If the rocket continues to burn fuel the speed of the rocket

increases and the path it takes rises away from the surface of the

Earth. When the fuel is exhausted, gravity is still there and still

slows down the rocket. As it slows down its path curves more sharply

until at its furthest point (apogee) it is traveling parallel to the

surface of the Earth. It is however traveling slower than the speed

necessary to maintain a circular orbit at that altitude and starts

to curve back towards the Earth. The force of gravity now acts in a

downward and (slightly) forward direction and speeds up the rocket

until at its lowest point (perigee) it once again is traveling

parallel to the surface of the Earth but now has enough speed to

rise away the surface. This process repeats each time around and

produces an elliptical orbit.(In this orbit, the center of the Earth

is one focus of the ellipse.

To place a satellite into an orbit, it must be given the right

amount of speed for the desired orbital altitude so that the orbital

velocity at apogee is such that it just balances the gravitational

pull of the Earth. Since the force of gravity decreases with

altitude, the orbital velocity is also different at different

altitudes. Any rocket can lift a small mass to a much higher

altitude than it can lift a large mass. How much and how high will

depend on the rocket itself.

5.3 Types of orbits

Different orbits are used for different purposes. Circular and

elliptical orbits come in various forms depending on the angle that

the plane of the orbit makes with the equator of the Earth. This

angle is known as the angle of inclination of the orbit (with

respect to the equator). A polar orbit has an angle of inclination

such that the spacecraft in that orbit can see the polar regions of

the Earth. A geostationary orbit is one which has an angle of

inclination parallel to the equator and an altitude of 22,240 miles

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 53

(35,790 km) and the spacecraft moves in its orbit at the same speed

as a point on the surface of the Earth below it. The spacecraft thus

appears to be stationary with respect to the Earth below it.

The direction in which the satellite moves around the Earth

determines the type of orbit. One which travels along its orbit in

the same direction as the rotation of the Earth (eastwards) is said

to be in a direct or prograde orbit. One which travels against the

rotation of the Earth (westwards) is said to be in a retrograde

orbit.

The movement of an object in space can be described mathematically.

In the early 17th century, the only known objects in space were the

Sun, the Moon, the Planets, a few comets and the stars. It was then

that Johannes Kepler formulated three laws that first described the

movement of the Planets about the Sun.

5.3.1 Kepler's Laws

Kepler's Laws are stated in the following paragraphs.

1. Each Planet revolves about the Sun in an orbit that forms a

circumference of an ellipse with the Sun at one focus of the

ellipse.

2. The line from the center of the Sun to the center of the Planet

(called the radius vector) sweeps out equal areas in equal

periods of time as the Planet travels along the circumference of

the ellipse.

3. The square of the time taken for a Planet to travel around the

circumference of the ellipse (period of revolution of the orbit

of a Planet) is proportional to the cube of the mean distance of

the Planet from the Sun.

In the first law, the focus within the Sun is actually at the

center of mass of the Earth-Sun system and not at the center of the

Sun.

5.4 Orbital Elements

The position of an object in space can be expressed in terms of its

relationship with other bodies. Each orbit can be described in terms

of a number of parameters which supply enough information to

accurately locate the satellite. Six basic parameters are used to

describe the position of a satellite in an elliptical orbit are

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 54

described in the following paragraphs. They are Angle of

inclination, Right Ascension of Ascending Node (RAAN), Eccentricity,

Semimajor Axis, Argument of Perigee and Epoch time of Ascending

Node.

Consider each one in turn.

5.4.1 Angle of Inclination

The angle of inclination of an orbit is the angle between the plane

of the orbit and the equator of the Earth. A satellite moving in a

direct orbit has an angle of inclination between 0 and 90 degrees,

one moving in a retrograde orbit has an angle of inclination of

between 90 and 180 degrees.

The maximum northern and southern latitudes reached by a satellite

are equal to the angle of inclination of its orbit.

5.4.2 Right Ascension of Ascending Node (RAAN)

While the spacecraft is moving around the Earth, the Earth is at

the same time rotating on its own axis, and is itself traveling in

an orbit about the Sun. The Right Ascension of Node is needed as a

fixed reference point in the sky.

Astronomers use the term celestial sphere to describe the sky for

two reasons. First, from where we are standing on the surface of the

Earth, the sky seems to be painted on the inside surface of a sphere

(with the stars in fixed positions on the sphere). Second, have you

ever known scientists to use a short commonly used word when they

can use long ones?

The orbital plane of a spacecraft intersects the equatorial plane

of the Earth in two places (one on each side of the globe). A line

drawn between these two points and continued out to the celestial

sphere is called the line of nodes. The two points on the line of

nodes where the planes intersect are called the point of nodes. Most

globes (and maps) show the north pole upwards. When the spacecraft

crosses the equatorial plane (passes above the equator) going

northward it is ascending from south to north and that node point is

known as the ascending node. Conversely when the satellite continues

on its way and travels half way around the world it crosses the

other node on the equatorial plane descending to the southern

hemisphere. This second node is called the descending node.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 55

The Earth is in an inclined orbit around the Sun, just like a

satellite is in orbit around the Earth. The Earth has an ascending

and descending node around the Sun, in a similar manner to a

spacecraft in orbit around the Earth. The orbital plane of the Earth

is known as the Ecliptic. The direction in space from the center of

the Earth through the intersection of the ecliptic and the Earth's

equatorial plane out to the celestial sphere is called the vernal

equinox or the First Point of Aries because it points to the

constellation of Aries (which is so far away that (for all practical

purposes) it is in fixed direction).

The angle between the line of nodes for the ascending node of the

orbit of the spacecraft continued out to the celestial sphere and

the vernal equinox when measured in an easterly (right as opposed to

westerly/left) direction along the earth's equator is defined as the

Right Ascension of the Ascending Node (RAAN).

5.4.3 Eccentricity and Semimajor Axis

In a circle, the radius of the circumference is constant. This

means that an object traveling along the circumference is always at

a constant distance from the center or focus of the circle. The

general shape of an orbit is an ellipse. Unlike a circle, an ellipse

has two focal points. The distance between each of the focal points

of the ellipse and an object on the circumference is constant. A

line through the two focal points and the circumference is called

the Semimajor Axis. The longest line perpendicular (at 90 degrees

to) the semimajor axis passing through the circumference of the

ellipse is called the Semiminor Axis.

The mathematical term describing the overall shape of an ellipse is

called Eccentricity. When the eccentricity of an ellipse is 0 the

length of the semimajor axis is equal to the length of the semiminor

axis and the shape being described is a circle. A value of

eccentricity greater than 0 means that there is a difference between

the lengths of the axes and the shape of the ellipse flattens out.

Eccentricity describes the shape of the orbit and the length of the

semimajor axis describes the size of the orbit. If these two

parameters are known, the apogee and perigee values for the orbit

can be calculated.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 56

5.4.4 Decay Rate

The Earth has an atmosphere. It is dense at ground level and thins

out with increasing altitude. The Earth's gravity attracts the

molecules of gas in the atmosphere and stops them from escaping. As

the satellite travels along its elliptical orbit around the Earth,

its altitude changes. When it is close to perigee, it bumps into

molecules of air. The lower it gets the greater the drag on it by

the air. When the orbital radius decreases as a result of drag, the

potential energy of the spacecraft also decreases as it comes closer

to the earth. This decrease in potential energy reappears in the

form of heat energy imparted to the atmosphere and to the skin of

the spacecraft, and in an increase of the kinetic energy of the

spacecraft. It is this latter that causes the velocity of the

spacecraft to speed up.

The rate of change of speed through the atmosphere at the perigee

pass will depend on the type of orbit and on the altitude of the

perigee point. In general, satellites with low perigee points will

be more affected than satellites with higher perigee points. The

effect of this over the long term is to lower the apogee point. Over

the long term, the satellite tends to spend more time in the denser

parts of the atmosphere which then tends to circularize the orbit

(at the perigee) at which point the air drag acts continuously on

the spacecraft and the orbit disintegrates. The rate of change of

the orbit measured at a particular epoch is called the decay rate.

5.4.5 Argument of Perigee

A line drawn between the perigee of an elliptical orbit and the

center of the Earth is called the line of perigee. This line also

passes through the apogee and is the semimajor axis of the orbit.

The angle between the line of perigee and the line of nodes is

called the argument of perigee. It is a measurement of the angular

distance between them and is measured in the ascending direction

from the line of nodes. The argument of perigee thus establishes the

position of the ellipse itself within the orbital plane.

5.4.6 Epoch Time (of Ascending Node) and Revolution Number

The Epoch time is a time when the satellite crosses its perigee

point. This time is given as a Julian date, and is the reference

time for when the orbital elements are valid. The Epoch Revolution

or orbit number is the orbit number (since first perigee crossing)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 57

for which the element set is valid.

5.4.7 The Mean Motion

The mean motion a satellite is a measurement of the number of

orbits completed in a day. It is equal to twice the value of PI

divided by the time that the spacecraft takes to complete one

revolution of its orbit (orbital period).

5.4.8 The Catalog Number

The catalog number is the number given to the object.

5.4.9 Mean Anomaly

As the satellite travels along its orbit, its position changes. The

angle (measured in the direction of forward rotation) between the

position of the spacecraft and the line of perigee is called the

true anomaly. The speed of the spacecraft is different at different

parts of the orbit. Calculations are simpler if the speed is

considered to be constant. The constant value is the average speed

of the spacecraft in its orbit. The mean anomaly is the hypothetical

position of the satellite in its orbit (along the ellipse) if it is

assumed to be traveling at its average speed.

5.5 Anticipated Spacecraft Lifetimes

There is not much point in setting up equipment to receive data

from these spacecraft if they are not going to be around for a

reasonable amount of time. Past experience points to three main

factors limiting the operational life of an OSCAR, namely orbital

decay, battery life, and total radiation dosage.

5.5.1 Orbital Decay

The last OSCAR to plunge back into the earth's atmosphere while

still active was UO-1. It was launched into a 500km orbit, and

lasted eight years. A chart in the Satellite Experimenter's Handbook

shows a lifetime of 40000-50000 days for spacecraft at the Microsat

altitude, or somewhere around 120 years; so orbital decay is not our

main worry. There has been some concern about AO-13's orbit, which

is expected to decay somewhere between 1994 and 1996. AMSAT however

are currently building Phase 3D as a replacement spacecraft so the

investment in receiving equipment will not be in vain.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 58

5.5.2 Battery lifetime

Battery breakdown has caused the demise of all amateur spacecraft

except UO-1. OSCARs 1, 2 and 3 were limited to whatever charge was

in the batteries when launched. Once Solar cells and nickel-cadmium

batteries were flown, the limiting factor became the number of times

the battery cells could be charged and discharged.

The batteries in OSCAR spacecraft in low earth orbit have lasted

between five and eight years. UO-1 was still looking good when it

re-entered the earth's atmosphere after 8 years. UO-2, launched in

March, 1984, has batteries with the same part number as those used

in the Microsats. These latter batteries were procured in much the

same manner, and were matched and tested by the same group of VITA

volunteers in Canada that performed the function for UO-2. The

batteries on UO-2 have shown no signs of weakening after almost six

years in orbit, so the prognosis for UO-2 and the Microsats looks

good.

5.5.3 Radiation Damage

Anything above the protection of the atmosphere is subjected to

exposure to the radiation due to the direct and secondary effects of

high energy particles, from the sun and elsewhere. The part of the

spacecraft most susceptible to such damage is the memory in the on-

board-computer (OBC). Such damage manifests itself as a bit flipping

from a 0 to a 1 or from a 1 to a 0, which is correctable. Since the

memory chips used are byte-wide, many types of single-chip failures

can be avoided in a manner analogous to locking out bad sectors on a

hard disk. UO-2, which uses somewhat similar technology chips, has

survived almost six years despite a failure of one small section of

memory several years ago.

Another part of the OBC is the controller itself which is

susceptible to a particle hitting it in a place that causes

uncorrectable errors, such as a wrong operation internal to the

microprocessor, or a CMOS latchup which can result in a high current

being drawn which overheats the part and destroys it.

The spacecraft are more likely to fail due to the cumulative

effects of this constant bombardment, which is known as total dose.

Once the total dose reaches a certain point, the gates in the

transistors that make up the computer and its memories will no

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 59

longer switch. This situation has occurred in AO-10. Predicting when

this might occur is difficult because of lack of knowledge

concerning two things; the total radiation dose in this orbit and

the tolerance limits for the parts used.

The MicroSats are in a much more benign orbit than AO-10 which with

its 4000 km perigee, spends more time in the Van Allen radiation

belts than was planned. AO-10's memories therefore failed sooner

than hoped, but the other electronics, the transponders, batteries,

and solar arrays live on. Through no-longer actively attitude

controlled, its transponders are still usable many weeks of the year

when its batteries are being charged by the sun.

AO-13 is in a better orbit with a lower perigee, and should not

suffer radiation degradation before other effects shorted its

lifespan.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 60

6.0 The Spacecraft

OSCAR spacecraft downlink signals in the amateur 145 MHz and 430

MHz bands modulated by means of Frequency Shift Keying (FSK),

Frequency Modulation (FM) or Phase Shift Keying (PSK). Some of the

characteristics of the downlinks of suitable OSCARs currently

operational are shown in Table 6-1. UO-2 and AO-13 send back BAUDOT

or ASCII data while AO-16, DO-17, WO-18, LU-19 and FO-20 downlink

packetized telemetry.

Table 6-1 Some of the Characteristics of OSCAR Downlinks.

Spacecraft  Beacon     Modulation      Data    Note

Frequency  Type            Rate

(MHz)

UO-2        145.825     FM    ASCII  1200 Baud   1

435.025     FM    ASCII  1200 Baud   1

AO-13       145.812    FSK    BAUDOT   50 Baud   2

PSK    ASCII   400 Baud   2

435.651    FSK    BAUDOT   50 Baud   2

PSK    ASCII   400 Baud   2

AO-16       437.025    PSK    AX.25  1200 Baud   3

437.025    PSK    AX.25  1200 Baud   3

DO-17       145.825    FM     ASCII  1200 Baud

WO-18       437.100    PSK    AX.25  1200 Baud   3

437.075    PSK    AX.25  1200 Baud   3

LO-19       437.150    PSK    AX.25  1200 Baud   3

437.125    PSK    AX.25  1200 Baud   3

FO-20       435.912    PSK    AX.25  1200 Baud

AO-21       145.987    FM     AX.25  1200 Baud   5,6

SO-23       145.955    FM     ASCII   300 Baud   4,6

Notes

1. Spacecraft also broadcasts bulletins and Various Telemetry

formats.

2. Spacecraft downlink modulation is changed according to a pre-

published schedule.

3. Alternate (back up) beacon frequency, may be used on Wednesdays.

4. Binary telemetry with ASCII identification.

5. Spacecraft also transponds FM signals uplinked on 435.016 MHz

and broadcasts voice bulletins from time to time.

6. Spacecraft no longer active.

Before discussing the equipment needed to receive signals from the

spacecraft, a brief word about the spacecraft themselves is in

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 61

order. Since these OSCARs rode into space as secondary payloads, the

orbits that they are in are close to those of the primary payload

and are not optimized for amateur radio communications. The

exception is AO-13 which contained a motor which was used by radio

amateurs to boost the spacecraft from the orbit the rocket placed it

in into its operational orbit. The ones that are in low earth orbits

can be received with simple equipment, but are in range for short

periods of time, AO-13 in an elliptical orbit is in range for many

hours each day, but needs more sophisticated receiving equipment.

The orbital parameters of the OSCARs under discussion are shown in

Table 6-2.

Table 6-21 Orbital Parameters of the OSCARs

Spacecraft       Apogee   Perigee Inclination  Period

(km)     (km)    (Degrees)   (Minutes)

UoSAT-2            699     670     98.0          98.3

AMSAT-OSCAR 13  39,000   2,500     26.1         686.65  [1]

AMSAT-OSCAR 16     804     780     98.7         100.8

DOVE-OSCAR 17      804     780     98.7         100.8

WEBER-OSCAR 18     804     780     98.7         100.8

LUSAT-OSCAR 19     804     780     98.7         100.8

FUJI-OSCAR 20    1,745     912     99.05        112.0

AMSAT-OSCAR 21   1,010     956     82.9         104.7

SARA-OSCAR 23      774     768     98.5         100.3

Notes

1 686.65 minutes is 11 hrs, 26 min.

6.1 Receiving system components

Consider the different components or building blocks that are used

in the different receiving configurations.

6.1.1 Antennas

Antennas receive signals, and each kind of antenna has some degree

of directive and polarization. When the spacecraft rises above the

local horizon, the ground station experiences acquisition of signals

(AOS). At this time the groundstation is receiving signals coming

from a particular direction (azimuth). As the spacecraft rises in

the sky, the elevation angle of the received signals changes, until

the spacecraft drops below the observer's horizon and the ground

station experiences loss of signals (LOS). As seen from the ground,

the spacecraft rises from a horizon in one direction, travels in an

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 62

arc across the sky and sets at a different horizon in a different

direction. Each pass for each spacecraft is different. Antennas for

receiving signals from spacecraft must thus be able to receive

signals coming in from almost any angle.

Antennas in this context, fall into two categories, omnidirectional

and rotatable. The simple turnstile antenna is horizontally

polarized and has a good response to signals arriving from high

angles and can be built for about $2.00. The ground plane and J Pole

antennas are vertically polarized and have a good response to

signals arriving from low angles. These antennas however do not have

much gain. Yagi Beam Antennas however have gain with respect to the

turnstile or ground plane, but only in specific directions. You can

think of the gain in some directions as being moved into the

direction that the antenna is pointed at. The gain of the antenna

depends on the number of elements in the antenna, and the higher the

gain, the narrower the area of the gain (lobe) is. Consequently,

these beam antennas must be moved to keep the spacecraft in the main

lobe of the antenna. Since the need for keeping the antenna pointed

at the spacecraft depends on the beam width of the antenna, the

lower the gain of the antenna the less accurate the tracking need

be. Luckily the orbits help out in this respect. UO-2 in low earth

orbit, which means it is fast moving, needs only a small amount of

gain, so TV style rotators can be employed to point antennas with

between 2 and 4 elements, while AMSAT-OSCAR 13 which is in an

elliptical orbit, moves so slowly for nearly 8 of its 11 hour orbit,

that again, TV style rotators can be used to point higher gain

antennas with between 8 and 11 elements.

Building your own antennas is an easy and worthwhile project.

Antennas for these OSCARS are simple and not very critical with

respect to the materials used. They can in fact be built from

recycled junk.

6.1.2 Receivers

There are two kinds of Receivers, namely FM and linear. FM

receivers are used for reception of the FM signals from DOVE and UO-

2, while linear receivers are needed for reception of the FSK and

PSK signals from the other spacecraft. All vhf/uhf scanner radios

are FM receivers. The linear receivers need single side band (SSB)

capability, something normally found in short wave communication

receivers. As a result of the growing popularity of amateur

satellite communications, suitable vhf/uhf transmitter-receivers

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 63

(transceivers) have been on the market for several years, however

these transceivers are expensive listing in the $800 to $1200 range.

An alternative approach to reception is to use a short wave

communications receiver listing around $500 together with a front

end downconverter which lists at under $100. The short wave radio

can also be used to tune in, not only the world of amateur radio,

but news broadcasts from overseas; a totally different are of

classroom activity.

"Expensive" is a relative term. These days, many people think

nothing of spending $1000 on a stereo system or on equipment for

photography or other hobbies.

6.1.3 Terminal Units and Modems

Digital radio links work much in the same way as digital signals

are transferred over the telephone line. However in this case,

instead of a phone wire, a radio link is used. Both links use modems

to convert the serial input/output digital RS-232 signals of the

computer to the audio tones used on the communications link.

Packet radio signals are demodulated by a radio modem known as a

Terminal Node Controller (TNC). The device is connected in between

the radio and the computer and provides hams with two way digital

communications. A packet only TNC lists for between $120 and $200.

For reception of the PSK signals from AO-16, WO-18, LO-19 and FO-20,

PSK Modems are available either as add-ons to a regular TNC or as

stand alone units, listing between $150 and $700.

The BAUDOT Radio Teletypewriter (RTTY) signals from AO-13 can be

demodulated by an RTTY Terminal Unit. These devices are listed at

between $100 and $300. On the other hand a multi-mode communications

controller listing between $250 and $700 can be used for AO-13 as

well as DOVE and the other spacecraft. AO-13 downlinks BAUDOT

because that is the most commonly used digital communications mode

used by radio amateurs at high frequencies (short waves)

The modem for UO-2 is a little more difficult, as its ASCII

encoding is the reverse of the standard used in the U.S.A. This is

because the spacecraft was built in the UK and its use of tones to

represent data reflects the encoding used in a popular tape

interface (in the UK) at the time the spacecraft was built (1982-

1984).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 64

Still a do-it-yourself circuit needs a few integrated circuits, is

simple to build, easy to test, and very low cost (under $50).

Summarizing the costs of the items mentioned above, the list prices

fall between a low and high cost depending on the amount you wish to

pay. The summary is shown in Table 6-3. It should be noted that the

high price items may not be better than the cheaper ones,

particularly in the educational environment. This table is of course

only a guide, since you will probably end up with something in

between.

Table 6-3 Range of Equipment List Prices

Item                     Low Price       High Price

Antenna                       $2.00        $100.00

Receiver                    $100.00       $1200.00

Radio Modem (TU/TNC)        $150.00        $700.00

Tracking Software            $25.00        $350.00

Telemetry Decoding Software  $35.00         $35.00

---------------------------------------------------

TOTAL                       $312.00       $2385.00

Consider the equipment needed to receive signals from each of the

spacecraft in turn.

6.2 Receiving Signals from DOVE

DOVE (DO-17) which transmits on a frequency 145.825 MHz and AO-21

which transmits on a frequency of 145.987 MHz are the easiest

spacecraft to receive usable signals from. The frequencies are is

available on most hand held scanners, and signals are strong enough

to be heard on nothing more than the simple antenna provided with

the scanning radio when it is purchased. However, the thrill of

receiving satellite signals wears off very quickly without any means

to know what those signals mean.

A somewhat better system is needed for reliable regular reception

of usable signals. A basic receiving system for DOVE/AO-21 is shown

in Figure 6.1. Signals are strong enough that the ground station

does not need a tracking antenna; an omnidirectional antenna is

sufficient. The antenna can be a ground plane, a turnstile or a J-

pole design. A preamplifier should be used to compensate for any

losses in the cable between the antenna and the receiver, or any

fades in the strength of the received signals. Any scanning radio

which receives narrow band FM can be used as the receiver. This is

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 65

the same type of modulation used on the public service channels. If

the scanner can hear the police and other services and can tune to

145.825 MHz, then it is capable of receiving signals from DOVE. The

digital signals from DOVE are encoded as audio tones and need a

modem to convert them to the RS-232 digital signals that can be

interfaced to the serial port of a PC. This type of modem is known

in Radio Amateur circles as a Terminal Unit (TU).

Figure 6.1 Basic Receiving System for DO-17/AO-21

OMNIDIRECTIONAL

ANTENNA

|

|

\|/

SCANNER

RECEIVER ---------> TNC ---------> COMPUTER

The signals are sent as packets using a modified version of the

X.25 protocol called AX.25. Radio Amateurs use this protocol for

communications, and DOVE employs it for telemetry transmission

purposes so that any Radio Amateur equipped for packet radio

communications is also equipped for receiving signals from DOVE.

6.3 Receiving Signals from UO-2

The same basic radio receiving system used to receive signals from

DOVE can be used to copy the telemetry from UO-2. This spacecraft

however has a lower powered transmitter than that of DOVE and

consequently has a weaker signal strength on the ground. This lower

signal level, coupled with the fact that the modulation is plain

ASCII data means that errors will be seen in the received data due

to signal fades. Better antennas are needed for reliable reception,

and antennas that track or move and always point at the spacecraft

are desirable.

The TU used for UO-2 is different to that used for DOVE due to the

different data encoding (ASCII instead of AX.25).

6.4 Receiving Signals from AO-13

So far all the spacecraft considered have been low earth orbits.

AO-13 however is in an elliptical orbit with a high apogee. It also

downlinks telemetry as BAUDOT and ASCII data. While signals from

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 66

this spacecraft can be heard on the simple DOVE type of receiving

configuration with an omnidirectional receiving antenna, the signals

are weak and barely audible, i.e. they are in the noise and cannot

be received in usable form without a tracking antenna.

6.5 Receiving PSK Modulated Signals in the 70 cm Band

Receiving signals from AO-16, WO-18 and LU-19 as well as from FO-

requires somewhat more complex equipment. These spacecraft transmit

on downlink frequencies in the 70 cm or 430 MHz band. As they use

PSK, the receiver has to be a conventional communications receiver.

This can be either a communications receiver designed for that

frequency range, or a conventional short wave receiver with a front

end down converter. A PSK modem attached to the TU is also required.

Typical receiving configurations for these satellites are shown in

Figures 6-2 and 6-3.

Figure 6-2 Basic Receiving System for PSK Modulation.

OMNIDIRECTIONAL

ANTENNA

|

|

|

\|/             PSK MODEM

VHF/UHF             | |

COMMUNICATIONS      | |

RECEIVER --------> TNC ----> COMPUTER

The difference between the two approaches is that the first uses a

communications receiver designed for the 70 cm band; the second

approach uses a general short wave receiver and a front end down

converter.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 67

Figure 6-3 Alternate Basic Receiving System for PSK Modulation.

OMNIDIRECTIONAL

ANTENNA

|

|

DOWNCONVERTER

|

|

SHORT WAVE                 PSK MODEM

COMMUNICATIONS                  | |

RECEIVER                     | |

|                         | |

|-----------------------> TNC --------------> COMPUTER

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 68

7.0 Decoding Spacecraft Telemetry

Apart from UO-2, WO-18 and SO-23, none of the OSCAR spacecraft are

designed for "Science" purposes. Their telemetry consists of

spacecraft housekeeping parameters, monitoring on-board

temperatures, voltages and currents. While much use can be made of

these data, there isn't much real science data available. Even the

scientific spacecraft are in the main unusable by the average

listener because information about the scientific payload is not

readily available. Let's make a start with these spacecraft, then

look to a follow on activity. An OSCAR does not have to be a

separate spacecraft. The Soviet Union has provided their amateurs

with payload space aboard two of their weather satellites [8]. NASA

could do the same for an amateur scientific spacecraft which would

monitor radiation, the earth's magnetic field and solar activity;

such data being of use to radio amateurs for predicting propagation

and providing schools with data about the earth's environment. NASA

has a 'Mission to Planet Earth' project to provide an Earth

Observation Platform in 1997. An attached secondary payload to that

platform, transmitting packetized scientific telemetry data (with

well publicized formats) in the 145 MHz amateur band or in the 136

MHz scientific band could really bring not only the space program,

but the educational and scientific use of space, into every

educational institution in the country. In the mean time, this

section discusses the usable OSCAR spacecraft and their telemetry

and the corresponding decoding equations.

The satellites have been built by different organizations at

different times and each uses different data formats. SARA used a

300 baud ASCII format. DOVE and FO-20 transmit in an ASCII Packet

format, yet while DOVE transmits the data in Hexadecimal format, FO-

20 uses Decimal Format. AO-16, WO-18 and LO-19 transmit their

telemetry in pure binary format. By using packetization, the data

quality is checked in the link itself and bad packets are not

normally passed to the computer from the TNC. AO-13 does not have

any error checking at all, so it is up to the receiving station to

visually inspect the data before trying to convert it to engineering

units. UO-2 also transmits its telemetry as ASCII text, but the

designers of the spacecraft recognized that the downlink was prone

to error and incorporated a checksum in its data format. This

section discusses the data formats and decoding equations associated

with several of the OSCAR spacecraft.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 69

7.1 DOVE (DO-17)

DO-17 was launched January 22, 1990. Its prime mission is to

provide an easily received Digital Orbiting Voice Encoded beacon for

educational and scientific use. Dr. Torres 'Junior' de Castro,

PY2BJO sponsored the DOVE experiment. DO-17 is licensed in Brazil

with the callsign PT2PAZ. DO-17 was built by AMSAT, occupies less

than a cubic foot of space, masses 8.5 kg and contains a V40

microprocessor and 8 Megabytes of RAM. Essentially it is a loaded PC

Clone in orbit. AO-16, DO-17, WO-18 and LO-19 are commonly known as

Microsats and were constructed as a set by AMSAT during 1989. Each

of the Microsats contains bar magnets which align them along the

earth's magnetic field and is spun around that axis by photon

pressure from the sun acting on the communication antennas which are

painted white on one side and black on the other.

Unfortunately a combination of two on-board hardware failures and

lack of available manpower in AMSAT (a volunteer organization for

all practical purposes) have kept DOVE's voice off the air. At this

time DOVE only transmits packet telemetry. DO-17 transmits telemetry

in several different transfer packets as shown in Figure 7-1.

Figure 7-1 A typical example of a DO-17 Telemetry Frame.

23-Jan-91  02:49:23  DOVE-1*>TIME-1:

PHT: uptime is 173/00:36:26. Time is Wed Jan 23 02:47:30 1991

23-Jan-91  02:49:26  DOVE-1*>TLM:

00:59 01:59 02:87 03:31 04:59 05:5A 06:6E 07:52 08:6D 09:72 0A:A2

0B:DC 0C:E9 0D:D8 0E:02 0F:26 10:CC 11:A8 12:01 13:04 14:AD 15:94

16:98 17:94 18:96 19:98 1A:94 1B:91 1C:9B 1D:98 1E:25 1F:5F 20:BA

23-Jan-91  02:49:27  DOVE-1*>TLM:

21:95 22:82 23:24 24:1E 25:2A 26:01 27:02 28:02 29:01 2A:02 2B:02

2C:01 2D:29 2E:02 2F:9E 30:CA 31:9E 32:11 33:CE 34:C4 35:9A 36:A8

37:A6 38:B6

23-Jan-91  02:49:28  DOVE-1*>STATUS:

80 00 00 8F 00 18 CC 02 00 B0 00 00 0C 0E 3C 05 0B 00 04 04

23-Jan-91  02:49:28  DOVE-1*>LSTAT:

I P:0x3000 o:0 l:13081 f:13081, d:0

23-Jan-91  02:49:28  DOVE-1*>WASH:

wash addr:0680:0000, edac=0xd6

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 70

Telemetry data are transmitted in 57 (3A hex) channels in two

segments in the TLM packets. The format is AA:BB where 'AA' is the

hexadecimal channel number and 'BB' the hexadecimal value of the

telemetry at the time it was sampled.

The telemetry decoding equations for DOVE are shown in Table 7.1-1.

The Equations are in the form of a quadratic equation,

Y = A*N^2 + B*N + C  where: N = Telemetry Count (00 - FF);

A, B, C = Equation Coefficients;

Y = Result (In Specified Units).

Note N must be converted from hexadecimal to decimal units before

performing the calculation. A sample decoded page is shown in Table

7.1-2.

Table 7.1-1 DOVE Telemetry Decoding Equations

Spacecraft:  DOVE-1:                Rev: 1 Date: 1/7/90

HEX   Description:     C:           B:          A:        Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx E/F Audio(W)+0.000       +0.0246      0.000       V(p-p)

1    Rx E/F Audio(N)+0.000       +0.0246      0.000       V(p-p)

2    Mixer Bias V:  +0.000       +0.0102      0.000       Volts

3    Osc. Bias V:   +0.000       +0.0102      0.000       Volts

4    Rx A Audio (W):+0.000       +0.0246      0.000       V(p-p)

5    Rx A Audio (N):+0.000       +0.0246      0.000       V(p-p)

6    Rx A DISC:     +10.427      -0.09274     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +9.6234      -0.09911     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000100    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.05075     0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +101.05      -0.6051      0.000       Deg. C

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 71

15    +X (RX) temp:  +101.05      -0.6051      0.000       Deg. C

16    Bat 1 V:       +1.7932      -0.0034084   0.000       Volts

17    Bat 2 V:       +1.7978      -0.0035316   0.000       Volts

18    Bat 3 V:       +1.8046      -0.0035723   0.000       Volts

19    Bat 4 V:       +1.7782      -0.0034590   0.000       Volts

1A    Bat 5 V:       +1.8410      -0.0038355   0.000       Volts

1B    Bat 6 V:       +1.8381      -0.0038450   0.000       Volts

1C    Bat 7 V:       +1.8568      -0.0037757   0.000       Volts

1D    Bat 8 V:       +1.7868      -0.0034068   0.000       Volts

1E    Array V:       +7.205       +0.07200     0.000       Volts

1F    +5V Bus:       +1.932       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.265       +0.0173      0.000       Volts

21    +10V Bus:      +7.469       +0.021765    0.000       Volts

22    BCR Set Point: -8.762       +1.1590      0.000       Counts

23    BCR Load Cur:  -0.0871      +0.00698     0.000       Amps

24    +8.5V Bus Cur: -0.00920     +0.001899    0.000       Amps

25    +5V Bus Cur:   +0.00502     +0.00431     0.000       Amps

26    -X Array Cur:  -0.01075     +0.00215     0.000       Amps

27    +X Array Cur:  -0.01349     +0.00270     0.000       Amps

28    -Y Array Cur:  -0.01196     +0.00239     0.000       Amps

29    +Y Array Cur:  -0.01141     +0.00228     0.000       Amps

2A    -Z Array Cur:  -0.01653     +0.00245     0.000       Amps

2B    +Z Array Cur:  -0.01137     +0.00228     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: +0.06122     +0.00317     0.000       Amps

2E    BCR Output Cur:-0.01724     +0.00345     0.000       Amps

2F    Bat 1 Temp:    +101.05      -0.6051      0.000       Deg. C

30    Bat 2 Temp:    +101.05      -0.6051      0.000       Deg. C

31    Baseplt Temp:  +101.05      -0.6051      0.000       Deg. C

32    FM TX#1 RF OUT:+0.0256      -0.000884    +0.0000836  Watts

33    FM TX#2 RF OUT:-0.0027      +0.001257    +0.0000730  Watts

34    PSK TX HPA Temp+101.05      -0.6051      0.000       Deg. C

35    +Y Array Temp: +101.05      -0.6051      0.000       Deg. C

36    RC PSK HPA Temp+101.05      -0.6051      0.000       Deg. C

37    RC PSK BP Temp:+101.05      -0.6051      0.000       Deg. C

38    +Z Array Temp: +101.05      -0.6051      0.000       Deg. C

39    S band TX Out: -0.0451      +0.00403     0.000       Watts

3A    S band HPA Temp+101.05      -0.6051      0.000       Deg. C

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 72

Table 7.1-2 Sample Decoded (General Housekeeping) Page of DOVE

Telemetry

PHT: uptime is 177/12:34:12. Time is Sun Jan 27 14:45:16 1991

-X Array Cur   : 0.174 A             Array V        :22.829 V

+X Array Cur   : 0.000 A             +Z Array V     :23.836 V

-Y Array Cur   : 0.000 A             Ext Power Cur  : 0.000 A

+Y Array Cur   : 0.000 A             BCR Input Cur  : 0.480 A

-Z Array Cur   : 0.000 A             BCR Output Cur : 0.314 A

+Z Array Cur   : 0.251 A             BCR Set Point  :   119

IR Detector    :    56               BCR Load Cur   : 0.241 A

+Z Array Temp  :   3.0 C

+Y Array Temp  :   4.8 C              Battery 1 V  : 1.330 V

Battery 2 V  : 1.346 V

+2.5V  VREF    : 2.506 V              Battery 3 V  : 1.337 V

Ground REF     : 0.020 V              Battery 4 V  : 1.325 V

Battery 5 V  : 1.350 V

Bat 1 Temp   :   3.0 C               Battery 6 V  : 1.431 V

Bat 2 Temp   : -24.8 C               Battery 7 V  : 1.343 V

TX#1 RF OUT  :   0.0 W               Battery 8 V  : 1.344 V

TX#2 RF OUT  :   3.7 W

The TIME-1 packets identify the time that the data were downlinked.

The Uptime value in the time packet tells you how long the software

has been running (since last upload).

The STATUS packets contain information about spacecraft on-board

status. These bytes, have in the past, been changed by AMSAT without

prior or post notification to the radio amateur community. The LSTAT

packet contains engineering status information which has not been

published other than that the last value "d:0" means the spacecraft

is not set up as a digipeater.

The WASH packets provide engineering information about the on-board

RAM memory.

7.2 UoSAT-2

UoSAT-2 (UO-2) which was launched March 1, 1984, is similar to and

is a follow on to the now re-entered UO-1. It was designed and built

at the Department of Electronic and Electrical Engineering at the

University of Surrey, England. It was built to develop scientific

experimentation and space education. While much invaluable

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 73

experience has been received by the UoSAT people, not much has been

published in the general educational and radio amateur press about

its on-board experiments and telemetry data formats. As such, apart

from a small group of dedicated users, UO-2 seems to have been

ignored by the majority of radio amateurs and educational

institutions.

UO-2 carries four on-board experiments:- a Digital Communications

Experiment, a Space Dust Experiment, a Charge Coupled Device (CCD)

Video Camera Experiment and a Digitalker Experiment.

The Digital Communications Experiment demonstrated the concept of

store-and-forward digital communications using spacecraft in low

earth orbit. The Space Dust Experiment measures the impact of dust

particles, and calculates the momentum of the particles. The CCD

Video Camera Experiment takes pictures of the earth at a resolution

of 384 x 256 pixels with 128 gray levels. This exper-

iment does not seem to have returned any usable pictures. The

Digitalker Experiment provides clear digitized voice using a fixed

vocabulary and is switched on from time to time.

UO-2 transmits a number of different types of telemetry. WHATS-UP

can only decode and display the real-time telemetry. Should you tune

in signals from UO-2, you may get anything. Hang in there, sooner or

later it will transmit real-time telemetry, if not on one pass, then

on the next. The real-time telemetry is transmitted as 1200 baud

ASCII data using FSK. The sense of the modulation is inverted with

respect to convention due to the wide popularity of a BBC computer

tape interface (which used the inverted modulation) in England at

the time the spacecraft was built. This means that either a special

modem has to be built to receive the data or the PK-232 has to be

modified before it will copy signals from UO-2. This modification is

needed because the PK-232 RXR parameter does not work above 300 baud

due to a hardware limitation. This modification performs the

equivalent of the RXR operation in hardware by taking advantage of

an unused inverter inside the PK-232.

What has to be done is to wire U15 pins 1 and 2 to a switch in

series with the output to JP4. Adding a green LED to show the state

of the switch (at a glance) is optional. The steps are as follows:

1. Drill a 1/4 inch hole in the front panel of the PK-232 above the

red DCD LED.

2. Mount a Double Pole Double Throw (SPDT) switch on the front

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 74

panel of the PK-232 where it will not interfere with other

components (near the upper left corner by the AEA logo above the

threshold control).

3. Carefully cut the circuit board trace connecting U15's pin 6 to

the inside pin of JP4. This trace is easiest to cut where it

comes out from under R20 on the top (component side) of the PC

board. Be sure to double check that this is the correct trace

with an ohm meter. Cut with care (with a sharp X-acto knife) so

adjacent traces are not touched.

4. Solder a jumper wire between U15's pins 1 and 6.

5. Connect the center of one pole on this switch to the inner pin

of JP4 or the trace that goes to it (which was cut to disconnect

it from U15 pin 6).

6. Connect the corresponding switch contact which will be hardware

"RXR OFF" to U15 pin 6.

7. Connect the corresponding switch contact which will be hardware

"RXR ON" to U15 pin 2.

8. Mount a green LED on the front panel of the PK-232 above the red

DCD LED.

9. Wire one leg of the LED to the other pole of the switch, the

other end to a 1K ohm resistance (test the LED first to make

sure you wire it the correct way).

10. Wire the other end of the resistance to +12V near the voltage

regulator. Do not wire it to the battery back up voltage (if the

LED stays lit when you turn the PK-232 off, you wired it

wrongly).

11. Wire one side of the switch to ground so the green LED lights

when the REVERSE position is selected.

To copy the UO-2 FM AFSK ASCII Telemetry on 145.825 Mhz set the new

hardware RXR switch in the "reverse" position. Be sure to return

your RXR switch to "normal" when you want to return to regular

operation, as this switch is in the signal path in all modes when

the PK-232's internal modem is used.

A typical telemetry frame starts with a non printing ASCII

character (1E hexadecimal) followed by the identification and date.

The date code can be deciphered using the following YYMMDDWHHMMSS

format where YY is the last two digits of the year, MM, the month,

DD, the day of the month, W, the day of the week (Sunday = 0), the

remainder being hours,minutes and seconds. All times are given in

UTC. A blank line follows, then follow seven lines worth of ten

channels per line. The format of each line is as shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 75

NNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDC

where NN is the channel number, DDD the data and C a checksum to

validate the data. The checksum is needed because there is no error

checking built into the link. Each channel thus comprises six

digits. The checksum is computed for each channel by 'exclusive

or'ing (XOR) each nibble (4 bits) of each of the 6 characters in

that channel. A zero result means that the data was received

correctly, a non-zero result means the data was corrupted. Figure

7.2-1 contains an example of the raw UO-2 data showing some of the

errors due to noise on the downlink.

Figure 7.2-1 Example of a Received Raw UO-2 Telemetry Data Frame

00519D0141370267650361400404660503;4 6019E07045608040C08036C

10519C11298312000313056114069A15529A!6188;175452185905195058

20519F21220322662223000124001725000726093E27541528564D294681

30519E31041732287C33568B34007035217236276637393D38426B39455E

40649F41117242647343061044162545000146000247444748454949422x

50456251108D52634653284p54663215000056p00357451258447A59460E

60826A615FC1625F4A63334164440265160466174267700668000E69000F

UOSAT-2           9101281004625

7.3 AO-13

AO-13 which was launched June 15, 1988 was built as a joint venture

between radio amateurs in the U.S.A. and in Germany organized as the

Radio Amateur Satellite Corporation (AMSAT). AO-13 is a spin

stabilized long life long range radio amateur communications

satellite which provides daily intercontinental communications

capability for hours at a time. It contains a number of analog and

digital transponders with communications links on several

frequencies. An on-board computer based on the RCA 1802

microprocessor controls the spacecraft and generates the downlink

telemetry. Schedules are published in the amateur radio press which

provide information as to which transponder is active at any time

during the orbit. AO-13 also contains a motor which was used by

radio amateurs to boost the spacecraft from the orbit the rocket

placed it in into its operational orbit.

AO-13 transmits telemetry in a number of ways on two beacons. The

two meter beacon is on a frequency of 145.812 MHz and carries CW, 50

baud RTTY, and 400 baud PSK telemetry according to a published

schedule. WHATS-UP can only decode and display the 50 baud RTTY

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 76

telemetry which is transmitted in the form of Z blocks shown in

Table 7.3-1. The block is identified as a 'Z' block by the letter

'Z' before the words "HI. THIS IS AMSAT OSCAR 13" which identify the

spacecraft.

The block starts with the letter 'Z' in the first line. Time and

status information follows, then the first telemetry channel begins

several lines down and is shown with the value of 193. Six lines of

telemetry, each line containing ten values are transmitted, with a

blank line separating the two halves. Following the telemetry lines,

the spacecraft may transmit plain text information of general

interest. Although the Z block only contains 60 channels, the

equations for decoding all 128 of the telemetry channels are

presented in Table 7-3.2 in which the Channel Numbers are shown in

Hexadecimal.

The Bytes in the block are identified in the followings ways:

C  - unsigned count (0 to +255)

Cs - signed count (-128 to +127, 2s complement)

Cx - signed count (+63 to -192, #3F=+63, #FF=-1, #80=-128,

#7F=-129, #40=-192, modified 2s complement)

All temperature channels are decoded identically using the equation

T = (C-120)/1.71 (in Degrees Centigrade). All channels measuring

currents use a linear equation with different calibrations

constants. Three equations are used providing maximum current values

of 1A, 2.5A and 5A, as follows:

1A:  I = (C-15)*4.854 mA

2.5A:  I = (C-15)*12.135mA

5A:  I = (C-15)*24.27 mA

Table 7.3-1 AO-13 RTTY Telemetry Block

Z  HI. THIS IS AMSAT OSCAR 13

05.02.54  8661

.0086   .0000   .07B9

64  6   0   1   16  218 1

193 170 158 143 181 144 147 140 200 7

147 7   7   7   165 29  100 7   149 7

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 77

10  7   145 115 34  7   153 129 122 180

152 73  7   145 137 55  7   183 136 151

7   154 137 169 211 142 127 100 9   140

161 7   173 149 150 154 14  131 127 210

HI THIS IS AMSAT OSCAR 13 08SEP90

NEW AO13 SCHEDULE FROM 17OCT90 AFTER MOVE TO LON 180  LAT 0

MODE B  MA 000 TO 095

MODE JL MA 095 TO 125

MODE LS MA 125 TO 130

MODE S MA 130 TO 135

MODE BS MA 135 TO MA 140

MODE B  MA 140 TO 256

Table 7.3-2 AO-13 Telemetry Decoding Equations

#    Label          Equation             Remarks

00   Uin-BCR        U=(C-10)*167mV   U-Panel: +0.6V @ 0.35A

+0.7V @ 1A

01   Tx-PWRout-L    Average power=(261-C)^2 / 724 Watts rectified

envelope voltage.

02   T-Rx-U         Temp             Mode-B receiver temperature.

03   ---

04   Uout-BCR       U=(C-10)*79.5mV  BCR output voltage.

05   ---

06   T-TX-U         Temp             Mode-B transmitter temperature.

07   I-14V-ST       5A               Transponder separation bus.

08   U-10V-C        U=(C-10)*53.2mV  Computer supply

09   Press He-Hi    off              Helium tank pressure.

0A   T-IHU          Temp             Integrated Housekeeping Unit

0B   I-14V-S        1A               Separation bus, 14V that

supplies torquer and LIU.

0C   BCR-Oscill1    >~6=OK           BCR status. No count = not

working. Typically C=80.

0D   Press He-Lo    off              Helium regulator output.

0E   T-BCR          Temp             Battery Charge Regulator.

0F   I-10V-C        1A               10V continuous power supply.

10   BCR-Oscill2    As channel 0C

11   Press Tank     off              N2O4 tank pressure.

12   T-SEU          Temp             Sensor Electronics Unit

13   IbatCharge     2.5A             Positive Current to battery.

14   L-Sensor (A)   U=(C-10)*8.53mV  Light-Sensor Antenna Side.

15   Motor Valve    off

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 78

16   T-ABAT1        Temp             Auxiliary battery 1

17   I-BCR-OUT      5A               14V line to battery and other

consumers.

18   L-Sensor (M)   U=(C-10)*8.53mV  Light-Sensor Motor Side.

19   ---

1A   T-ABAT2        Temp             Auxiliary battery 2

1B   I-BCR-IN       Equivalent to total current of all panels,

not available due to sensor failure.

1C   Spin rate      C>131,  Spin rate=479/(C-109)-2 rpm

C<=131, Spin rate=(131-C)*0.85+20 rpm Sensor

angular position oscillator.

Lock indication channel #47.

1D   Rx-L-AGC       Gain reduction=(C-75)^2 / 1125 dB

1E   T-MBAT         Temp             Mean battery (normally in use)

1F   I-Panel6       1A               Solar panel 6

20   Tx-PWRout-U    Average power=(287-C)^2 / 1796 Watts As channel

01.

21   T-He-Tank      Temp

22   T-Panel1       Temp

23   I-Panel5       1A

24   Rx-U-AGC       Gain reduction=(C-71)^2 / 2465 dB

25   T-Tx-L         Temp             Mode-JL transmitter.

26   T-Panel3       Temp

27   I-Panel4       1A

28   ---

29   T-Rx-L         Temp             Mode-L receiver.

2A   T-Panel5       Temp

2B   I-Panel3       1A

2C   U-14V-ST       U=(C-10)*66.8mV  Transponder separation bus.

2D   T-RUDAK        Temp             RUDAK temperature.

2E   T-top          Temp             Arm 1, Antenna side.

2F   I-Panel2       1A

30   U-9V-U         U=(C-10)*54mV    Internal 9 volt bus from Mode-B

transponder.

31   T-wall-arm2    Temp

32   T-bottom       Temp             Arm 1, Motor side.

33   I-Panel1       1A

34   ---

35   T-wall-arm1    Temp

36   T-N2O4         Temp

37   ---

38   U-ABAT         U=(C-10)*78.5mV  Auxiliary battery.

39   T-S-xpnder     Temp             Mode-S transponder.

3A   T-L-Sensor     Temp             Light sensor antenna side.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 79

3B   ---

3C   U-9V-L         U=(C-10)*45.4mV  As channel 30, Mode-L

3D   T-AZ50-Tank    Temp

3E   T-nutation damper  Temp         Arm 3.

3F   ---

40   ES-Sensitivity 2MUX0            Earth Sensor sensitivity

threshold. Bit significance

0 LSB  20mV

1      37mV      Hysteresis

200mV

2      75mV      Threshold

600mV

3     150mV

4     300mV

5     600mV

6     1.2V

7 MSB 2.4V

41   Antenna/SERI   2MUX1            Antenna relays and SERI

resistors.

bit   significance

0 LSB Hi-gain  2m  to U

1     Hi-gain 70cm to L

2     --+ SERI-1  load resistor

for

3     -+! SERI-2  both Light-

Sensors       !! resistance

00                7.5 Ohm

01                3.9 Ohm

10                2.3 Ohm

11                5.9 Ohm

42   RUDAK-Status   2MUX2            IN-B (ex Motor-PWR).

C=82, Standard-ROS  (S)

C=78, Emergency-ROS (N, Not-ROS)

C=80, Primitive-ROS (P)

43   S&RUDAK-CNTL   2MUX3            Mode-S and RUDAK interface.

bit   significance

0 LSB RUDAK OFF

1       "   NMI/

2       "   Byte Clock

3       "   Byte count reset

4      ---

5     Mode-S Beacon ON

6       "    Squelch defeated

7 MSB   "    Squelch Hi Sensitivity

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 80

44   BCR-Sin        2MUX4            Array voltage offset.

U=29.1+(Cs*100)mV  (16.3V...41.8V)

45   BCR-Sout       2MUX5            Battery knee voltage offset.

U=14.98+(Cx*20)mV  (11.14...16.2V = 192...63)

46   BCR-relays     2MUX6

bit   significance

0 LSB BCR-2 ON

1     Auxiliary battery charging

2     Auxiliary battery connected,

Main battery disconnected.

47   SS-1           C=255 or C=0, PLL locked

Sun-Sensor angular position

oscillator, Slit antenna

side.

48   SS-2           Time offset from SS-1

49   Flag-SS        C=1, SS-1        Sun sensor data.

C=2, SS-2

4A   SPIN-RAW                        Raw spin count (1/256).

4B   Sensor-control bit   significance  (OUT4)

0 LSB --+ MUX-CTRL for sensor elec. module

1     -+!

!!

00 - Sun data

01 - spin ref./spin counter

10 - ES lower beam

11 - ES upper beam

2     Earth sensor positive edge select.

(Strobes value of spin count at

transition.)

3     Motor Instrumentation ON.

(Pressure sensors, motor valve

indicator.)

4     0.3V Sun Sensor Sensitivity

5     0.6V  "    "         "

6     1.2V  "    "         "

7 MSB 2.4V  "    "         "

(Max. threshold #F = 1 solar constant)

4C   SS-correction

4D   Last ES-A      Z                last ES-pulse Antenna side.

4E     "            O#               (Orbit number and MA value)

4F   Last ES-M      Z                last ES-pulse Motor side.

50     "            O#               (Orbit number and MA value)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 81

51   Lockoutrange   Within +- C counts from sun sensor pip, earth

sensor handler ignores data (Spin count 1

circle = 256 counts.)

52   ES-A           Strobed spin count at edge selected,

Antenna Side beam.

53   Update Flag1   Indicates update, Antenna Side beam.

54   ES-M           As channel 52     Motor Side beam.

55   Update Flag2   As channel 53      "    "    "

56   S/C STATUS     bit   significance

0 LSB LIU power ON

1     S/A plug status 0=Safe, 1=Arm

2     RUDAK-out (lock)

3     Mode-S Squelch open

4     ---

5     Memory Soft error Counter

6       "         "      "

7 MSB   "         "      "

57   ---

58   ---

59   ---

5A   ---

5B   N              no of 20ms per dot, morse speed.

5C   n              running count of units for morse.

5D   ---

5E   TRANSPONDER    bit   significance  (OUT7)

0 LSB GB OFF

1     GB FSK (1=+170Hz)

2     DPSK OFF

3     EB ON

4     --+ PSK source

5     -+! for GB (EB: don't care)

!!

00 - no PSK

01 - ranging

10 - EB source

6     Low power transponder ON

7 MSB Passband OFF (Beacons and Mode-J +3dB)

5F   ---

60   MODUS          bit   significance  (magnet control)

0     magnet system ON

1     underspin magnet

61   M-Soll         magnet vector desired angle to the despun sun

(clockwise as seen from top, 1 circle = 256)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 82

62   M-Out          bit significance (OUT3, also L,S,J control)

0 LSB polarity Arm 1

1     polarity Arm 2

2     polarity Arm 3

3     Magnet power ON

4     Mode-J ON

5     ---

6     Mode-S ON

7 MSB Mode-L ON

63   O-FRAC-lo      Fractional Z increment in 20ms

64   O-FRAC-hi                       Counts down to 0 from preset

value. 255th Z has different

value of O-FRAC. ~7000

counts/Z.

65   O/256          Z from perigee

66   O#-lo          Orbit number

67   O#-hi

68   UHR            10ms             UTC

69                  sec

6A                  min

6B                  hour

6C                  day              1 January 1978 = AMSAT day 0.

6D                  256day

6E   SU0            10ms             IPS stopwatch 0.

6F                  sec

70                  min

71                  min*256

72   SU1            10ms             IPS stopwatch 1.

73                  sec

74                  min

75                  min*256

76   SU2            10ms             IPS stopwatch 2.

77                  sec

78                  min

79                  min*256

7A   SU3            10ms             IPS stopwatch 3.

7B                  sec

7C                  min

7D                  min*256

7E   Event-Lo                        Used as event ID word in

7F   Event-Hi                        intermediate buffer.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 83

7.4 AO-16, WO-18 and LO-19

AO-16, WO-18 and LO-19 were launched January 22, 1990 together with

DO-17. AO-16 and LO-19 are designed to provide a platform for

experiments with digital store-and-forward communications techniques

as a follow-on to the Digital Communications Experiment of UO-2 11.

LO-19 is sponsored by AMSAT in Argentina. WO-18 is an engineering

project of the Center for Aerospace Studies at Weber State

University in Utah. It has the capability for digital communications

but is not used as such. It contains an on-board video camera which

has returned pictures of the earth using a non standard format

picture transmission format. WO-18 also carries a number of

experiments. The Spectrometer experiment is designed to observe the

spectrum of sunlight reflected off the earth's atmosphere and

surface. The Particle Impact Detector is a piezoelectrical crystal

mounted on the side of the spacecraft which produces an output

voltage each time a microparticle impact occurs. The Magnetometer

Experiment contains two orthogonal flux gate magnetometers. As they

were not calibrated they can only provide information about relative

changes in the magnetic environment of the spacecraft. As in the

case of UO-2, data about the experiments and their telemetry

calibrations is lacking in the general amateur radio press.

These spacecraft downlink telemetry in the 437MHz band similar to

that of DOVE but using a BINARY format in an unpublished and

apparently "changeable at any time without notice" sequence. WHATS-

UP intercepts the binary telemetry and converts it to a pseudo DOVE

format as shown in Table 7.4-1 before decoding and displaying the

information. The decoding equations first published by AMSAT in The

AMSAT Journal are given in Tables 7.4-2, 7.4-3 and 7.4-4.

Table 7.4-1 Example of Pseudo DOVE Display of Intercepted Binary

Telemetry and other intercepted Microsat Packets

27-Jan-91  17:04:22  LUSAT-1*>AMARG:

Jan 23, 1600Z.

AART driver loaded.

Reload will continue in a short time

73, LU7XAC

AMARG Control Team

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 84

27-Jan-91  16:41:50  PACSAT-1*>TIME-1:

PHT: uptime is 004/10:14:01. Time is Sun Jan 27 16:40:51 1991

27-Jan-91  16:41:54  PACSAT-1*>AMSAT:

Jan  22, 8:03 UTC - Reload in progress. Digi on. No BBS till

the weekend. NK6K

27-Jan-91  16:41:54  PACSAT-1*>LSTAT:

I P:0x3000 o:0 l:13884 f:13884, d:1 st:0

27-Jan-91  16:42:48  N8ITP>PACSAT-1*>N8ITP [C]

27-Jan-91  16:42:50  N8ITP>PACSAT-1*>N8ITP (UA)

27-Jan-91  16:42:55  N8ITP>PACSAT-1*>N8ITP:

test1122334455

27-Jan-91  17:04:22  LUSAT-1*>BCRXMT:

vmax=762169 battop=766771 temp=218292

27-Jan-91  17:06:31  LUSAT-1*>LSTAT:

I P:0x3000 o:0 l:13417 f:13417, d:0 st:2

27-Jan-91  17:06:38  LUSAT-1*>TIME-1:

PHT: uptime is 005/14:48:41. Time is Sun Jan 27 17:05:33 1991

27-Jan-91  17:06:41  LUSAT-1*>TLM:

00:DC 01:0E 02:30 03:0F 04:3A 05:10 06:DE 07:11 08:84 09:12

0A:00 0B:13 0C:E4 0D:14 0E:A9 0F:15 10:A8 11:16 12:6C 13:17

14:64 15:18 16:69 17:19 18:69 19:1A 1A:66 1B:1B 1C:6D 1D:1C

1E:5F 1F:1D 20:62 21:1E 22:D9 23:1F 24:62 25:20 26:BA 27:21

28:B0 29:22 2A:79 2B:23 2C:2C 2D:24 2E:24 2F:25 30:28 31:00

14-Feb-91  03:02:48  N4HY>LUSAT-1*>N4HY [D]

14-Feb-91  03:02:49  N4HY>LUSAT-1*>N4HY (UA)

14-Feb-91  03:03:09  LUSAT-1*>COMLUS:

COMMAND 9 ACK

14-Feb-91  03:06:51  LUSAT-1*>STATUS:

31 34 2D 46 65 62

Table 7.4.2 AO-16 Telemetry Decoding Equations

Spacecraft:  PACSAT-1:                            Rev:

Date: 7/25/91

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 85

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:        A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +9.202       -0.08990     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +9.179       -0.09277     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +9.837       -0.08838     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +9.779       -0.09144     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.817      -0.09911     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000250    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    +8.5V Bus:     +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.0500      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +101.05      -0.6051      0.000       Deg. C

15    +X (RX) temp:  +101.05      -0.6051      0.000       Deg. C

16    Bat 1 V:       +1.8225      -0.0038046   0.000       Volts

17    Bat 2 V:       +1.9418      -0.0046890   0.000       Volts

18    Bat 3 V:       +1.8699      -0.0041641   0.000       Volts

19    Bat 4 V:       +1.7403      -0.0032880   0.000       Volts

1A    Bat 5 V:       +1.8792      -0.0042492   0.000       Volts

1B    Bat 6 V:       +2.0499      -0.0054532   0.000       Volts

1C    Bat 7 V:       +1.9062      -0.0045331   0.000       Volts

1D    Bat 8 V:       +1.7536      -0.0033192   0.000       Volts

1E    Array V:       +8.055       +0.06790     0.000       Volts

1F*   +5V Bus:       +2.864583    4.090715E-2 -1.930042E-4 Volts

20*   +8.5V Bus:     +7.720951    +8.25979E-3 -1.76254E-5  Volts

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 86

21*   +10V Bus:      +8.882535    +1.39771E-2  0.000       Volts

22    BCR Set Point: -6.1130      +1.1270      0.000       Counts

23    BCR Load Cur:  -0.0477      +0.00767     0.000       Amps

24    +8.5V Bus Cur: -0.00179     +0.000894    0.000       Amps

25    +5V Bus Cur:   -0.00104     +0.00406     0.000       Amps

26    -X Array Cur:  -0.00995     +0.00243     0.000       Amps

27    +X Array Cur:  -0.02370     +0.00254     0.000       Amps

28    -Y Array Cur:  -0.02220     +0.00273     0.000       Amps

29    +Y Array Cur:  -0.01810     +0.00259     0.000       Amps

2A    -Z Array Cur:  -0.02230     +0.00221     0.000       Amps

2B    +Z Array Cur:  -0.02000     +0.00232     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D*   BCR Input Cur: -2.103334E-2 +3.382738E-3 0.000       Amps

2E*   BCR Output Cur:-7.146611E-3 -5.247935E-5 4.878499E-5 Amps

2F    Bat 1 Temp:    +101.05      -0.6051      0.000       Deg. C

30    Bat 2 Temp:    +101.05      -0.6051      0.000       Deg. C

31    Baseplt Temp:  +101.05      -0.6051      0.000       Deg. C

32    PSK TX RF Out: -0.0291      +0.00361     +0.0000869  Watts

33    RC PSK TX Out: +0.0055      +0.00172     +0.0001180  Watts

34    PSK TX HPA Temp+101.05      -0.6051      0.000       Deg. C

35    +Y Array Temp: +101.05      -0.6051      0.000       Deg. C

36    RC PSK HPA Temp+101.05      -0.6051      0.000       Deg. C

37    RC PSK BP Temp:+101.05      -0.6051      0.000       Deg. C

38    +Z Array Temp: +101.05      -0.6051      0.000       Deg. C

39    S band HPA Temp:  0.00       1.0000      0.000       Counts

3A    S band TX Out: -0.0088      +0.00435     0.000       Watts

* Revised in this release

Table 7.4.3 WO-18 Telemetry Decoding Equations

Spacecraft:  WEBER-1:                            Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 87

HEX   Description:        C:           B:          A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +11.087      -0.08949     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +10.322      -0.09448     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +10.348      -0.09004     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +11.387      -0.09535     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.746      -0.09348     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.03523     0.000       Volts

B    +5V Rx Current:+0.000       +0.000234    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0133      0.000       Volts

D    8.5V BUS:      +0.000       +0.0524      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000033    0.000       Amps

10    +10V Bus:      +0.000       +0.0767      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +100.01      -0.5980      0.000       Deg. C

15    +X (RX) Temp:  +100.01      -0.5980      0.000       Deg. C

16    Bat 1 V:       +1.8292      -0.0037196   0.000       Volts

17    Bat 2 V:       +1.8202      -0.0036943   0.000       Volts

18    Bat 3 V:       +1.8050      -0.0036721   0.000       Volts

19    Bat 4 V:       +1.8576      -0.0038979   0.000       Volts

1A    Bat 5 V:       +1.8095      -0.0037439   0.000       Volts

1B    Bat 6 V:       +1.8979      -0.0041754   0.000       Volts

1C    Bat 7 V:       +1.8246      -0.0038126   0.000       Volts

1D    Bat 8 V:       +1.7486      -0.0030475   0.000       Volts

1E    Array V:       +7.800       +0.06790     0.000       Volts

1F    +5V Bus:       +1.838       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.793       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

22    BCR Set Point: -6.1963      +1.1277      0.000       Counts

23    BCR Load Cur:  -0.0405      +0.00620     0.000       Amps

24    +8.5V Bus Cur: +0.00384     +0.000830    0.000       Amps

25    +5V Bus Cur:   -0.00763     +0.00394     0.000       Amps

26    -X Array Cur:  -0.00140     +0.00210     0.000       Amps

27    +X Array Cur:  +0.00946     +0.00226     0.000       Amps

28    -Y Array Cur:  -0.01018     +0.00224     0.000       Amps

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 88

29    +Y Array Cur:  -0.01168     +0.00239     0.000       Amps

2A    -Z Array Cur:  -0.01516     +0.00237     0.000       Amps

2B    +Z Array Cur:  -0.02111     +0.00239     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.02189     +0.00332     0.000       Amps

2E    BCR Output Cur:-0.03019     +0.00327     0.000       Amps

2F    Bat 1 Temp:    +100.01      -0.5980      0.000       Deg. C

30    Bat 2 Temp:    +100.01      -0.5980      0.000       Deg. C

31    Baseplate Temp:+100.01      -0.5980      0.000       Deg. C

32    PSK TX RF Out: +0.2104      -0.01203     +0.0001786  Watts

33    RC PSK TX Out: +0.0340      -0.00969     +0.0002198  Watts

34    PSK TX HPA Temp+100.01      -0.5980      0.000       Deg. C

35    +Y Array Temp: +100.01      -0.5980      0.000       Deg. C

36    RC PSK HPA Temp+100.01      -0.5980      0.000       Deg. C

37    RC PSK BP Temp:+100.01      -0.5980      0.000       Deg. C

38    +Z Array Temp: +0.0000      +1.0000      0.000       Counts

Table 7.4.5 LO-19 Packet Telemetry Decoding Equations

Spacecraft:  LUSAT-1:                             Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:         A:       Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +9.802       -0.08779     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +8.429       -0.09102     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +9.291       -0.08317     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +9.752       -0.08310     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.110      -0.08610     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000250    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 89

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.0508      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +93.24       -0.5609      0.000       Deg. C

15    +X (RX) Temp:  +93.24       -0.5609      0.000       Deg. C

16    Bat 1 V:       +1.7343      -0.0029740   0.000       Volts

17    Bat 2 V:       +1.7512      -0.0032113   0.000       Volts

18    Bat 3 V:       +1.7790      -0.0034038   0.000       Volts

19    Bat 4 V:       +1.7286      -0.0030036   0.000       Volts

1A    Bat 5 V:       +1.8114      -0.0036960   0.000       Volts

1B    Bat 6 V:       +1.7547      -0.0032712   0.000       Volts

1C    Bat 7 V:       +1.7151      -0.0030739   0.000       Volts

1D    Bat 8 V:       +1.6846      -0.0028534   0.000       Volts

1E    Array V:       +8.100       +0.06790     0.000       Volts

1F    +5V Bus:       +2.035       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.614       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

22    BCR Set Point: +3.7928      +1.0616      0.000       Counts

23    BCR Load Cur:  -0.0244      +0.00628     0.000       Amps

24    +8.5V Bus Cur: +0.00412     +0.000773    0.000       Amps

25    +5V Bus Cur:   +0.02461     +0.00438     0.000       Amps

26    +X Array Cur:  -0.01614     +0.00232     0.000       Amps

27    -X Array Cur:  -0.01158     +0.00238     0.000       Amps

28    -Y Array Cur:  +0.00278     +0.00206     0.000       Amps

29    +Y Array Cur:  +0.00136     +0.00218     0.000       Amps

2A    -Z Array Cur:  +0.00370     +0.00209     0.000       Amps

2B    +Z Array Cur:  -0.00793     +0.00216     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.00901     +0.00283     0.000       Amps

2E    BCR Output Cur:+0.00663     +0.00344     0.000       Amps

2F    Bat 1 Temp:    +93.24       -0.5609      0.000       Deg. C

30    Bat 2 Temp:    +93.24       -0.5609      0.000       Deg. C

31    Baseplt Temp:  +93.24       -0.5609      0.000       Deg. C

32    PSK TX RF Out: +0.1059      +0.00095     +0.0000834  Watts

33    RC PSK TX Out: +0.0178      +0.00135     +0.0000833  Watts

34    PSK TX HPA Temp+93.24       -0.5609      0.000       Deg. C

35    +Y Array Temp: +93.24       -0.5609      0.000       Deg. C

36    RC PSK HPA Temp+93.24       -0.5609      0.000       Deg. C

37    RC PSK BP Temp:+93.24       -0.5609      0.000       Deg. C

38    +Z Array Temp: +93.24       -0.5609      0.000       Deg. C

39    LU Bcn Temp A: +93.24       -0.5609      0.000    *  Deg. C

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 90

3A    LU Bcn Temp D: +93.24       -0.5609      0.000    ** Deg. C

3B    Coax Rly Stat: +0.000       +1.0000      0.000       Counts

3C    Coax Rly Stat: +0.000       +1.0000      0.000       Counts

*  Note 1: Thermistor located near box center adjacent to LU

thermistor channel no. 5.

** Note 2: Thermistor located near -X face of box on the experiment

baseplate.

LO-19 also transmits 8 channels of CW telemetry, at 12 W.P.M. with

reduced morse code (to save about 44% power). The format of the

telemetry is:

E LUSAT HI HI NL 111 222 333 444 555 666 777 888

where:

E is a calibration point to measure the output power.

NL:N is the EPROM version number. There are 7 copies of the

program in the EPROM, to prevent crashes or hang ups

due to degradation effects. The on board computer

automatically loads a valid version, and N indicates

which version was loaded.

L is the result of an internal RAM memory test by the 6805

microprocessor. If L=0, the RAM is "OK". If L=E, an

Error was detected.

The telemetry decoding equations for the LO-19 CW are shown in

Table 7.4.6.

Table 7.4.6 LO-19 CW Telemetry Decoding Equations

CH# 1 = N1 +5 Reg Volts  :     636/N1         (Volts)

CH# 2 = N2 +10 Volts Battery   0.064*N2       (Volts

CH# 3 = N3 CW TX Temperature   0.354(134.7-N3) (Deg. C)

CH# 4 = N4 CW TX Power Output  (10.9+N)^2/40.1 (Watts)

CH# 5 = N5 Temp. BOX No. 4     0.356(136-N5)  (Deg. C)

CH# 6 = N6 +10 V Current       0.7*N6         (mAmps.)

CH# 7 = N7 Panel +Z Volts      0.15*N7        (Volts)

CH# 8 = N8 Reg. +8.5 Volts     0.056*N8       (Volts)

A typical example of a CW telemetry frame is shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 91

E LUSAT HI HI 1O 128 167 042 162 040 148 045 156

decoding it, We have: E LUSAT HI HI

NL = 1O :  N=1 (first version) ; L=O (Memory 6805 RAM  "OK")

N1=128: 636/128          = + 4.97  V. REG.

N2=167: 0.064*167        = +10.69  V. BATTERY

N3=042: 0.354(134.7-042) =  32.81  Deg. C

N4=162: (10.9+162)^2/40.1= 745.5   mW.

N5=040: 0.356(136-040)   =  34.17  Deg. C

N6=148: 0.7*148          = 103.6   mA. (over 10 V)

N7=045: 0.15*045         = + 6.75  V. +Z

N8=156: 0.056*156        = + 8.736 V. REG.

NOTE: the reduced cw code is USED ONLY for the numeric data, and

AFTER the "HI HI" as in the following example.

L     U     S     A      T       H    I     H   I      1   O

1: .-      (A)       6: -.... (6)

2: ..-     (U)       7: -...  (B)

3: ...-    (V)       8: -..   (D)

4: ....-   (4)       9: -.    (N)

5: .       (E)       0: -     (T)

7.5 Fuji-OSCAR 20

FO-20 which was launched on February 7, 1990 is a communications

satellite in low earth orbit providing simultaneous analog and

digital communications capability. FO-20 was built in Japan for

Japanese radio amateurs and is the second Japanese built OSCAR.

On February 7 1990, the National Space Development Agency of Japan

(NASDA) put the Marine Observation Satellite (MOS) 1b into orbit.

The launch vehicle also carried two secondary payloads, FO-20 and

the Deployable Boom and Umbrella Test (DEBUT) spacecraft which is

similar in shape and weight to FO-20. MOS-1b was placed into a

circular polar orbit, then DEBUT and Fuji-OSCAR 20 separated from

the launch vehicle at 0233, above Santiago, Chile. First signals

from the spacecraft were received in Tokyo around 0309 UTC.

FO-20 is similar in construction to FO-12. In fact, much of it was

originally constructed as a backup to FO-12 and designated as JAS-

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 92

1B. It has since been modified and improved as a result of the

lessons learned during the flight of FO-12. FO-12 was known as Fuji-

1 in Japan, so this spacecraft is known by the Japanese as Fuji-2

and as FO-20 by the rest of the world.

FO-20's planned service life is 5 years. It is in a sunsynchronous

elliptical polar orbit, having a perigee of about 900 km and an

apogee of about 1740 at an inclination of 99 degrees. The Period of

the orbit is about 105 minutes. This orbit is optimal for MOS-1b

which is to study oceanographic resources and observe agricultural

environmental conditions. In this orbit, the spacecraft passes over

a given line of latitude at approximately the same time each day. In

this orbit, the spacecraft is shielded from the sun by the earth for

about 33% of the time. This eclipse means that the solar cells can

only provide power for about 70 minutes in each orbit and that the

on-board nickel cadmium storage batteries have to power the

spacecraft for the remaining 35 minutes.

FO-20 weighs about 50 kg. and is a polyhedron shaped spacecraft

440mm in diameter and 470mm in height covered by approximately 1500

gallium arsenide solar cells which provide about 11 Watts of power

to keep the 11 series-connected NiCad cells (rectangular) with a

capacity of 6 AH charged. There are 26 sides to the polyhedron which

almost makes it spherical for all practical purposes other than

sticking solar cells to it. FO-12 was the same shape but only

carried about 600 cells. This larger number of cells means that FO-

20 has a positive power budget and should not need to be switched

off to recharge.

The Power supply converts the raw bus voltage of +11 to +18 V (+14

V average) to the three regulated voltages (+10 V, +5 V, -5 V) used

by the rest of the satellite with an efficiency greater than 70%.

The attitude of the satellite is maintained by using the torque

generated by the interaction of two permanent magnets with the

earth's magnetic field. This is a fairly conventional technique used

in the OSCAR series. Temperature stability is achieved by using

thermal insulation.

FO-20 carries two Mode J transponders, both of which may be

operational at the same time. One transponder is analog (Mode JA),

the other is digital (Mode JD).

The frequencies and capabilities of the analog transponder are

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 93

similar to those of FO-12. It consists of an inverted heterodyne

linear translator with a passband 100 kHz wide, operating with a

mode J Uplink passband of 145.9 to 146.00 MHz, and a corresponding

Downlink Passband of 435.9 to 435.8 MHz. The spacecraft has a

Transmitter Output of approximately 1 watt. A ground station needs

an Uplink EIRP of about 100 W to communicate through the

transponder. The JA telemetry beacon is on the nominal frequency of

435.795 MHz with a power output of about 100 mW and can use CW or

PSK modulation. FO-20 is using the callsign 8J1JBS and the beacons

transmit telemetry in the same manner as FO-12.

The digital transponder provides store-and-forward packet

communication using AX.25 link level protocol, version 2. Stations

who used FO-12 are able to use FO-20 without making any

modifications to their equipment. The uplink requires Bi-phased

Manchester code on an FM signal, at a bit rate of 1200 bps. There

are four Uplink Frequencies: 145.85 MHz, 145.87 MHz, 145.89 MHz,

145.91 MHz. The necessary ground station Uplink EIRP is also about

100 W. The transponder has an output power of about 1 W on a

downlink frequency of 435.91 MHz and uses NRZI PSK at 1200 bps. The

same PSK modem used to copy FO-12 or the Microsats is needed to copy

FO-20. The downlink channel also carries packet telemetry.

The on-board 144 MHz receiving antenna is a ring turnstile mounted

at the bottom of the side panels. The 435 MHz transmitting antenna

is a turnstile antenna mounted at the top of satellite. Both

antennas are circularly polarized. Ground tests have shown that the

transmitting antenna is more omnidirectional than that of FO-12,

however due to the structure of the hybrid circuitry which allow

both transponders to share the same antenna, the sense of the

circular polarization on the downlink is different for each mode. As

the apparent polarization is different depending on the geometry

between the spacecraft and the groundstation, you will probably have

to change between left hand and right hand circular polarization

during a pass. The spacecraft is designed so that you can usually

keep the uplink and downlink polarization the same.

Mode JA has provided strong transatlantic signals and many CW and

SSB QSOs. Mode-JD was switched on for the first time during Orbit

#95. To Digipeat via FO-20 you don't need to use a digipeater call.

With the present version of the software, all AX.25 frames with a

valid CRC heard by the spacecraft will be digipeated.

The spacecraft also carries a BBS which is accessed by means of the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 94

same commands used to access a terrestrial WA7MBL/W0RLI/AA4RE type

of BBS. You access the BBS by connecting to 8J1JBS on any of the

four uplink channels. When you do connect to it, make sure that you

disconnect before LOS because FO-20 only allows 16 simultaneous

connections. Stations that hang in there after the satellite drops

below their local horizon block access by other stations and have

been christened 'Zombies'. The BBS program is a modified version of

the BBS program written for FO-12 and allows the use of 4 banks

(1Mbyte) of memory. A typical list of messages copied by KI6QE is

shown in Figure 7.5-1.

Figure 7.5-1 Typical Message Listing from the BBS (copied by KI6QE)

Fuji-OSCAR 20/JAS1b Mailbox ver. 2.00

commands [B/F/H/M/R/U/W]

Use H command for Help

JAS>JAS>NO. DATE   UTC    FROM    TO      SUBJECT

0086 04/13 05:15  WB6GFJ  W6SHP   Welcome

0085 04/13 05:14  WB6LLO  KI6QE   SOFTWARE

0084 04/13 05:14  WB6GFJ  W9FMW   Our Chat

0082 04/13 03:38  W9FMW   WA4EJR  MESSAGE ON CIS

0080 04/13 03:36  KG6EX   N1GCR   From Ashley

0078 04/13 03:32  KG6EX   KD8SI   From Ashley

0077 04/13 03:31  KG6EX   N8AM    From Ashley

0076 04/13 03:30  KG6EX   DD4YR   From Ashley

0075 04/13 03:27  KG6EX   DL1CR   From Ashley

0074 04/13 03:25  KG6EX   G3RUH   From Ashley

The spacecraft telemetry is transmitted either as CW or as PSK. The

CW telemetry monitors 12 analog data points and 33 status points,

the PSK telemetry monitors 29 analog data points and 33 status

points. Telemetry data from Fuji-OSCAR 20 is transmitted on both the

mode JA and JD beacons. Mode JA sends data by Morse code on the

beacon signal of 435.795 MHz, repeating one frame every one minute.

Mode JD sends a telemetry packet every 2 seconds on the digital

downlink channel of 435.91 MHz when the telemetry mode is operating,

otherwise, one frame is downlinked every one minute. The spacecraft

can downlink up to 30 items of data and 31 items of status in the

telemetry. The Mode JA beacon however only carries 12 data elements

and most of status bytes.

Mode JA Telemetry Data The Mode JA beacon transmits the telemetry

data in the format shown below. These data are sent by Morse code

with a "HI HI" at the beginning of each frame, with a speed of about

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 95

100 characters every minute, and always in this format repeatedly.

HI  HI  1A  1B  1C  1D

2A  2B  2C  2D

3A  3B  3C  3D

4A  4B  4C  4D

5A  5B  5C  5D

The number identifies the group, the letters A through D are

decimal values expressed in two digits. Let this two-digit be N, for

each item, true value or engineering value is obtained by decoding N

as shown below. For example, a value of 123 for 1A means group 1 and

23 is the measured value of the solar array current. Groups 4 and 5

contain status information about the bird, where A, B, C and D

represent octal two-digit combinations of 00 through 37. This

corresponds to a combination of five binary digits. Each bit shows

status of each designated item in the order from MSB (Most

Significant Bit) to LSB (Least Significant Bit). The decoding

equations for the CW Mode JA telemetry are shown in Figure 7.5-2.

Figure 7.5-2 Fuji-OSCAR 20 Mode JA Telemetry Conversion Equations

CH  DESCRIPTION                       CALIBRATION   UNITS

1A  total solar array current          19x(N+0.4)   mA

1B  battery charge/discharge current  -38x(N-50)    mA

1C  battery voltage                   (N+4)x0.22    V

1D  center tap voltage of battery     (N+4)x0.1     V

2A  bus voltage                       (N+4)x0.2     V

2B  +5 V regulator voltage            (N+4)x0.062   V

2C  JTA output power                  2.0x(N+4)^1.618mW

2D  calibration voltage               (N+4)/50      V

3A  battery temperature               1.4x(67-N)  deg. C

3B  baseplate temperature #1          1.4x(67-N)  deg. C

3C  baseplate temperature #2          1.4x(67-N)  deg. C

3D  baseplate temperature #3          1.4x(67-N)  deg. C

The status byte conversions are shown in Figure 7.5-3. This method

is used because all items whose status is represented in this manner

only have two possible situations, either ON or OFF, or binary

values 0 or 1. For example, if the first item of status 4A were 423,

the 4 identifies group 4, and the 23 should be thought of as its

equivalent binary code (10011). This shows  the status in the order

of MSB to LSB, or bit 4 to bit 0. Using the decoding data 423 can be

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 96

decoded as follows.

1: Beacon is PSK,

0: Engineering data #2 is blank,

0: Engineering  data  #1 is blank,

1: JTD power is ON,

1: JTA power is ON.

Figure 7.5-3 Fuji-OSCAR 20 Mode JA System Status Bytes

CH BIT  DESCRIPTION     STATE

1     0

4A  0   JTA power       ON    OFF

4A  1   JTD power       ON    OFF

4A  2   Eng. data #1    ---   ---

4A  3   Eng. data #3    ---   ---

4A  4   Beacon          PSK   CW

4B  0   UVC             ON    OFF

4B  1   UVC level       1     2

4B  2   Battery         tric  full

4B  3   Battery logic   tric  full

4B  4   Main relay      ON    OFF

4C  0   PCU             bit 1 (LSB)

4C  1   PCU             bit 2 (LSB)

4C  2   PCU             manual  auto

4C  3   Eng. data #3    ---   ---

4C  4   Eng. data #4    ---   ---

4D  0   Memory bank #0  ON    OFF

4D  1   Memory bank #1  ON    OFF

4D  2   Memory bank #2  ON    OFF

4D  3   Memory bank #3  ON    OFF

4D  4   Computer power  ON    OFF

5A  0   Memory select   bit 1 (LSB)

5A  1   Memory select   bit 2 (MSB)

5A  2   Eng. data #5    ---   ---

5A  3   Eng. data #6    ---   ---

5A  4   Eng. data #7    ---   ---

5B  0   Solar panel #1  lit   dark

5B  1   Solar panel #2  lit   dark

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 97

5B  2   Solar panel #3  lit   dark

5B  3   Solar panel #4  lit   dark

5B  4   Solar panel #5  lit   dark

5C  0   JTA CW beacon   CPU   TLM

5C  1   Eng. data #8    ---   ---

5C  2   Eng. data #9    ---   ---

5C  3   Eng. data #10   ---   ---

5C  4   Eng. data #11   ---   ---

5D  0   Eng. data #12   ---   ---

5D  1   Eng. data #13   ---   ---

5D  2   Eng. data #14   ---   ---

5D  3   Eng. data #15   ---   ---

5D  4   Eng. data #16   ---   ---

Telemetry data are downlinked on Mode JD by means of packets. These

data are transmitted the ASCII format shown in Figure 7.5-4. In the

ASCII telemetry (RA and RB) XXX is a 3 digit decimal number with a

range between 000 to 999. This number represents the value of N in

Table 5 for channels denoted #00 - #26.

Figure 7.5-5 contains the equations for converting the received

data into engineering values. The YYY bytes are three hexadecimal

bytes of system status data, denoted #27a - #29c and can be decoded

as shown in Figure 7.5-6. The SSS byte in the last row are binary

status data, denoted #30a - #39c. Figure 7.5-7 provides the

information needed to decode them in a manner similar to the Mode JA

status points shown in Figure 7.5-3.

Figure 7.5-4. Fuji-OSCAR 20 Mode JD PSK telemetry data format

JAS-1b FF YY/MM/DD HH:MM:SS

XXX XXX XXX XXX XXX XXX XXX XXX XXX XXX

XXX XXX XXX XXX XXX XXX XXX XXX XXX XXX

XXX XXX XXX XXX XXX XXX XXX YYY YYY YYY

SSS SSS SSS SSS SSS SSS SSS SSS SSS SSS

where, FF is the Frame Identifier, which may contain the following

types:

RA: Realtime telemetry, - ASCII

RB: Realtime telemetry, - Binary

SA: Stored telemetry, - ASCII

SB: Stored telemetry, - Binary

M0: Message #0

M1: Message #1

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 98

...........

M9: Message #9

YY/MM/DD is year/month/day, and HH:MM:SS is hour/minute/second, all

in UTC.

Figure 7.5-5. Fuji-OSCAR 20 Mode JD Telemetry Decoding Equations

CH   DESCRIPTION                  CALIBRATION/UNITS

#00  total solar array current     1.91x(N-4)mA

#01  battery charge/discharge     -3.81x(N-508)mA

#02  battery voltage              Nx0.022V

#03  battery center voltage       Nx0.009961V

#04  bus voltage                  Nx0.02021 V

#05  +5 V regulator voltage       Nx0.00620 V

#06  -5 V regulator voltage      -Nx0.00620 V

#07  + 10 V regulator voltage     Nx0.0126  V

#08  JTA output power             5.1x(N-158)mW

#09  JTD output power             5.4x(N-116)mW

#10  calibration voltage #2       N/500 V

#11  offset voltage #1            N/500 V

#12  battery temperature          0.139x(669-N)deg. C

#13  JTD temperature              0.139x(669-N)deg. C

#14  Baseplate Temperature #1     0.139x(669-N)deg. C

#15  Baseplate Temperature #2     0.139x(669-N)deg. C

#16  Baseplate Temperature #3     0.139x(669-N)deg. C

#17  Baseplate Temperature #4     0.139x(669-N)deg. C

#18  temperature calibration #1   N/500 V

#19  offset voltage #2            N/500 V

#20  Solar Cell Panel Temp   #1   0.38x(N-685)deg. C

#21  Solar Cell Panel Temp   #2   0.38x(N-643)

#22  Solar Cell Panel Temp   #3   0.38x(N-646)

#23  Solar Cell Panel Temp   #4   0.38x(N-647)

#24  -------------------------

#25  temperature calibration #2   N/500 V

#26  temperature calibration #3   N/500 V

---------------------------------------------------------

Figure 7.5-6. Fuji-OSCAR 20 Mode JD HEX System Status Bytes

CH   DESCRIPTION

#27a  Spare (TBD)

#27b  Spare (TBD)

#27c  Spare (TBD)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 99

#28a  Spare (TBD)

#28b  Spare (TBD)

#28c  error count of memory unit #0

#29a  error count of memory unit #1

#29b  error count of memory unit #2

#29c  error count of memory unit #3

Figure 7.5-7. Fuji-OSCAR 20 Mode JD BINARY System Status Bytes.

CH    DESCRIPTION             STATE

1      0

#30a  JTA power             on     off

#30b  JTD power             on     off

#30c  JTA beacon            PSK    CW

#31a  UVC status            on     off

#31b  UVC level             1      2

#31c  main relay            on     off

#32a  engineering data #1     -----

#32b  battery status        tric   full

#32c  battery logic         tric   full

#33a  engineering data #2     -----

#33b  PCU status            bit 1 (LSB)

#33c  PCU status            bit 2 (MSB)

#34a  memory unit  #0       on    off

#34b  memory unit  #1       on    off

#34c  memory unit  #2       on    off

#35a  memory unit           on     off

#35b  memory select         bit 1 (LSB)

#35c  memory select         bit 2 (MSB)

#36a  engineering data #3     ------

#36b  engineering data #4     ------

#36c  computer power        on    off

#37a  engineering data #5     ------

#37b  solar panel #1        lit  dark

#37c  solar panel #2        lit  dark

#38a  solar panel #3        lit  dark

#38b  solar panel #4        lit  dark

#38c  solar panel #5        lit  dark

#39a  engineering data #6     ------

#39b  CW beacon source      CPU   TLM

#39c  engineering data #7     ------

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 100

A typical set of raw PSK telemetry packets are those captured by

KI6QE and shown in Figure 7.5-8. A typical decoded and display page

from WHATS-UP is shown in Figure 7.5-9 for a different set of raw

data.

Figure 7.5-8 Fuji-OSCAR 20 PSK telemetry (as copied by KI6QE)

03-Apr-90  17:40:32  8J1JBS*>BEACON:

JAS1b RA 90/04/03 17:45:18

554 433 700 686 757 837 841 823 398 666

617 001 503 516 526 523 526 523 654 000

683 675 685 684 999 643 875 316 002 000

110 111 000 000 100 000 001 011 111 000

03-Apr-90  17:40:34  8J1JBS*>BEACON:

JAS1b RA 90/04/03 17:45:20

566 427 699 705 746 837 841 824 541 659

617 001 503 516 526 523 526 523 654 000

683 675 686 683 999 642 874 316 002 000

110 111 000 000 100 000 001 011 111 000

Figure  7.5-9 Sample Decoded Display (General Housekeeping)

Page from Fuji-OSCAR 20.

JAS1b RA 91/01/13 00:40:58

Solar Panel Temp #1:  15.20 Deg.C Total Array Current:1105.89 mA

Solar Panel Temp #2:  31.92 Deg.C Battery Charge     : 102.87 mA

Solar Panel Temp #3:  32.68 Deg.C Battery Voltage    : 14.806 V

Solar Panel Temp #4:  29.64 Deg.C Battery Center     :  6.744 V

Baseplate Temp. #1 :  40.73 Deg.C Bus Voltage        : 17.259 V

Baseplate Temp. #2 :  41.42 Deg.C   +5 V Regulator   :  5.214 V

Baseplate Temp. #3 :  40.87 Deg.C   -5 V Regulator   :  0.000 V

Baseplate Temp. #4 :  41.14 Deg.C  +10 V Regulator   : 10.471 V

Temperature Cal. #1:   1.30 V     Offset Voltage #1  :  0.000 V

Temperature Cal. #2:   1.29 V     Offset Voltage #2  :  0.000 V

Temperature Cal. #3:   1.75 V     Calibration Volt #2:  1.230 V

Battery Temp.     :  45.04 Deg.C JTA TX Output Power:   0.46 W

JTD Temperature   :  42.12 Deg.C JTD TX Output Power:   3.52 W

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 101

8.0 Spacecraft No Longer Active

This section is provided for post mission analysis of digital data

from spacecraft that are no longer active in case you have access to

data from those satellites and wish to the tools in WHATS-UP to view

and analyze the data.

8.1 Fuji-OSCAR 12

The FO-12 PSK Telemetry Data Format. It is practically identical to

the FO-20 format. The format is shown in Table 8.1-1, the decoding

equations are presented in Table 8.1-2.

Table 8.1-1 Fuji-OSCAR 12 PSK Telemetry Frame Format

Table 8.1-1 Fuji-OSCAR 12 PSK Telemetry Frame Format

JAS-1 FF YY/MM/DD HH:MM:SS

xxx xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx yyy yyy

sss sss sss sss sss sss sss sss sss sss

FF := Frame Identifier   RA: Realtime Telemetry - ASCII

RB: Realtime Telemetry - Binary

SA: Stored Telemetry - ASCII

SB: Stored Telemetry - Binary

M0: Message #0

M1: Message #1

.......

M9: Message #9

YY/MM/DD  =  Date

HH:MM:SS  =  Time (The command station attempts to keep the clock

as close as possible to UTC)

[ Following is valid only for RA and SA frames ]

xxx =  000 - 999  Format:  3  digit  decimal  (Analog  Data)   28

samples  in  row 0 column 0 through row 2  column  7

(denoted #00 - #27 below)

y =  0 - F one byte Hex (System Status Data)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 102

9 samples in row 2 column 8  through  row 2 column 9

(denoted #28a - #29c below)

s =  0 or 1  Binary Status Data

30 samples in row 3  through  row 3 column 9

(denoted #30a - #39c below)

Table 8.1-2 Fuji-OSCAR 12 Telemetry Calibration Equations

Ch.    Item                          Equation    Units

#00     Total Solar Array Current     1.91 * ( N - 4 )    mA

#01     Battery Charge/Discharge      3.81 * ( N - 528 )  mA

#02     Battery Voltage               N * 0.0210    V

#03     Half-Battery Voltage          N * 0.00937   V

#04     Bus Voltage                   N * 0.0192    V

#05     + 5 V. Regulator Voltage      N * 0.00572   V

#06     - 5 V. Regulator Voltage      N * -0.00572  V

#07     +10 V. Regulator Voltage      N * 0.0116    V

#08     JTA Power Output              5.1 * ( N  - 158 )  mW

#09     JTD Power Output              5.4 * ( N - 116 )   mW

#10     Calibration Voltage #2        N / 500  V

#11     Offset Voltage #1             N / 500  V

#12     Battery Temperature           0.139 * ( 689 - N )  Deg. C

#13     JTD Temperature               0.139 * ( 689 - N )  Deg. C

#14     Baseplate Temperature #1      0.139 * ( 689 - N )  Deg. C

#15     Baseplate Temperature #2      0.139 * ( 689 - N )  Deg. C

#16     Baseplate Temperature #3      0.139 * ( 689 - N )  Deg. C

#17     Baseplate Temperature #4      0.139 * ( 689 - N )  Deg. C

#18     Temperature Calibration #1    N / 500  V

#19     Offset Voltage #2             N / 500  V

#20     Facet Temperature #1          0.38 * ( N - 684 )  Deg. C

#21     Facet Temperature #2          0.38 * ( N - 684 )  Deg. C

#22     Facet Temperature #3          0.38 * ( N - 690 )  Deg. C

#23     Facet Temperature #4          0.38 * ( N - 683 )  Deg. C

#24     Facet Temperature #5          0.38 * ( N - 689 )  Deg. C

#25     Temperature Calibration #2    N / 500  V

#26     Temperature Calibration #3    N / 500  V

#27     Depth of Battery discharge    ( N - 500 ) / 189   AH

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 103

Fuji-OSCAR 12 System Status Telemetry Bytes

Ch.    Item

#28a    Spare (TBD)

#28b    Spare (TBD)

#28c    Memory Unit #0 error count

#29a    Memory Unit #1 error count

#29b    Memory Unit #2 error count

#29c    Memory Unit #3 error count

Fuji-OSCAR 12 Binary Status Data Points

Ch.   Item                       1         0

#30a   JTA Power                On        Off

#30b   JTD Power                On        Off

#30c   JTA Beacon               PSK       CW

#31a   UVC Status               On        Off

#31b   UVC Level                1         2

#31c   Main Relay               On        Off

#32a   Engineering Data #1      ----      ----

#32b   Battery Status           Tric      Full

#32c   Battery Logic            Tric      Full

#33a   Engineering Data #2      ----      ----

#33b   PCU Status                Bit 1 (LSB)

#33c   PCU Status                Bit 2 (MSB)

#34a   Memory Unit #0           On        Off

#34b   Memory Unit #1           On        Off

#34c   Memory Unit #2           On        Off

#35a   Memory Unit #3           On        Off

#35b   Memory Select             Bit 1 (LSB)

#35c   Memory Select             Bit 2 (MSB)

#36a   Engineering Data #3      ----      ----

#36b   Engineering Data #4      ----      ----

#36c   Computer Power           On        Off

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 104

#37a   Engineering Data #5      ----      ----

#37b   Solar panel #1           Lit       Dark

#37c   Solar panel #2           Lit       Dark

#38a   Solar panel #3           Lit       Dark

#38b   Solar panel #4           Lit       Dark

#38c   Solar panel #5           Lit       Dark

#39a   Engineering Data #6      ----      ----

#39b   CW beacon source         CPU       TLM

#39c   Engineering Data #7      ----      ----

8.2 SARA-OSCAR 23

On 17 July 1991, the attention of AMSAT and the rest of world's

amateur radio satellite communicators was focused on UoSAT-F. UoSAT-

F became UoSAT 5 when it separated from the launcher and another

OSCAR carrying a packet amateur communications payload was in orbit.

The University of Surrey calls the spacecraft UO-5, AMSAT and radio

amateurs call it OSCAR 22 or UO-22. UO-5/UO-22 is a scientific

satellite similar to UO-4, launched in 1990.

Hardly anyone noticed that the same Ariane launch vehicle carried

the SARA spacecraft, which became OSCAR 23 a few seconds after

UoSAT-F became UoSAT-5. SARA-OSCAR 23 (SO-23) was built at

ESIEESPACE, an aerospace club at the Ecole Superieure d'Ingenieurs

en Electrotechnique et Electronique (ESIEE), in France. SARA is the

culmination of six years of development work which included building

payloads for balloon and sub orbital rocket launches.

SARA is not an amateur radio communications satellite. Although not

as complex, it is also an educational and experimental satellite

similar to the spacecraft built at the University of Surrey in

England, (UO-1, UO-2, UO-3, UO-4 and UO-5). It holds (and identifies

using) the callsign FX0SAT. SARA stands for "Satellite for Amateur

Radio Astronomy".

Radio astronomy was born in 1932 with the discovery by Karl Jansky

at the Bell Telephone Laboratories in New Jersey that radio waves

were coming from a source in the sky. SARA is not amateur radio's

first connection with radio astronomy. In 1937, Grote Reber, W9GFZ,

designed and built the world's the first radio telescope, a 31ft.

(9.4m) dish antenna, in his own backyard in Wheaton, IL. Using this

antenna, he discovered the first discrete radio sources in the sky

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 105

and mapped the distribution of radio emissions in the Milky Way.

Grote Reber was the only person doing research in Radio Astronomy

before World War II and single-handedly brought radio astronomy to

the attention of professional astronomers. Like Grote Reber, SARA is

also a pioneer, albeit with a two fold mission.

8.2.1 The Primary Mission

SARA's primary mission was a Radio Astronomy Experiment to listen

for hf radio signals from Jupiter's radio-electric activities in the

decametric wavelengths. Jupiter is known to be emitting radio noise

in the hf frequency bands. This Jovian DX cannot be investigated by

a terrestrial station because for much of the time, either the Earth

or its atmosphere blocks the signals. In the past, a few other

satellites have measured the Jovian emissions, but not for a long

enough period of time to achieve meaningful results.

Jupiter's radio emissions in the decametric band is powerful enough

to wipe out all other natural extra terrestrial signal sources under

normal conditions. In the vicinity of the earth, the flux received

from Jupiter ranges from 10.E-20 to 5.10.E-19 W.m^-2.Hz^-1 which is

much stronger than the galactic background noise. During the Sun's

calm period, the flux received from the sun is about 10.E-24 W.m^-

2.Hz^-1. During periods of high solar activity it increases to 10.E-

17 W.m^-2.Hz^-1. The solar eruptions can be distinguished from the

Jovian signals by their signal strength, length of time, and by

correlation with signals received on other wavelengths. Jupiter's

radio activity in the decametric band is partially known: above 15

MHz the ionosphere sometimes becomes transparent and makes it

possible to do some measurements, which can then be extrapolated to

the 2 to 15 MHz band.

The Jovian decametric emission is irregular. It occurs mostly

during storms that last from periods ranging from one minute to an

hour. Depending on the type of storm, radio-electrical energy is

concentrated into peaks of 1 to 50 mS or 1 to 10 S, in frequency

bands about 50 KHz wide sweeping across the spectrum. These storms

are closely related to the rotation of Jupiter's satellite Io and to

Jupiter's own rotation. Solar eruptions may also influence them.

Voyager 1 carried out reception tests in the vicinity of Jupiter,

but because of electromagnetic interference from its own instruments

in the decametric band, it could only detect the strongest peaks of

the Jovian emissions. To date, no measurements have yet been made in

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 106

the 2 to 15 MHz band during a period of high solar activity. SARA is

a pioneer following a voyager.

The radio-electrical waves are received by means of three pairs of

aerials placed perpendicularly to each other. This configuration

allows computation of the intensity of the field regardless of its

direction and polarization.

As the received electromagnetic field is quite strong, three pairs

of short aerials, five meters long are used. One of the pairs is

also used to downlink telemetry data to earth in the 2 m band. The

aerials are made of 100 mm wide steel tape. They were rolled up for

the launch and unrolled themselves as soon as they were freed in

orbit.

The Jovian emissions between 2 and 15 MHz are measured on eight

channels, each of them 100 KHz wide. The on board equipment averages

their amplitude over a time interval of 150 S. The average amplitude

produces the envelope of the storms but hides the peaks, which

represent the internal structure of the storms. A single receiver is

switched between the channels and between the three pairs of

aerials. The receiver thus gets in succession the three

polarizations of each of the eight channels. This cycle is executed

several times during the 150 S to prevent the measurements on the

different channels from being separated in time.

The receiver has a 40 dB dynamic range in order to detect the

Jovian peaks as well as certain solar ones without saturating the

receiver. This allows for enough sensitivity to detect the galactic

background noise which is constant at a known level, and which will

serve as a reference standard when Jupiter and the sun are silent.

8.2.2 The Secondary Mission

Consider the reliability of the electronic components in a

spacecraft. SARA uses consumer-made components, rather than military

or space qualified parts, for reasons of cost and availability. Why

do professional satellites use the most expensive components, and

why did SARA do otherwise? The launch causes vibrations amounting to

10 g. Once the payload gets into orbit, outer space is a favorable

environment for electronic equipment. As a result, SARA is made from

components that have been tested and burnt in.

Commercial mass produced components are just as reliable as

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 107

conventional space qualified parts. For instance, a TV set that can

work for 10 years or more without needing repairs has to be built

out of very reliable components. While SARA doesn't use hardened

components, it doesn't use risky components either. Chemical

capacitors are banished, the Power Amplifier transistors are

oversized and well cooled, the PROMs are debugged after programming,

as advised by the manufacturer. SARA's equipment is simple and

conservatively designed and that is why, statistically, a long

operational life can be anticipated for the satellite.

8.2.3 The Downlink

The satellite circles the earth in 100 minutes on its low, sun

synchronous, quasi-polar orbit at an altitude of about 770 km

downlinking its data continuously. One telemetry transfer frame

cycle takes 2 minutes 48 seconds. An uplink command capability can

shut the beacon down if it causes QRM.

The downlink transmitter power of the satellite was about 1 W at

145.955 MHz. The carrier wave was modulated in amplitude with a +/-

3400 Hz spectrum using AFSK coding at frequencies of 1200 and 2200

Hz at a speed of 300 bits/s. The aerial polarization was linear.

8.2.4 The Onboard Electronics

The electronics are controlled by a sequencer because data

acquisition and transmission are made at different rates and times.

All the logic state machines required for the experiment are built

on a single printed circuit card, the remaining cards being analog

or combinational logic. This approach simplified the design of the

equipment and testing.

The sequencing card manages the measurements: it specifies the

frequency and the pair of aerials to be used by the receiver,

digitizes and stores the data, interprets received commands, and

prepares the telemetry. Data stored during a 24 hour time period is

time-tagged and downlinked in telemetry cycles lasting a few

minutes.

The functions described above are performed by a microcontroller.

As the program must operate over a period of years in spite of

errors that could occur due to interference or ionizing particles,

it will be reset regularly.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 108

8.2.5 The Power System

The power system supplies power to the electronics equipment, and

also started the satellite up after the launch, having detected the

separation from the launcher. After a proper time delay, the energy

system freed the aerials and started supplying power to the

equipment. The equipment needs about 3 W and uses a non-regulated

voltage bus that is locally regulated to 5 V.

During certain times of the year, the satellite is eclipsed by the

earth for up to a third of each orbit. A storage battery is thus

used as a permanent power supply. The battery is charged when sun

shines on the photovoltaic cells. The battery is protected against

overloading.

Because the satellite does not have an attitude control system,

sunlight can hit it from any direction. Solar cells were placed on

each side of the space craft, which is in the form of a cube. Its

size was calculated so that each side is able to supply power for

the whole experiment. This means 60% of the surface of the 470 mm

cube was covered with high quality cells.

Separation from the launcher was detected by a push button that

directly controlled the power supply for the on-board experiment and

the cutter to free the aerials. In addition, the power supply is

controlled by a security circuit.

8.2.6 Mechanical integration

Integration is based on a single plate 400 mm by 400 mm, on which

everything else is mounted; the electronic equipment, the aerials,

the solar cells, the interface with the launcher and the manual

controls.

The solar cells are placed on two half shells made of aluminum

sheets that constitute the case when they are brought together. The

integration plate is easily accessible when they are removed. Each

piece of equipment is a box. The cables are directly connected to

the pieces they connect. The whole mechanical structure is made of

aluminum pieces. The junctions that carry strains are soldered. The

spacecraft when assembled is a cube measuring about 470 mm each

side.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 109

8.2.7 Thermal Control

To accomplish their mission, the electronic equipment and the

batteries need moderate temperatures. These temperatures can be

achieved using passive controls. The internal temperature only

depend on exterior conditions (Sun and Earth) and on the satellite's

characteristics.

The target temperature is an average of + 20 C, which can be

achieved by choosing appropriate coatings for the external surfaces

of the satellite. About 40% of the surface was not covered with

solar cells and was thermally coated. The shape of the box was

chosen so as to have a quasi-constant section in all directions,

temperature being then independent from the orientation of the

satellite relative to the sun. The equipment dissipates 3 W and does

not modify the thermal balance of the structure.

The heat must be able to pass easily from the lighted sides to the

dark ones so that temperatures remain close to the average. The

problem is simplified by the small size of the satellite, and by the

use of aluminum shells 2 mm thick which are enough to limit the

thermal gap to 30 degrees. The satellite then does not need to be

spun to maintain thermal control.

8.2.8 Educational Opportunities

SARA provided a unique educational opportunity in orbital dynamics.

The radio signals SARA monitored are generated by the interaction

between Jupiter and and one of its moons (Io) as a beam of radio

energy. Why? How? The signals are synchronized to sidereal time and

appear roughly four minutes earlier successive day. Why? It takes

about two hours for the beam to swing across the earth. Why?

Ideally SARA should have been placed in an orbit in which the earth

never gets between the spacecraft and Jupiter. SARA however took the

orbit the Ariane rocket gave it, an orbit optimized for the primary

payload, the Earth Resources Satellite (ERA-1). When the earth gets

between SARA and Jupiter there will be breaks in reception of Jovian

signals for up to 40 minutes each orbit.

Plot the current positions of Jupiter and the Earth in the solar

system. Look at the angles between Jupiter and the orbital plane of

SARA around the Earth. Does the Earth eclipse SARA? If so, how long

before the geometry is such that SARA will get uninterrupted viewing

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 110

of Jupiter? If SARA has uninterrupted viewing, how long will this

condition last? If SARA receives signals when it is eclipsed by the

earth, what are they and where are they coming from?

8.3 ARSENE

The ARSENE satellite was launched in the spring of 1993. It carried

a conventional AX.25 digipeater using frequency modulated (FM)

uplinks and downlinks. Unfortunately ARSENE's 2 meter downlink

failed during launch. Several stations made contacts via the S Band

transponder until it also failed several months later.

ARSENE began in the early 1980's about the same time as Phase 3.

The spacecraft is built from parts left over from other projects by

students and apprentices. ARSENE was 3 axis stabilized and carried

two transponders, Mode B (digital) and Mode S (analog). The Mode S

downlink is linear on 2446.540 MHz (0.8W). The uplink was centered

on 435.110 MHz with a 16 kHz Bandwidth. The mode B uplinks were

digital at 435.050, 435.100 and 435.150 MHz, the downlink was on

149.975 MHz (15/2W) using standard AX.25 FM equipment. The orbit is

36000 km apogee with a 20000 km perigee at an inclination of 0

degrees. This results in an orbit with an approximate 16 hour

period.

8.3.1 Arsene Telemetry Equations

This information is provided in case anyone has any ARSENE

Telemetry. The ARSENE packet beacon transmitted 30 analog telemetry

channels providing information about the function of the different

on-board modules on the satellite. The telemetry decoding equations

for the analog telemetry channels are shown below.

Channel Units   Parameter                               Equation

A3      I    Battery unload current          y=1.664v +0.198

A4      V    Battery end charge threshold    y=1.008v +12.94

A5      V    Battery voltage                 y=2v +8.72

A6      W    VHF PA output power             y=0.05v^3+0.531v^2+0.25v

A7      V    Primary bus voltage             y=7.152v

A8      I    VHF PA current (10 V)           y=0.365v +0.149

A9      I    VHF PA current (26 V)           y=0.365v +0.149

A11     V    RSSI voltage TC channel         y=v

A13     I    Primary bus current             y=0.619v +0.019

A15     V    First VHF PA voltage            y=9v

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 111

A17     I    Solar generator current         y=0.573v

A18     I    Dc-dc converter input current   y=0.256v +0.02

A19     I    Shunt regulator current         y=0.669v -0.185

A21     I    Battery load current            y=0.174v -0.027

A22     T    Battery temperature             y=14v

A23     T    Solar generator #1 temper.      y= - 0.679033v^5 +

9.57784v^4 -

51.2412v^3 +

127.622v^2 - 170.345v

+ 97.7918

A24     T    Solar generator #5 temper.      y= - 0.679033v^5 +

9.57784v^4 -

51.2412v^3 +

127.622v^2 - 170.345v

+ 97.7918

A38     V    Fifth battery element voltage   y=0.982v + 3.258

A42     T    Plateau temperature             y= - 0.337702v^5 +

4.60193v^4 -

25.4481v^3 +

71.3428v^2

-119.362v + 109.681

A44     T    Shunt regulator temperature     y= - 0.337702v^5 +

4.60193v^4 -

25.4481v^3 +

71.3428v^2 - 119.362v

+ 109.681

A46     T    Solar generator #3 temperature  y= - 0.771085v^5 +

10.2258v^4 -

51.6250v^3+123.311v^2

- 154.219v + 62.7882

A48     T    Electro gates 5-6 temperature   y= - 0.337702v^5 +

4.60193v^4 -25.4481v^3

+ 71.3428v^2

- 119.362v + 109.681

A53     P    Nitrogen tank pressure          y=69.463v

A54     T    VHF PA temperature              y= - 0.337702v^5 +

4.60193v^4 -

25.4481v^3 +

71.3428v^2 - 119.362v

+ 109.681

A55     T    Nitrogen tank #4 temperature    y= - 0.337702v^5 +

4.60193v^4 -

25.4481v^3 + 1.3428v^2

-119.362v + 109.681

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 112

A58     V    RSSI voltage channel 1          y=v

A60     V    RSSI voltage channel 2          y=v

A62     V    RSSI voltage channel 3          y=v

A69     T    Nitrogen tank #1 temperature    y= - 0.337702v^5 +

4.60193v^4 -

25.4481v^3 +

71.3428v^2 - 119.362v

+ 109.681

A71     V    Dosimetre 5 experiment          y=v

The telemetry value of each channel is between 0-255. This

corresponds to a voltage "v" between 0 - 5 Volts. The real value "y"

of measured parameters is calculated from the formulas.

- Voltage V in Volt.

- Current I in Amp.

- Temperature in Celsius.

- Pressure P in Bars and power W in Watt.

The next table gives the normal range and the alarm thresholds of

different channels:

Alarm threshold

Channel Normal Values           Low     High            Parameter

A3      0 - 6.954 A             0       7.054 A         Unload

battery

current

A4      14.850 - 17.055 V       14.850  17.055 V        End charge

battery

threshold

A5      12.089 - 16.386 V       12.011  16.503 V        Battery

voltage

A6      0.859 - 20.539 W        0.859   20.539 W        VHF PA

output power

A7      25.982 - 26.960 V       25.004  27.938 V        Primary bus

voltage

A8      0 - 1.412 A             0       1.466 A         VHF PA

current

(10V)

A9      0 - 1.412 A             0       1.466 A         VHF  PA

current

(26V)

A11     0.195 - 4.688 V         0.039   4.883 V         RSSI voltage

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 113

TC channel

A13     0.055 - 2.002 A         0.043   2.098 A         Primary bus

current

A15     9.668 - 26.895 V        9.141   27.949 V        First VHF PA

voltage

A17     0.705 - 2.596 A         0.504   2.697 A         Solar

generator

current

A18     0.050 - 0.760 A         0.030   0.915 A         Dc-dc

converter

input

current

A19     0.102 - 1.801 A         0.050   2.206 A         Shunt

regulator

current

A21     0.072 - 0.721 A         0.072   0.721 A         Battery load

current

A22     7.227 - 31.563 .xC      6.680   31.836 .xC      Battery

temperature

A23     -70.734 - 9.656 .xC     -80.688 19.609 .xC      Solar

generator #1

temperature

A24     -69.969 - 9.656 .xC     -79.922 20.375 .xC      Solar

generator #5

temperature

A38     5.502 - 7.401 V         5.406   7.497 V         Fifth

battery

element

voltage

A42     -19.672 - 40.125 .xC    -21.016 40.797 .xC      Plateau

temperature

A44     -19.672 - 40.125 .xC    -21.016 40.797 .xC      Shunt

regulator

temperature

A46     -30.422 - 30.047 .xC    -31.766 40.125 .xC      Solar

generator #3

temperature

A48     -19.672 - 40.125 .xC    -21.016 40.797 .xC      Electro gate

5-6

temperature

A53     3.984 - 190.313 Bars    1.641   199.688         Nitrogen

tank

pressure

A54     -17.988 - 41.809 .xC    -19.332 42.481 .xC      VHF PA

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 114

temperature

A55     -19.672 - 40.125 .xC    -21.016 40.797 .xC      Nitrogen

tank #4

temperature

A58     0.195 - 4.688 V         0.039   4.883 V         RSSI voltage

channel 1

A60     0.195 - 4.688 V         0.039   4.883 V         RSSI voltage

channel 2

A62     0.195 - 4.688 V         0.039   4.883 V         RSSI voltage

channel 3

A69     -19.672 - 40.125 .xC    -21.016 40.797 .xC      Nitrogen

tank #1

temperature

A71     0.488 - 0.508 V         0.488   0.508 V         Dosimetre 5

voltage

8.3.2 ARSENE Digital status telemetry

ARSENE packet beacon also transmitted 9 status bytes containing

status information of some of the modules aboard the vehicle.

There are first two words STA and STB that give 15 status bits ST1

to ST16, four words X1 to X4 for message handling and three words

T1, T2 and T3 representing counters.

The next table give the significance of bits ST1-ST16.

SIGNIFICANCE

WORD    Bit     MODULE                  State 1         State 0

STA     ST1     TNC 1                   On              Off

STA     ST2     TNC 2                   On              Off

STA     ST3     TNC 3                   On              Off

STA     ST4     VHF transmission        CIM TLM         Packet

STA     ST5     VHF power               Low             Normal

STA     ST6     Experim. power supply   On              Off

STA     ST7

STA     ST8     Squelch                 Off             On

STB     ST9     Battery regulator       On              Off

STB     ST10    OK to load battery      Yes             No

STB     ST11    Battery loading mode    Normal       Housekeeping

STB     ST12    Packet transponder      On              Off

STB     ST13    Mode B or S             B packet        S Linear

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 115

STB     ST14    10 Volts power supply   On              Off

STB     ST15    High pressure gates     Open            Closed

STB     ST16    ACS power supply        On              Off

Here is an example of the packet beacon telemetry format that had

been received at F6BVP during a test of the ARSENE packet

transponder performed during winter 1991 to validate the circuits

designed by Gerard F6FAO and to verify the software coded by Antoine

F6DWJ.

ARSENE-1>MESURE-3:

1 1X10 2A53 3A69 4A38 5X20 6A54 7A22

2 1X30 2A55 3A71 4A40 5X40 6A56 7A24

3 1STA 2A90 3A17 4A42 5STB 6A58 7A18

4 1A30 2A11 3A19 4A44 5A40 6A60 7T10

ARSENE-1>MESURE-3:

5 1A50 2A13 3A21 4A46 5A60 6A62 7T20

6 1A70 2A15 3A23 4A48 5A80 6A64 7T30

ARSENE

ARSENE

8.4 AO-21 (RM-1)

AO-21 (AO-21) was launched in February 1991 from the North

Cosmodrome at Plesetsk. The orbit is a slightly elliptical polar

orbit with an apogee of 1000 km at an inclination of 83 degrees. The

period of the orbit will be 105 minutes.

AO-21 was the first international OSCAR in which radio amateurs

from the Soviet Union took part. RM1 stands for "RADIO M-1", which

is the official prelaunch name of AO-21, emphasizes that the

spacecraft was built by, and for, Radio Amateurs around the world.

The idea of a joint effort between the two groups, one in the USSR

and the second, in Germany first appeared in the spring 1989. The

discussions about what and how things had to be done lasted till the

meeting of the representatives of the two groups in Surrey in July

1989 when the preliminary agreement about the cooperation was

signed. The final version of the cooperation agreement was later

signed in the autumn of 1989 after much of the work had been

completed.

According to the mutual agreement, Amsat-U-Orbita developed and

made the linear transponder, command radio link, telemetry system,

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 116

power supply system and decided all the problems with the official

and other government organizations about the location of the

equipment and launching.

The RUDAK group of AMSAT-DL developed and built the digital part,

called RUDAK-2 which contains a digipeater and an AX.25 mailbox. It

also provides other possibilities for experiments in transmitting of

information using modern digital methods. It also contains its own

RF input and output circuits.

The ground command station was developed by the Amsat-U-Orbita and

Amsat-U-Sputnik groups. The Rudak group however provided some

special digital part for it.

During the launch and orbital test period ground, the command

stations were UC1CWA situated in Molodechno and RK3KP in Moscow. The

ground command stations for Rudak-2 are situated near Munich at

DK1YQ and near Hanover, at DB2OS. The final agreement was signed on

behalf of Amsat-U-Orbita by the technical director of project

"RADIO-M1" - V.Chepyzhenko, RC2CA; and on behalf of AMSAT-DL, by

their president, K. Meinzer, DJ4ZC. The Project Manager for the

RUDAK-2 is Hanspeter Kuhlen, DK1YQ. The coordinators for the project

are P. Guelzow, DB2OS and L. Labutin, UA3CR.

AO-21 was an attached secondary payload (Piggy-back) aboard a USSR

geological research satellite which provides a Mode B communications

transponder in low earth orbit as well as an orbiting experimental

digital communications capacity.

The AO-21 Specifications are as listed below.

Dimension and shape: Cylinder of height about 4 meters and

diameter 1.8 meters

System configuration: Professional geological research equipment,

telemetry system, command link equipment, transponders and

power supply, thermal control. Amateur linear and digital

transponders, telemetry system, command link equipment, power

supply.

Attitude control: Satellite attitude will be maintained using a

gravity gradient approach in the form of a rod 9 meters long

pointing away from the earth.

Planned service life: 3 years.

Two sets of the equipment are installed aboard the satellite:

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 117

Linear transponder #1 is Mode B and contains the RUDAK-2 and other

subsystems, while Linear transponder #2 mode B with subsystems The

Primary transponder is Linear Transponder #1, the second one is a

spare which can be put into operation in the event of a failure of

the primary system. Mode B in low earth orbit provides a very good

communications link as anyone who worked AO-7 Mode B will testify.

The Transponder RF Frequency Assignments and Beacon Data for the

Primary Payloads are shown in Table 8.4.1, and those for the Backup

payload in Table 8.4-2. The 1100 bps is not a misprint, it is real.

Apparently this data rate is used by a popular PC tape cassette

interface in the Soviet Union. Because this PC is simple and cheap

for the Hams in the USSR, the AMSAT-U-Orbita team decided to use it

on this spacecraft, much in a similar manner to the use of reversed

AFSK tones in UO-2 due to the wide availability of a popular

interface for those tones in the United Kingdom. It is unclear as of

the time of writing this, if the telemetry is HDLC or some

proprietary synchronous PSK such as the 400 baud AO-13 downlink. As

such you may not be able to copy this telemetry.

Table 8.4-1 Primary Payload

Beacons and telemetry #1

CW telemetry 8 channels  145.822 MHz 0.2 Watts

Digital telemetry        30 channels 145.952 MHz 0.4 Watts

1100 bps,BPSK/FM, deviation 2kHz

Digital telemetry

Rudak-2                  145.983 MHz 3.0 Watts

BPSK 1200 bps AX.25 (like F-O 20)

Transponders #1

Linear transponder:      inversely heterodyned translator

Uplink passband          435.102 to 435.022 MHz

Downlink passband        145.852 to 145.932 MHz

Transmitter output max. 10 Watts

Bandwith (3db)           80 kHz

Uplink EIRP required about 100 Watts

Digital transponder Rudak-2: digipeater and store&forward packet

communication (AX.25), telecommunications experiment with digital

signal processing up to nearly 20 kHz, 1 MByte RAM disc, four

separate uplink channels.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 118

Uplink frequencies:

RX-1  435.016 MHz 1200bps,FSK,NRZIC/Biphase-M

RX-2  435.155 MHz (AFC) 2400 bps,BPSK, Biphase-S

RX-3a 435.193 MHz (AFC) 4800 bps,RSM

RX-3b 435.193 MHz (AFC) 9600 bps,RSM

RX-4  435.041 MHz (digital AFC) RX for RTX-DSP

Downlink frequency: 145.983 MHz 3 Watts

The downlink can be switched to the following operating modes:

Mode 1: 1200 bps, BPSK, NRZI,(NRZ-S) (like FO-20)

Mode 2: 400 bps, BPSK, Biphase-S (Like AO-13 beacon)

Mode 3: 2400 bps, BPSK, Biphase-S

Mode 4: 4800 bps, RSM, NRZIC (Biphase-M) (like 4800 bps uplink)

Mode 5: 9600  bps, RSM, NRZI (NRZ-S) +Scrambler (like 9600 bps

uplink)

Mode 6: CW keying (only for special events)

Mode 7: FSK (F1 or F2B),e.g. RTTY, SSTV, FAX, etc.(for special

events)

Mode 8: FM modulated by D/A signals from DSP-RISC processor (speech)

Table 8.4-2 Secondary Payload

Beacons and Telemetry #2

CW telemetry 8 channels          145.948 MHz 0.2 Watts

Digital telemetry 30 channels    145.838 MHz 0.4 Watts 1100 bps,

BPSK/FM, deviation 2kHz

Digital telemetry  30 channels   145.800 MHz 2.0 Watts 1100 bps

BPSK/FM, deviation 2kHz

Transponder #2

Linear transponder:      inversely heterodyned translator

Uplink passband          435.123 to 435.043 MHz

Downlink frequencies     145.866 to 145.946 MHz

Transmitter output max.  10 Watt max.

Bandwith (3db)           80 kHz

Uplink EIRP required about 100 Watts

The spacecraft uses two antennas. The 435 MHz receiving antenna

which is shared by the analog and digital modes is a Helix with up

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 119

to +3 db gain using Right Hand circular Polarization. The 145 MHz

transmitting antenna is a Half wave dipole. AO-21 draws up to 100

Watts from the main spacecraft's power supply system. The Primary

RM-1 payload including the RUDAK draw up to 47 Watts, the secondary

payload draws up to 40 Watts. The Primary AO-21 payload including

the RUDAK weighs approximately 28 kg, the secondary payload weighs

approximately 22 kg. Both payloads are about 480x400x300 mm^3.

The CW telemetry started up as soon as the combined spacecraft

separated from the launch vehicle and was powered by main satellite

power system. A CW Morse-Code telemetry frame consists of the call

RS14 and 8 channels of four digits in the following format:

RS14 S0AB=S1AB=S2AB=S3AB=S4AB=S5AB=S6AB=S7AB

Channels 0 to 6 contain analog telemetry data. Channel 7 contains

engineering calibration parameters.

The first digit (S) identifies which system the telemetry is from.

A prefix of 7 identifies a general status, a prefix of 5 identifies

a command status. The second digit (0 to 7) are the numbers of the

line (channel). The remaining digits (A and B) are the analog

telemetry data which can be decoded according to the equations shown

in Table 8.4-3.

Table 8.4-3 AO-21 CW Telemetry decoding parameters

Channel        Parameter                Formula   Unit

0        Transponder power output      0.05*N    Watts

1        Transponder PA Temperature    N         Deg. C

2         +24 V Regulated              N         Volt

3         +16 V Regulated              N         Volt

4         +9  V Regulated              N         Volt

5         +24 V Regulated              N         Volt

6        Inside Temperature            N         Deg. C

7        Engineering Value             N         *

A typical frame such as

"RS14=7080=7137=7224=7316=7409=5524=5032=57PPRS14"

may be decoded as shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 120

RS14

7080 - 4 Watts   - general

7137 - 37 Deg. C - general

7224 - 24 Volts  - general

7316 - 16 Volts  - general

7409 - 9 Volts   - general

5524 - 24 Volts  - command

5032 - 32 Deg. C - command

57PP             - command (transponder #1)

Examples of other frames copied and edited by G3ZCZ/W3 are

28-Feb-91 01:40 RS14=7032=7121=7224=7316=7410=7500=7600=77PP

01-Mar-91 02:19 RS14=7028=7121=7224=7316=7410=7500=7600=77PP

The "PP" at the end of the frame identifies the telemetry as having

come from transponder #1. Transponder #2 identifies its telemetry

with the "PPPP" sequence.

The AO-21 Digital PSK telemetry consists of 30 parameters

monitoring on-board conditions and 2 calibration verification

points. If you intend to receive, decode and display the digital

telemetry you will need an FM receiver, a TNC, a PSK modem, and a

computer or terminal. Even then you may not be able to decode the

data since it may not be downlinked in HDLC format. The decoding

equations for the digital telemetry are shown in Table 8.4-4. The

raw digital telemetry is expected to show up on your screen in the

format shown in Table 8.4-5.

Table 8.4-4 Decoding Equations for AO-21 Digital Telemetry

(Version 26-Dec-90)

Line Parameter                  Formula     Unit  Hex-Format-Line

1   "Zero" of the comparator      0C         N/A      0000

2   Reference voltage             6D         N/A      0010

3   Transponder #1 HF output pwr  0.2N^2    Watt      0020

4   Transponder #1 PA temperature 0.8*N     Deg. C    0030

5   DC/DC converter temperature   0.8*N     Deg. C    0040

6   +14 V Regulated               10*N      Volt      0050

7   +24 V Regulated               10*N      Volt      0060

8   +16 V Regulated               10*N      Volt      0070

9   +12 V Regulated               10*N      Volt      0080

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 121

10  +9 V Regulated                10*N      Volt      0090

11  +7.5 V Regulated              10*N      Volt      00A0

12  +5 V Regulated                10*N      Volt      00B0

13  +9 V Regulated (linear)       10*N      Volt      00C0

14  +9 V Regulated (digital)      10*N      Volt      00D0

15  Service                          N      *         00E0

16  Service                          N      *         00F0

17  Transponder #2 HF output pwr  0.2*N^2   Watt      0100

18  Transponder #2 PA temperature 0.8*N     Deg. C    0110

19  +24 V Regulated               10*N      Volt      0120

20  +16 V Regulated               10*N      Volt      0130

21  +10 V Regulated               10*N      Volt      0140

22  +9 V Regulated                10*N      Volt      0150

23  +7.5 V Regulated              10*N      Volt      0160

24  Status command link           1         *         0170

25  Status command link           2         *         0180

26  Status command link                     *         0190

27  Status command link                     *         01A0

28  RPC +5V for Rudak-1           2.47*N    Volt      01B0

29  RPC +5V for Rudak-RTX         2.47*N    Volt      01C0

30  RPC +5V for Ramdisk           2.47*N    Volt      01D0

31  RPC +14V total supply current 627-289*N mA        01E0

32  RPC module temperature        56.7*N-49.5 Deg. C  01F0

Note: RPC - Rudak Power Conditioner. The Service and command link

channels are reserved for use by the ground control team.

Table 8.4-5 Expected Format of Raw AO-21 PSK Telemetry

0000 0C E6 0C E6 0C E6 0C E6 0C E6 0C E6 0C E6 0C E6

0010 6D E6 6D E6 6D E6 6D E6 6D E6 6D E6 6D E6 6D E6

0020 i E6 i E6 i E6 i E6 i E6 i E6 i E6 i E6

....................................................

01F0 j E6 j E6 j E6 j E6 j E6 j E6 j E6 j E6

i...j - the value of the parameter in the hex format, repeated 8

times E6 - separation

The RUDAK system is a message store-and-forward package. Its

downlink is not expected to contain any telemetry. An example of

some RUDAK signals copied by W3/G3ZCZ a few days after launch is

shown in Table 8.4-6.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 122

Table 8.4-6 Example of AO-21 RUDAK Signals.

26-Feb-91  02:31:36  RUDAK*>BEACON:

* RUDAK-II / AMSAT OSCAR 21/RM1

* Up: 435.016MHz (1200)

*     435.155MHz (2400)

* Mailbox=RUDAK Mheard=RUDAK-1

26-Feb-91  02:32:37  RUDAK*>BEACON:

RUDAK-Telemetry (91-02-25 23:03:00):

Used stack entries: 0

Uplink Carrier Detect (during last minute): 0%

26-Feb-91  02:33:36  RUDAK*>BEACON:

* RUDAK-II / AMSAT OSCAR 21/RM1

* Up: 435.016MHz (1200)

*     435.155MHz (2400)

* Mailbox=RUDAK Mheard=RUDAK-1

26-Feb-91  02:34:37  RUDAK*>BEACON:

RUDAK-Telemetry (91-02-25 23:05:00):

Used stack entries: 0

Uplink Carrier Detect (during last minute): 0%

26-Feb-91  02:35:09  RUDAK,*>WB5BZE (UA)

26-Feb-91  02:35:11  RUDAK*>WB5BZE:

+-------------------------------------------------------+

+  Welcome to the RUDAK II Bulletin Board System V0.04  +

+-----------

26-Feb-91  02:35:13  RUDAK*>WB5BZE:

--------------------------------------------+

Logged in at 91-02-25 23:05:33,  1 User

This is a preliminary release.

Please rep

26-Feb-91  02:35:15  RUDAK*>WB5BZE:

ort deficiencies to DL2MDL.

73 de AMSAT-UA/AMSAT-DL/RUDAK-Group.

Enter H for help.

WB5BZE de RUDAK>

26-Feb-91  02:35:36  RUDAK*>BEACON:

* RUDAK-II / AMSAT OSCAR 21/RM1

* Up: 435.016MHz (1200)

*     435.155MHz (2400)

* Mailbox=RUDAK Mheard=RUDAK-1

26-Feb-91  02:35:51  RUDAK*>WB5BZE [D]

26-Feb-91  02:35:56  RUDAK*>WB5BZE [D]

26-Feb-91  02:36:32  KF4WQ>RUDAK*>KF4WQ [C]

26-Feb-91  02:36:37  RUDAK*>BEACON:

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 123

RUDAK-Telemetry (91-02-25 23:07:00):

Used stack entries: 0

Uplink Carrier Detect (during last minute): 0%

26-Feb-91  02:37:35  RUDAK*>WB5BZE (UA)

The AO-21 Packet Telemetry is formatted for display in the

spacecraft and downlinked as ASCII packets. Typical packets are

shown below.

18-Jun-94  20:38:18  RUDAK2*>BEACON [UI]:

++ Hi, this is the RUDAK-II experiment on AMSAT OSCAR 21 ++

18-Jun-94  20:38:21  RUDAK2*>TLM-2 [UI]:

5V-RAM: 4.97 V    3-RNG    : off      AGC

total current    4-soft:  0.2 V    RX3: 142   temperature

14V-I :  307 mA                     RX4: 207    21.6 deg C

chksum: 0   upcnt: 0   stack: 2   noint: 0

18-Jun-94  20:38:23  RUDAK2*>TLM-3 [UI]:

RAMDISK:

wash sectors memory errors audio sectors picture sectors

start: 3100  single: 215    start: 165     start: 3100

end  : 4095  multi : 0      end  : 2412    end  : 3452

ptr  : 3785           sample freq: 8000   chksum: -19652 (-19652)

18-Jun-94  20:38:24  RUDAK2*>BEACON [UI]:

RUDAK-II Schedule: (down 145.987, up 435.016)

min/10  Beacon Mode

0..8   FM Repeater

9    AFSK TLM

The same information seen without the headers is shown below.

5V-RAM: 4.97 V    3-RNG    : off      AGC

total current    4-soft:  0.2 V    RX3: 142   temperature

14V-I :  307 mA                     RX4: 207    21.6 deg C

chksum: 0   upcnt: 0   stack: 2   noint: 0

RAMDISK:

wash sectors memory errors audio sectors picture sectors

start: 3100  single: 215    start: 165     start: 3100

end  : 4095  multi : 0      end  : 2412    end  : 3452

ptr  : 3785           sample freq: 8000   chksum: -19652 (-19652)

RUDAK-II Schedule: (down 145.987, up 435.016)

min/10  Beacon Mode

0..8   FM Repeater

9    AFSK TLM

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 124

9.0 File formats

This section contains details of the contents of the different

files used by WHATS-UP.

9.1 Configuration File (WHATS-UP.SYS)

The contents of the Configuration file (WHATS-UP.SYS) are as

follows:

1 Your callsign (e.g. G3ZCZ)

2 Default spacecraft configuration file (e.g. Dove)

3 station latitude (e.g. 35.00)

4 station longitude (e.g. 74.00)

5 station altitude (e.g. 100)

6 station minimum antenna elevation for acquisition (e.g. 0)

7 default Kepler file (e.g. whats-up.TLE)

8 UTC offset (e.g. EST = 5)

9 default directory path (e.g. C:)

10 default extracted data file (e.g. whats-up.txt)

11 default file name with list of telemetry parameters to extract

file (e.g. ARRAYS)

12 TNC Type (e.g. PK-232)

13 PC serial port to TNC (e.g. 1)

14 PC serial TNC port baud rate (e.g. 1200)

15 PC Serial TNC port data bits (e.g. 8)

16 PC Serial TNC port Stop bits (e.g. 1)

17 PC Serial TNC port parity (e.g. N)

18 status (top) window color (e.g. 79)

19 Incoming window color (e.g. 14)

20 outgoing window color (e.g. 30)

21 prompt window color (e.g. 15)

22 alarm window color (e.g. 15)

23 bottom window color (e.g. 79)

24 Emphasis color (e.g. 14)

25 option color (e.g. 78)

26 parameter changed color (e.g. 95)

27 parameter limit exceeded color (e.g. 14)

28 Orbit element window color

29 Orbit element window Orbit element window In range color

30 Orbit element window early warning color

31 Orbit element window next one up color

32 Logbook window color

33 SAREX call color in status window

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 125

34 Active color

35 Post pass color in orbit element window

36 Orbit alert dit time

37 Orbit alert note

38 Flag Sound

39 Doppler display Flag

40 DSP-1200 Modem CW command

41 DSP-1200 Modem 1200 ASCII command

42 DSP-1200 Modem 1200 PSK command

43 DSP-1200 Modem 400 PSK command

44 DSP-1200 Modem 9600 FM command

45 DSP-1200 Modem 1200 FM command

46 Logbook path and name (.DBF may be left out)

47 QSO Logging Flag

48 Minimum angle of the sun for darkness at your QTH

49 TNC stream switch character, default '|'

50 The next few lines link the spacecraft configuration files.

51 The * that follows denotes the last line of SC ID data.

52 CW Memory contents: - The next ten lines contain the cw memory

data.

54 The * marker line

55 The remaining lines contain TNC configuration parameters.

You must configure WHATS-UP before you try any Real Time activity.

The contents of the configuration file are described below.

9.1.1 Your callsign

This item is the callsign displayed at the top of the screen and

appended to the capture files when capture-to-disk is turned on.

9.1.2 Default spacecraft Name

This item is the default spacecraft name (e.g. Dove). WHATS-UP adds

the ".SYS" to the end of the name (e.g. DOVE.CNF) and loads that

configuration file at start up.

9.1.3 Station Latitude

This item is your station latitude (e.g. 35.00). In the southern

hemisphere, enter a negative number.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 126

9.1.4 Station longitude

This item is your station longitude in degrees WEST of Greenwich.

(e.g. 74.00)

9.1.5 Station Altitude

This item is the altitude of your station antenna above sea level

in Meters. (e.g. 100)

9.1.6 Station minimum antenna elevation for acquisition

This item is the minimum angle of elevation of your antennas at

which you can acquire a signal. If you are on top of a hill or tall

building the angle my be less than 0, if you are in a valley, it may

be greater, such as 5 degrees.

9.1.7 Default Kepler file

This item is the default file name that contains the orbit element

data used when the program is first turned on (e.g. whats-up.TLE).

9.1.8 UTC offset

This item is the time difference (in hours) between the local time

in your PC and GMT or Universal Coordinated Time (UTC). WHATS-UP

always displays UTC time. (e.g. EST = 5)

9.1.9 Default directory path (e.g. C:)

This item is the directory path  (e.g. C:\TLM) for the spacecraft

capture-to-disk (YYMMDD.S/C), spacecraft configuration (*.CNF) and

spacecraft operations schedule files (*.OPS).

9.1.10 Default extracted data file

This item is the name of the file to which data which is extracted

from a playback file will be written to in a comma delimited format

so that the data can be imported into a spreadsheet package for

further analysis (e.g. whats-up.txt).

9.1.11 Default file name with list of telemetry parameters to

extract file

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 127

This item is the name of the file containing the channel numbers to

extract from a playback file for further analysis (e.g. ARRAYS)

9.1.12 TNC Type

This item is the type of TNC or Multi-mode controller you are

using.

9.1.13 Serial port to TNC

This item is the Communications port number between the TNC and the

PC (e.g. 1). If you set it to 0, WHATS-UP will never try to access

the TNC.

9.1.14 PC TNC Serial baud rate

This item is the baud rate used between the TNC and the PC (e.g.

1200).

9.1.15 PC TNC port data bits

This item is the number of data bits used between the TNC and the

PC (e.g. 8).

9.1.16 PC TNC port Stop bits

This item is the number of stop bits used between the TNC and the

PC (e.g. 1).

9.1.17 PC TNC Port parity bits

This item is the parity setting used between the TNC and the PC.

The letters to use are defined as

N No parity, O Odd, E Even, M Mark, S Space.

The following items are the color values used for different windows

or messages.

9.1.18 Status (top) window color (e.g. 79)

9.1.19 Incoming window color (e.g. 14)

9.1.20 Outgoing window color (e.g. 30)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 128

9.1.21 Prompt window color (e.g. 15)

9.1.22 Alarm window color (e.g. 15)

9.1.23 Bottom window color (e.g. 79)

9.1.24 Emphasis color (e.g. 14)

9.1.25 Option color (e.g. 78)

9.1.26 Parameter changed color (e.g. 95)

9.1.27 Parameter limit exceeded color (e.g. 14)

9.1.28 Orbit element window color

9.1.29 Orbit element window In range color

9.1.30 Orbit element window early warning color

9.1.31 Orbit element window next one up color

9.1.32 Logbook window color

9.1.33 SAREX call display color in Status window

9.1.34 Active color

9.1.35 Orbit element Post pass color

9.1.36 Orbit alert dit time

This item is the speed of the morse code annunciator used to alert

you of satellite AOS, EWT and LOS.

9.1.37 Orbit alert note

This item is the tone of the morse code annunciator used to alert

you of satellite AOS, EWT and LOS.

9.1.38 Flag Sound

This item controls the sounds coming from the PC. A

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 129

1 is on,

0 is off.

9.1.39 Doppler display Flag

This item determines the type of Doppler display as follows:-

0 display beacon frequency,

1 display Doppler shift.

9.1.40 TNC Command to select modem for CW

9.1.41 TNC Command to select modem for UoSAT ASCII 1200 baud

9.1.42 TNC Command to select modem for 1200 baud PSK

9.1.43 TNC Command to select modem for 400 baud AO-13 PSK

9.1.44 TNC Command to select modem for 9600 baud packet

9.1.45 TNC Command to select modem for 1200 baud FM AFSK

9.1.46 Logbook file

This item is the default file name for the logbook (e.g. vhf.dbf).

9.1.47 QSO Logging flag

This item is the flag that determines if acquisition of signal is

logged. A:

1 means log,

0 means don't log.

9.1.48 Minimum angle of the sun for darkness at your QTH

This item sets the value for viewing of satellites. To see a

spacecraft, the spacecraft must be in sunlight and the ground viewer

must be in darkness. This parameter lets you adjust the apparent sun

angle for darkness at your location. Note while some spacecraft may

be in sunlight, and you may be in darkness, the spacecraft may not

be bright enough to be visible with the naked eye. MIR and the space

shuttle are bright enough.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 130

9.1.49 TNC stream switch character, default '|'

9.1.50 Spacecraft Configuration File Linkages

The next few lines contain the information which WHATS-UP uses to

link the spacecraft configuration files to the Keplerian elements

for orbit determination and automatic selection of spacecraft at AOS

time. There should be NO SPACE characters before or between the

commas, in the first two data elements. The lines contain data as

shown below.

UO-11,UOSAT11.CNF,     0

AO-13,OSCAR13Z.CNF,    0

AO-16,PACSAT.CNF,      0

DO-17,DOVE.CNF,        1

WO-18,WEBER.CNF,       0

LO-19,LUSAT.CNF,       2

FO-20,FUJI.CNF,        0

The first item is the Keplerian element identifier for the

spacecraft as used in the orbit element files.

The second item is the FULL configuration file name for the

spacecraft.

The third item is the automatic AOS selection flag as itemized

below.

'0', automatic selection is inhibited.

'1', WHATS-UP selects that spacecraft configuration file at EWT

time (if it is not selected at that time), and tunes the radio

to the beacon frequency + offset defined in that file.

'2', WHATS-UP selects that spacecraft configuration file at AOS

time (if it is not selected at that time), and tunes the radio

to the beacon frequency + offset defined in that file.

9.1.51 Marker and Comment line

This line must be present and begin with an '*' character. It

signals WHATS-UP that the spacecraft default parameters have been

read.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 131

9.1.52 CW Memory contents

The next ten lines contain the cw memory data

9.1.53 The marker line

This line must be present and begin with an '*' character.

9.1.54 TNC configuration commands

The remaining lines are commands sent to the PK232 when you

configure the TNC. Note to avoid lock ups FLOW and XFLOW MUST be

OFF. Typical commands are as follows:-

HEAD ON

ECHO OFF

DAYSTAMP ON

MONITOR 6

MSTAMP ON

FLOW OFF

XFLOW OFF

MFILTER None

9.2 Spacecraft Parameter Files

You need a Different spacecraft parameter file for each spacecraft.

Spacecraft parameter files are named by the spacecraft and given the

extension '.CNF'. Examples are 'DOVE.CNF' and 'Fuji20.CNF'. These

files determine how the individual channels are decoded, and where,

in which screen page, and in which color the decoded data are

displayed. Some of the items are unique to WHATS-UP and some to the

particular spacecraft.

The entries in the file are:

1 Spacecraft ID

2 Spacecraft Suffix

3 Beacon Frequency

4 Doppler Measurement File

5 Spacecraft Identification in Keplerian Element File

6 Doppler Measurement Sample Interval

7 Initial Frequency Offset

8 Autotrack flag

9 Modulation

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 132

10 Data Type

11 Receiver Type

12 Receiver Address

13 PC Serial port to Radio

14 PC Radio port Serial baud rate

15 PC Radio port data bits

16 PC TNC port Stop bits

17 PC Radio Port parity bits

18 Post pass delay

19 Station minimum usable pass time

20 Early warning time

21 SAREX/Mir Callsign

22 SAREX/Mir Header Delay

23 SAREX/Mir Attack Mode

24 SAREX/Mir Beacon Text

25 LPT parallel port radio memory parameters

26 Selected or default display page number

27 MET Start Time

28 Page Definitions

29 Telemetry Parameter Configuration

30 Digital Telemetry Status Channels

31 Packet/Link Parameters

The contents of each line in the SPACECRAFT.CNF file are as

described below.

9.2.1 Spacecraft ID

This is the callsign of the spacecraft. For example,

Spacecraft    ID.

---------------------

DO-17         DOVE-1

FO-20         8J1JBS

9.2.2 Spacecraft Suffix

This becomes the filetype for the capture-to-disk files. The

default suggestions are as shown below.

Spacecraft          Suffix

UoSAT-2             U11

Fuji-OSCAR 12       F12

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 133

AMSAT-OSCAR 13      O13

AMSAT-OSCAR 16      O16

DOVE-OSCAR 17       D17

WEBER-OSCAR 18      W18

LUSAT-OSCAR 19      L19

Fuji-OSCAR 20       F20

AMSAT-OSCAR 21      O21

9.2.3 Beacon Frequency

This is the frequency of the spacecraft beacon (in MHz) that you

are monitoring. It will be displayed in the status window.

9.2.4 Doppler Measurement File

This is the default name of the file used to store the Doppler

frequency measurements.

9.2.5 Spacecraft Identification in Keplerian Element File

Examples are as follows.

UO-11   AO-13     AO-16     DO-17     WO-18     LO-19

9.2.6 Doppler Measurement Sample Interval

This is the default sample interval (in seconds) used between

Doppler frequency measurements.

9.2.7 Initial Frequency Offset

This is the default value (in kHz) added to the beacon frequency,

and output to the Radio Receiver when a particular spacecraft is

selected. The default value is 5 to tune the radio 5 kHz above the

beacon. This will allow the signal to slide into lock, whereby the

TAPR PSK Modem will lock on and follow the frequency for the rest of

the pass.

9.2.8 Autotrack flag

This item tells WHATS-UP if it should allow another AOS to

interrupt data collection from this spacecraft.

0 Don't let another AOS to interrupt collection.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 134

1 Allow another spacecraft AOS to interrupt collection, namely,

tune to that spacecraft at its AOS.

9.2.9 Modulation

This is the type of modulation to set the Radio to to copy data

from the spacecraft. Current options are :-

C CW

F FM L LSB

P PSK

U USB

D FM Duplex. Use this mode for SAREX. If duplex is selected, then

the uplink frequency (in MHz) must be added following the D and

separated by a comma character (i.e. D,144.49).

9.2.10 Data Type

This is the type of data downlinked by the spacecraft.

A ASCII as used by UoSAT-2.

B BAUDOT as used by AO-13.

C CW as used by AO-21.

P Packet as used by DO-17, Fuji-OSCAR 20 and the Microsats.

9.2.11 Receiver Type

This is the manufacturer of the radio receiver you are using. This

version of WHATS-UP supports the:

Kenwood   series.

Lpt       line printer parallel port.

Icom      (not tested).

9.2.12 Receiver Address

The assignment is as follows:

9.2.12.1 Kenwood

Not used.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 135

9.2.12.2 LPT parallel port

This item defines the printer port as shown below.

Assignment     Printer Port    PC Assignment

Bit       Address (Hex)

1             $0378          LPT1

2             $0278          LPT2

3             $03BC          Custom for my laptop

For example, to use LPT1 as the printer port, set this value to 1.

You may use different ports for different radios on different bands

9.2.12.3 Icom

Set the line to the decimal value of the radio address according to

the Icom radio manual.

9.2.13 PC Serial port to Radio

This item is the Communications port number between the PC and the

Radio (e.g. 2). If you set it to 0, WHATS-UP will never try to

access the radio via the serial port. If you select the LPT radio

option, set this value to 0.

9.2.14 PC Radio port Serial baud rate

This item is the baud rate used between the PC and the Radio (e.g.

4800).

9.2.15 PC Radio port data bits

This item is the number of data bits used between the Radio and the

PC (e.g. 8).

9.2.16 PC TNC port Stop bits

This item is the number of stop bits used between the Radio and the

PC (e.g. 2).

9.2.17 PC Radio Port parity bits

This item is the parity setting used between the Radio and the PC.

The letters to use are defined as

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 136

N No parity, O Odd, E Even, M Mark, S Space.

9.2.18 Post pass delay

This item is the time (in minutes) that WHATS-UP after the computed

LOS time before returning the radio to the default frequency (e.g.

2). This delay is to be used in the event that the elements do not

quite predict the exact times for your system.

9.2.19 Station minimum usable pass time

This item is the minimum time in minutes for a usable pass for

collecting data (e.g. 5).

9.2.20 Early warning time (EWT)

This item is the early warning time in minutes you want for notice

that a spacecraft is about to come up above your local horizon (e.g.

5).

9.2.21 SAREX/Mir Callsign

This item is the callsign to which the automatic beacon/connect

attempt will be made. if you wish to beacon, then you must set the

'UNProto' parameter in the TNC to the digipeater callsign. for

example to beacon via MIR, when MIR is using R0MIR, then set the

SAREX call to R0MIR, and set the TNC parameter as 'UNP CQ VIA

R0MIR'.

9.2.22 SAREX/Mir Header Delay

This item is the number of packet headers to count before issuing

the next beacon packet. Try to resist the temptation to set it at a

low value.

9.2.23 SAREX/Mir Attack Mode

This item is the SAREX/MIR Attack mode. Options are to try:

B - both connect and digipeat

C - connect to SAREX call

Q - call CQ via the SAREX call

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 137

9.2.24 SAREX/Mir Beacon Text

This item is the text to beacon via the SAREX/MIR. Put your grid

locator at the beginning of the line for APRS compatibility, i.e.

[FM19LA]. If you put an * as the first character, WHATS-UP will

ignore the value in the file.

9.2.25 LPT parallel port radio memory parameters

This line contains two items separated by a comma, e.g., 7,23.

The first item is the memory channel assigned to the spacecraft.

The second item is the number or total number of memory channels.

When the file is loaded at EWT or AOS, WHATS-UP will step through

the radio memories to the selected channel.

The maximum parameter is required so that when WHATS-UP is scanning

through the memories, it resets the count to 0 at the correct

channel.

9.2.26 Selected or default display page number

This is the default display page for the Real-time and Playback

modes, when WHATS-UP is first loaded.

9.2.27 Page Definitions

These are the page definitions, with two items on the line. The

format is PAGE_TITLE, Page_Color, as in the example below.

SPACECRAFT HOUSEKEEPING, 30

9.2.28 Telemetry Parameter Configuration

The next set of items are the Telemetry parameter configurations

(maximum = 99). You must have at least one of these lines in the

file. If you want a value to show up in more than one page (other

than the wild card [0]) you must enter it twice (once per page).

Typically each row contains 17 items in the format shown below.

TLM_Channel,   TLM_Segment_ID,     TLM_Description,    TLM_Eqn_Type,

TLM_Ceof_C,    TLM_Ceof_B,         TLM_Ceof_A,         TLM_Units,

TLM_Page,      TLM_Row,            TLM_Col,            TLM_Width,

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 138

TLM_Dec,       TLM_Limit_Check,    TLM_Limit_Low,

TLM_Limit_High,

TLM_Negative_Blank.

Each item is described in the following sections.

9.2.28.1 TLM_Channel

This is the channel number of the telemetry data in the frame. The

DOVE channel number is hexadecimal (e.g. '0F'), Fuji is decimal.

Each entry must be two digits.

A special identifying TLM_Channel is defined in WHATS-UP. If the

value is '99 then the segment identifier and position of the segment

identifier is defined in tow positions in the line. This special

channel identifies the type of telemetry frame.

9.2.28.2 TLM_Segment_ID

This is the segment identifier as described in Section 9.2.28.99

below.

9.2.28.3 TLM_Description

This item is the text string or description of the telemetry

channel that will be displayed on the screen page. (e.g. '+Z Array

Temp.')

9.2.28.4 TLM_Eqn_Type

This item tells WHATS-UP the type of equation to use to decode the

telemetry.

Type 1 is the general purpose equation used by AMSAT-NA in the

Microsats.

Fuji (FO-12 and FO-20) uses two other types of equations (2 and 3).

They are in the formats of

Y = D*(N+E), and Y = F*(G-N).

If you know some algebra you can convert both of Fuji's equations to

the Format used by AMSAT, but since a computer is involved, why not

let it do the job. You do however have to convert an equation of the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 139

form Y=(N+a)/b.

AO-13 also needs three more types (4 5 and 6) of equations to

decode the RTTY Z blocks.

A type 1 equation in WHATS-UP is a quadratic of the form of

Y = A*N^2 + B*N + C, where: N = raw telemetry data value,

A, B, C = Equation

Coefficients; Y, N are decimal

values.

A type 2 equation in WHATS-UP has the format in the form of

Y = B*(A+N) + C  where C, B, A are coefficients; Y, N are

decimal values.

A type 3 equation in WHATS-UP has the format in the form of

Y = B*(A-N) + C  where C, B, A are coefficients; Y, N are

decimal values.

A type 4 equation in WHATS-UP has the format in the form of

Y = B*(N+A)^2 +C  where C,B, A are coefficients; Y, N are

decimal values.

A type 5 equation in WHATS-UP has the format in the form of

Y = B*(A-N)^2 + C  where C, B, A are coefficients; Y, N

are decimal values.

A type 6 equation in WHATS-UP is a special case. It is the AO-13

(Channel 1C) Spin Rate

equation as specified below.

if N > 131 then Y := 479/(N - 109) - 2

else Y := (131 - N) * 0.85 + 20;

where N is the raw decimal number in Channel 1C and Y is the spin

rate.

9.2.28.5 TLM_Ceof_C

This item is the equation Coefficient C.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 140

9.2.28.6 TLM_Ceof_B

This item is the equation Coefficient B.

9.2.28.7 TLM_Ceof_A

This item is the equation Coefficient A.

9.2.28.8 TLM_Units

This item is the Units text string (e.g. '.C') in the screen

display. However if the Tlm_Channel is '99' then this item is the

segment identifier string.

9.2.28.9 TLM_Page

This item is the Display page number. A 0 is a 'wild card' which

will be displayed on every page.

9.2.28.10 TLM_Row

This item is the Display page row. It identifies which row in the

screen the data element will be displayed.

9.2.28.11 TLM_Col

This item is the Display page column. It identifies which column in

the screen the data item will be displayed.

9.2.28.12 TLM_Width

This item is the Display width for Engineering Units. It tells

WHATS-UP how many characters wide the display is to be. You can set

it to any value you want. For example, you can display a voltage as

'1.3' or '1.28567'. Before you widen the display too much, remember

the sampling accuracy of the analog-to-digital converter in the

spacecraft.

9.2.28.13 TLM_Dec

This item defines the number of digits after the decimal point in

the display.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 141

9.2.28.14 TLM_Limit_Check

This item tells WHATS-UP to perform limit checking on the telemetry

channel. It may have several values as

described below.

0 = do nothing,

1 = check below low limit,

2 = check above high limit,

3 = check for EITHER [below low limit] or [above high limit]).

9.2.28.15 TLM_Limit_Low

This item is the Low limit value (e.g. -4.00).

9.2.28.16 TLM_Limit_High

This item is the High limit value (e.g. +10.6).

9.2.28.17 TLM_Negative_Blank

This item is a 1 if you want computed negative values to be

displayed as a zero. Use this for example, in Solar Cell Voltage

computations, when negative values are produced by the equation

supplied even though the negative values are not real. The negative

values are produced because the equation used to convert the data is

not valid at low or zero values of light.

A line with an '*' as the first character terminates this section.

9.2.28.99 The Telemetry Identifier Line

The Telemetry identifier line has the same format as a regular

line, but contains different parameters as follows:

9.2.28.99.1 The Line Identifier

This item has to be '99' to identify the telemetry identifier line.

9.2.28.99.2 The Segment Identifier

This is used when the spacecraft transmits telemetry in more than

one segment. It contains the segment identifier showing which

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 142

segment the channel is down linked in.

9.2.28.99.3 Spare/Not Used

9.2.28.99.4 Location of Segment Identifier in Line

This item contains the location of the segment identifier in the

first line of the telemetry data.

9.2.28.99.5 Spare/Not Used

9.2.28.99.6 Spare/Not Used

9.2.28.99.7 Spare/Not Used

9.2.28.99.8 Segment Identifier String

This item is the segment identifier string as follows.

9.2.28.99.82.1 Fuji

The Fuji frame contains one real time segment (Segment 1) in a

frame addressed as 8J1JBS>BEACON. A typical frame is shown below.

19-Apr-90 17:14:34 8J1JBS*>BEACON:

JAS1b RA 90/04/19 17:13:58

609 430 687 676 744 837 845 829 498 681

617 001 505 516 526 524 526 523 654 000

683 675 686 695 999 643 875 471 099 000

110 111 000 000 111 100 001 111 111 000

The segment identifier is in the seventh and eighth characters of

the first line of the data. A segment identifier of that position

identifies the second segment. The segment identifier is the 'RA'

located on the first line of the data just after the JAS1b where the

'R' in 'RA' is the seventh character in the line.

Any telemetry frame addressed to BEACON received without that

segment identifier is assumed by WHATS-UP to be Segment 2.

9.2.28.99.8.2 DOVE

DOVE transmits telemetry in two frames each addressed as DOVE-

1>TLM. The Microsat ASCII frame thus contains two segments. Two

typical segments of DOVE telemetry are shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 143

DOVE-1>TLM [01/29/90 22:08:46]:

00:59 01:59 02:86 03:30 04:58 05:58 06:6D 07:45 08:6C 09:66 0A:A1

0B:D9 0C:E8 0D:D8 0E:01 0F:23 10:CC 11:A8 12:00 13:01 14:A8 15:94

16:96 17:94 18:95 19:96 1A:93 1B:90 1C:9A 1D:98 1E:23 1F:5E 20:BC

DOVE-1>TLM [01/29/90 22:08:47]:

21:98 22:7B 23:24 24:21 25:2E 26:00 27:00 28:00 29:00 2A:00 2B:00

2C:00 2D:29 2E:00 2F:9B 30:C8 31:9C 32:11 33:DA 34:C0 35:95 36:A4

37:A4 38:B2 39:96 3A:00

The default segment identifier used by WHATS-UP is in the first

and second characters of the first line of the data. A segment

identifier of '00' identifies the first segment, and anything else

in that position identifies the second segment.

9.2.28.99.8.3 AO-13

AO-13 RTTY Telemetry is transmitted in the form of Z blocks shown

below (several blank lines have been deleted to same space in this

document).

Z  HI. THIS IS AMSAT OSCAR 13

05.02.44  4661

.0086   .0000   .07B9

64  6   0   1   16  218 1

193 170 158 143 181 144 147 140 200 7

147 7   7   7   165 29  100 7   149 7

10  7   145 115 34  7   153 129 122 180

152 73  7   145 137 55  7   183 136 151

7   154 137 169 211 142 127 100 9   140

161 7   173 149 150 154 14  131 127 210

HI THIS IS AMSAT OSCAR 13 08SEP90

NEW AO13  SCHEDULE FROM 17OCT90 AFTER MOVE TO  LON

180  LAT 0

MODE B  MA 000 TO 095

MODE JL MA 095 TO 125

MODE LS MA 125 TO 130

MODE S MA 130 TO 135

MODE BS MA 135 TO MA 140

MODE B  MA 140 TO 256

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 144

Note the non telemetry information at the end of the block. UO-2's

ASCII telemetry looks different, so WHATS-UP is told where to look

for the telemetry by the following items.

9.2.28.99.9 Spare

9.2.28.99.10 Number of Lines

This item is used in non packet telemetry to tell WHATS-UP how

many lines of data are in a frame.

9.2.28.99.11 First Line

This item is used in non packet telemetry to tell WHATS-UP which

line in the frame contains the first line of data.

9.2.28.99.12 Last Line

This item is used in non packet telemetry to tell WHATS-UP which

line in the frame contains the last line of data.

9.2.28.99.13 Header Lines

This item is used in non packet telemetry to tell WHATS-UP how

many lines there are in the header.

9.2.28.99.14 Character Count

In non packet telemetry this item tells WHATS-UP the maximum

number of characters on a line. Use it in BAUDOT,ASCII and CW

telemetry to stop the display overrunning the page and to force

recognition of the identifier and turn the automatic capture-to-disk

on.

9.2.29 Digital Telemetry Status Channels

The next set of lines instruct WHATS-UP how to display digital

telemetry status in the FO-20 frame. Digital status channels contain

a number of status points. FO-12 and 20 have three data elements in

each digital status channel. Typically, each line in the WHATS-

UP.SYS file contains ten items in the following format.

Status_Channel,     Status_Display_Page,     Status_Text,

Status_Bit_Mask,    Status_Row, Status_Col,  Status_ON_Text,

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 145

Status_OFF_Text,    Status_ON_Color,         Status_OFF_Color.

Each item is described in the following sections.

9.2.29.1 Status_Channel

This is the channel number of the digital status telemetry data in

the frame. Each entry must be two digits.

9.2.29.2 Status_Display_Page

This item is the Display page number. A 0 is a 'wild card' which

will be displayed on every page.

9.2.29.3 Status_Text

This is the text that is displayed in the page.

9.2.29.4 Status_Bit_Mask

This is the bit mask to mask out the position of the desired bit.

The masks are in decimal. i.e. 1, 10 and 100.

9.2.29.5 Status_Row

This item is the Display page row. It identifies which row in the

screen the data element will be displayed.

9.2.29.6 Status_Col

This item is the Display page column. It identifies which column in

the screen the data item will be displayed.

9.2.29.7 Status_ON_Text

This item is the text that is displayed when the spacecraft

telemetry contains a 1 value.

9.2.29.8 Status_OFF_Text

This item is the text that is displayed when the spacecraft

telemetry contains a 1 value.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 146

9.2.29.9 Status_ON_Color

This item is the color the text is displayed in when the spacecraft

telemetry contains a 1 value.

9.2.29.10 Status_OFF_Color

This item is the color the text is displayed in when the spacecraft

telemetry contains a 0 value.

These lines also terminate with an '*' character. A few typical

lines from a Fuji.CNF file are shown below.

30,3,JTA Power    :,100, 4, 1, ON  , OFF ,11,10

30,3,JTD Power    :, 10, 4,25, ON  , OFF ,11,10

30,3,JTA Beacon   :,  1, 4,45, PSK , CW  ,11,10

38,3,Solar Panel 3:,100,12, 1, LIT , DARK,11,10

38,3,Solar Panel 4:, 10,12,25, LIT , DARK,11,10

38,3,Solar Panel 5:,  1,12,45, LIT , DARK,11,10

Note the blanks in the Text ON and OFF positions. The blanks are

used to ensure that a word such as "LIT" which contains three

letters fully overwrites a word which contains four characters such

as "DARK".

9.2.30 Packet/Link Parameters

The next set of items are the Packet/Link Parameters configurations

(maximum = 16). If you want a value to show up in more than one page

(other than the wild card [0]) you must enter it twice (once per

page). Typically each row contains 10 items in the sequence shown

below.

Packet_title,  Packet_Type,   Packet_Lines,  Packet_Page,

Packet_Color,  Packet_Row,    Packet_Col,    Link_Page,

Link_Row,      Link_Col,      Binary_Byte_Count.

Each item is described in the following sections.

9.2.30.1 Packet_title

This item is the name of UNP address (e.g. TLM,WASH, BCXRT).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 147

9.2.30.2 Packet_Type

This item is used by WHATS-UP to define the type of telemetry. The

following assignments have been allocated but are not necessarily

used in this release of WHATS-UP.

0 AO-13 Non Packet Decimal telemetry.

1  AMSAT Microsat Packet Telemetry (TLM) with the format CC:DD

where CC is the hexadecimal channel number and DD the

hexadecimal data.

2  AMSAT Microsat Packet Telemetry, hexadecimal ASCII STATUS

telemetry.

3  Fuji Packet Telemetry format of decimal data in which the line

and the position on the line identify the channel. WHATS-UP

allows for up to 60 channels.

4  AMSAT Microsat Packet Telemetry hexadecimal Binary STATUS

telemetry.

5  AMSAT Microsat Binary TLM Packet Telemetry.

6  UO 1/2 ASCII Telemetry.

9.2.30.3 Packet_Lines

This item is the number of lines of text in the packet. For

example, the AMSAT TLM packets contain three lines, the WASH packets

contain only one.

9.2.30.4 Packet_Page

This item is the page that the raw contents of the packet will be

displayed on. A '0' is a wild card which will make WHATS-UP display

it on every page. By careful use of this item, you can display both

raw and decoded packet data on the same page.

9.2.30.5 Packet_Color

This item is the color that the raw packet data will be displayed

in.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 148

9.2.30.6 Packet_Row

This item is the row position that the raw packet will be displayed

in, on the selected page.

9.2.30.7 Packet_Col

This item is the column position that the raw packet will be

displayed in, on the selected page.

9.2.30.8 Link_Page

This item is the Display page for the cumulative count of the

packet type. The wild card '0' applies.

9.2.30.9 Link_Row

This item is the row position that the packet header will be

displayed in, on the selected page.

9.2.30.10 Link_Col

This item is the column position that the packet header will be

displayed in, on the selected page.

9.2.30.11 Binary_Byte_Count

This item is the number of bytes in a binary packet. As no standard

currently exists, we have to tell the computer how many bytes to

expect.

These lines also terminate with an '*' character.

9.3 Telemetry Channel Extraction File

The contents of this file are the defaults for extracting data from

the playback file. A typical set are shown below. WHATS-UP does a

string match, and looks for the first time that a particular string

occurs. You may thus use the contents of a time packet, or the time

mark in a header.

ZCZC (default start time string) {start of file}

NNNN (default stop time string) {end of file}

2F

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 149

35

38

The first line contains the start string. 'ZCZC' tells WHATS-UP to

start extracting at the beginning of the file. The second line

contains the stop string. 'NNNN' tells WHATS-UP to stop extracting

at the end of the file. The remaining lines are the individual

channels (uppercase letters), one channel per line.

9.4 Extracted Telemetry Data File

This an ASCII string, comma delimited file which can be imported

into your spreadsheet. The format of the file is such that each line

starts with a date code or packet header. Then each channel and the

datum associated with that channel follow for all channels in the

packet segment or non packet frame for each and every channel

displayed in the selected page on the screen. If the data from that

channel is not displayed on the screen in the extract mode, the data

will not be extracted.

A typical example of three lines from this file is shown below.

"01-Mar-91 03:39:46 DOVE-1*>TLM:","35", 6.7,"38", -9.7

"01-Mar-91 03:39:56 DOVE-1*>TLM:","35", 6.7,"38", -10.3

"01-Mar-91 03:40:06 DOVE-1*>TLM:","35", 6.7,"38", -10.3

9.5 Doppler File

Doppler data are stored in this file. If the file does not exist,

it is created when needed. If it does exist, data are appended to

the file. The first line of data identifies the time, place and

spacecraft as shown below.

"*** 22-Feb-91 02:57 Doppler Track STARTED for WEBER @ G3ZCZ"

The next line provides the headings for the five columns as

follows.

"Time","Doppler Mark","Frequency","Doppler Shift","Measured Shift"

9.5.1 The Time

The Time is obtained from the PC clock.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 150

9.5.2 The Doppler Mark

The Doppler Mark is a sequential count incrementing at each sample.

It can be used to provide an 'X' axis in a graph. Elapsed time will

be the Doppler mark multiplied by the Doppler sampling interval in

seconds.

9.5.3 The Frequency

The Frequency is the measured radio frequency.

9.5.4 The Doppler Shift

The Doppler Shift is the calculated/predicted Doppler Shift in kHz.

9.5.5 The Measured Shift

The Measured Shift is the difference (in kHz) between the measured

radio frequency and the beacon frequency in the configuration file.

These parameters should allow you to plot Doppler curves and

determine orbit parameters using the time of closest approach

technique.

Typical entries in the file are shown in the following lines.

"02:57:32", 2, 437110740, 8.1494,10.740

"02:57:37", 3, 437110740, 8.1354,10.740

"02:57:42", 4, 437110740, 8.1202,10.740

"02:57:47", 5, 437109860, 8.1045,9.860

"02:57:52", 6, 437110670, 8.0880,10.670

"02:57:57", 7, 437110620, 8.0709,10.620

"02:58:13", 8, 437110620, 8.0525,10.620

"02:58:18", 9, 437109780, 7.9868,9.780

.

.

"03:07:17", 115, 437092570, -7.7884,-7.430

"03:07:22", 116, 437092570, -7.8204,-7.430

"03:07:27", 117, 437092570, -7.8511,-7.430

"03:07:32", 118, 437092570, -7.8802,-7.430

"*** 22-Feb-91 03:07 Auto End"

The lines of data end with a termination statement.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 151

All text items are enclosed in double quotation marks ("). All

elements are delimited by comma characters (,).

If you plan to do Doppler measurements on the Microsats, then you

may be interested in the following information.

The following frequencies were taken after the completion and final

tuning of the completed transmitter assemblies. The measurements

were made at a temperature of 23 deg. C. As the frequency does

change with temperature the current frequency will be slightly

different.

PACSAT:   Normal PSK TX:        437.02625 MHz

Raised Cosine TX:     437.05130 MHz

S-Band TX:           2401.14280 MHz

DOVE:     FM TX No. 1:          145.82516 MHz

FM TX No. 2:          145.82438 MHz

S-Band TX:           2401.22050 MHz

WEBERSAT: Normal PSK TX:        437.07510 MHz

Raised Cosine TX:     437.10200 MHz

LUSAT:    Normal PSK TX:        437.15355 MHz

Raised Cosine TX:     437.12580 MHz

9.6 Kepler Element Files (*.TLE)

The following outlines the "NASA Two-Line" Keplerian data format.

Data for each satellite consists of three lines in the following

format:

Mir

1 16609U          91 36.87776287 0.00038608           39705-3 0  2481

2 16609  51.6077 232.9299 0024950 205.6681 154.3223 15.64092124284608

Line 1 contains an eleven-character name.

Lines 2 and 3 are the standard Two-Line Orbital Element Set used by

NASA and NORAD. The format description is:

Line 2  Column Description

01-01 Line Number of Element Data

03-07 Satellite Number

10-11 International Designator (Last two digits of launch

year)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 152

12-14 International Designator (Launch number of the

year)

15-17 International Designator (Piece of launch)

19-20 Epoch Year (Last two digits of year)

21-32 Epoch (Julian Day and fractional portion of the

day)

34-43 First Time Derivative of the Mean Motion or

Ballistic Coefficient (Depending on ephemeris type)

45-52 Second Time Derivative of Mean Motion (decimal

point assumed; blank if N/A)

54-61 Radiation pressure coefficient. (Decimal point

assumed)

63-63 Ephemeris type

65-68 Element number

69-69 Check Sum (Modulo 10) (Letters, blanks, periods =

0; minus sign = 1; plus sign = 2)

Line 3

Column   Description

01-01 Line Number of Element Data

03-07 Satellite Number

09-16 Inclination (Degrees)

18-25 Right Ascension of the Ascending Node (Degrees)

27-33 Eccentricity (decimal point assumed)

35-42 Argument of Perigee (Degrees)

44-51 Mean Anomaly (Degrees)

53-63 Mean Motion (Revs per day)

64-68 Revolution number at epoch (Revs)

69-69 Check Sum (Modulo 10)

All other columns are blank or fixed.

9.7 AMSAT Format Element File (*.AMS)

The AMSAT format file is the AMSAT file as received by means of

packet radio. WHATS-UP scans the text for the word "Satellite:".

When it finds it, it assumes that the next few lines carry the data

in the fixed order. You thus, do not need to edit the file, just

capture it to disk, and save with a filetype of 'AMS'. For example

the following file could be saved as ORBS-19.AMS.

If you need to enter data in by hand, edit an AMSAT format file,

such as this one.

4116] B   BID: ORBS-019.D

Date: 20 Jan 91 02:58:24 Z

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 153

From: N4QQ@N4QQ

To: ALL@AMSAT

Subject: Orbital Elements   019.MICROS

R:910120/0258z 18616@N4QQ.MD.USA [Silver Spring,Md]  Z:20901

From: N4QQ@N4QQ.MD.USA

To: ALL@AMSAT

HR AMSAT ORBITAL ELEMENTS FOR THE MICROSATS FROM N3FKV

HEWITT, TX      JANUARY 19, 1991

TO ALL RADIO AMATEURS BT

Satellite: AO-16

Catalog number: 20439

Epoch time:      91014.08680572

Element set:     188

Inclination:       98.6853 deg

RA of node:        94.6412 deg

Eccentricity:    0.0010690

Arg of perigee:   259.0593 deg

Mean anomaly:     100.9381 deg

Mean motion:   14.28942714 rev/day

Decay rate:       3.95e-06 rev/day^2

Epoch rev:            5098

Satellite: DO-17

Catalog number: 20440

Epoch time:      91014.07510019

Element set:     187

Inclination:       98.6867 deg

RA of node:        94.6574 deg

Eccentricity:    0.0010787

Arg of perigee:   258.7019 deg

Mean anomaly:     101.2949 deg

Mean motion:   14.29004143 rev/day

Decay rate:       4.60e-06 rev/day^2

Epoch rev:            5098

9.8 Spacecraft Operations File

This is a text file which can usually be obtained from a packet

radio bulletin. You may create one with the editor, if you can't

find one on a packet radio BBS. A Typical file is shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 154

AO-13 TRANSPONDER SCHEDULE

Mode-B : MA 060 to MA 165 :

Mode-JL: MA 165 to MA 190 :

Mode-LS: MA 190 to MA 195 :

Mode-S : MA 195 to MA 200 : <= Mode B is Off - no swishing!

Mode-BS: MA 200 to MA 205 : <= QRP on BS please.

Mode-B : MA 205 to MA 256 :

Omnis  : MA 240 to MA 060 :

This schedule is expected to continue through 27 March 91

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 155

10.0 Glossary

AMSAT     The Radio Amateur Satellite Corporation

AO        AMSAT-OSCAR

AOS       Acquisition of Signals

ARRL      American Radio Relay League

CCD       Charge Coupled Device

DO        DOVE-OSCAR

DOVE      Digital Orbiting Voice Encoder, also used interchangeably

with DOVE-OSCAR or DO

EWT       Early Warning Time

FM        Frequency Modulation

FO        Fuji-OSCAR

FSK       Frequency Shift Keying

LO        LUSAT-OSCAR

LOS       Loss of Signals

OSCAR     Orbiting Satellite Carrying Amateur Radio

PSK       Phase Shift Keying

RTTY      Radio Teletypewriter

SSB       Single Side Band

TNC       Terminal Node Controller

TU        Terminal Unit (Radio Modem)

UO        UoSAT-OSCAR

WO        WEBER-OSCAR

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 156

11.0 References and Further Reading

The best book is "The Space Radio Handbook" by John Branegan,

GM4IHJ. It is published by the Radio Society of Great Britain. The

book is available from the RSGB, Lambda House, Cranborne Rd, Potters

Bar, Herts., England EN6 3JE. The price is 13.34 pounds sterling.

John has also written several booklets on MIR, DOVE and UOSAT-2

specifically for the educator and beginner. He can be reached at 8

Whitehills, Saline, Fife, Scotland KY12 9UJ. (Send a SASE or IRCs

please)

Other books and articles of interest are listed below.

Space Radio Handbook, John Branegan, GM4IHJ, Radio Society of Great

Britain.

Satellite Experimenter's Handbook, Martin Davidoff, K2UBC, ARRL

1990, 2nd Edition.

A Turnstile Antenna for Two Meters, Joe Kasser, G3ZCZ, 73 Magazine,

June 1978.

Antennas for Microsat Ground Stations, Dick Jansson, WD4FAB, The

AMSAT Journal, Volume 13, Number 1, March 1990.

Satellite Antennas from Recycled Junk, Howard Sodja, W6SHP,

Proceedings of the OSCAR Seminar, September 29 & 30, 1990. Available

from Project OSCAR Inc.

Microcomputer Processing of UoSat-OSCAR 9 Telemetry, Robert J.

Diersing, N5AHD, The Satellite Anthology, Pages 46-51, ARRL, 1988.

The First Flock of Microsats, Tom Clark, W3IWI, Jan King, W3GEY,

Bob McGwier, N4HY and the AMSAT team, The AMSAT Journal, Volume 12

Number 1, May 1989.

Ariane Launch Vehicle Malfunctions - Phase 3A Spacecraft Lost!, Tom

Clark, W3IWI, Joe Kasser, G3ZCZ, Orbit Magazine, Volume 1 Number 2,

June/July 1980.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 157

12.0 Change History

1.00 (4/01/91) Initial Release.

1.10 (7/15/91) Converted to TP 6.0. Break procedure added on TNC

port. Radio port parameters added to WHATS-UP.SYS. Autotrack

added. Default radio freq/mode added. TNC and Radio Serial

Port values 0 added to allow WHATS-UP to run in a multi-

tasking environment without interfering with the serial ports.

Directory path changed to apply only to capture-to-disk files.

Debug Menu added. Radio Menu changed. CW telemetry automatic

capture-to-disk added (for A-O 21).

1.20 (11/08/91) SARA, TNC1, TNC2, PK-88, KAM and MFJ-1278 added.

1.30 (07/04/94) DSP-2232, PK-900 added. SAREX/MIR Attack Mode and

logging added. TLE and CNF nomenclature adopted.

1.40 (01/01/96) Mutual visibility mode, sunlight and visible

indicators, CW terminal functions, parallel port interface for

memory up/down control, mission elapsed time (MET) window

added. Julian dates for multi-year functions. Several

additions in various menus.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 158

13.0 Obtaining Further Information

For further information about any of the spacecraft and the Radio

Amateur Satellite program, photocopy and mail the following form

together with a self addressed stamped envelope (SASE) to :-

To: [ ] Project OSCAR Inc. POB 1136, Los Altos, CA. 94023-1136.

[ ] AMSAT-UK, 94 Herongate Road, Wanstead Park, London E12 5EQ.

Telephone (081) 989 6741.

I read about the Radio Amateur Satellite program in WHATS-UP and am

interested in it. Please send me further information about the

program, and details of membership in your organization.

CALL ________________ TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY ___________________________________________

STATE _________ POSTCODE ______________________

Comments and questions ...

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 159

14.0 Other Products by Joe Kasser, W3/G3ZCZ

14.1 PC-HAM 3.52

PC-Ham contains the following suite of programs.

14.1.1 LOGBOOK

Full blown logging package. With automatic check of logs for awards

such as DXCC. Allows you to recall any entry by call sign within

seconds. Indexed displays, QSLing, Contest mode QSLing (prints the

lot) and lots more. Although written in dBASE3 the package contains

a compiled version (LOGBOOK.EXE), so you don't need dBASE to run it.

The source code is ONLY given to registered users. Ideal for DX-

peditions or for DX robot users to handle QSLing and log statistics.

14.1.2 CONTEST

Keeps Dupes in memory, logs QSO's to disk in format which can be

processed by the LOGBOOK package. Now compiled in Turbo BASIC,

source code is supplied so that you can modify the program to meet

your requirements.

14.1.3 CQSS

Sweepstakes game compiled in Turbo BASIC. Work the ARRL Sweepstakes

contest on your computer. You are located just outside Washington

DC. A propagation model is built in to the program. This program is

REQUIRED training for all sweepstakes operators. Earlier version of

the program is described in detail in Software for Amateur Radio by

Joe Kasser G3ZCZ, published by TAB Books, Blue Ridge Summit, PA.

17214.

14.1.4 WHATSON

Predict HF Propagation for given days. Contest mode with printout

to whole world at hourly intervals. Needs BASIC.

14.2 STARTREK The Computer Program

An ideal task for the beginner to learn a language on, is a

simulation game which is written to run on a computer that the

beginner has access to. For in that case, there is complete control

of all inputs and outputs. This kind of game (in which the player

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 160

makes decisions based on the information available to him or her

available at the time), can be made sufficiently sophisticated and

complex so as to make writing it an adequate challenge for anybody.

The techniques used in writing a good game are the same that

programmers use in professional activities. Writing a good game

poses a challenge that allows you to develop good habits and

techniques for programming and also allows you to learn a language

in an interesting manner. By taking an orderly approach to the game

design, complex operations may be clearly understood and converted

to computer code with the aid of a language reference manual,

irrespective of the language being used.

This product teaches the techniques for writing such a game using

the STARTREK game as an example, and the BASIC language as the

programming language in which to write the code.

Registration fees.

Single Copy        $15.00

10 - 50 Copies     $12.00 per registered copy.

50 - 100 Copies    $10.00 per registered copy.

100 + Copies       $8.00 per registered copy.

14.3.LAN-LINK 2.34

LAN-LINK is a Personal Packet Terminal Program for the TNC1, TNC2,

KPC-2 and most of all a smart multi-mode digital communications

controller for the PK-900, DSP-2232, KAM, MFJ1278 and the PK-232.

LAN-LINK is designed to optimize the configuration of the TNC in

each communications mode and to provide some smart terminal

features. LAN-LINK is a sophisticated program. In its basic state it

allows you to use the TNC in an optimal manner. It configures the

TNC (it types the commands) for you to maximize the communications

efficiency in the communications mode of your choice. That means,

for example, when working Packet on HF you need to program the TNC

parameters to different values than you would use on VHF to make

maximum use of the mode. One significant difference is the length of

the packet itself, for the longer it is, the greater the probability

of QRM destroying it. This program will adjust the packet parameters

for you.

PROGRAM HIGHLIGHTS: Function key and Menu driven. Expert system

(ELMER) in packet mode for building smart servers. Automatic logbook

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 161

entries for Packet and Mailbox/Beacon Mode AMTOR Connects,

semiautomatic logbook entries for other modes. Logbook file is dBASE

compatible and can be processed by the LOGBOOK Package of PC-HAM by

G3ZCZ for indexed listings, tracking of DXCC and other AWARDS, etc.

Contest operation, sends standard message and automatically

increments QSO count. Automatic optimized configuration of the TNC

for each communications mode. All mode Function key 'OVER' feature

(End). There are 60 files with fixed names (LAN-LINK.001 through

LAN-LINK.010) which may be viewed and transmitted by means of

function keys. They may also be edited from the Edit Menu. Set up of

TNC for AMSAT-OSCAR Telemetry reception. Time display and event

scheduler. ASCII Text Editor. Customizable Colors.

Access to the TNC Command Mode is provided in case the user wishes

to override any defaults. Automatic capture to disk of all packet

radio connects. Automatic indication of the number of Packet

connects. Local Area Network (LAN) message store and forward

capability. Capable of automatic connect attempts to download a QTC

from another station in the LAN. Capable of automatic connect

attempts to a packet BBS to download your incoming messages, when

your callsign appears on the BBS mail beacon annunciator. Capable of

automatically requesting Bulletins on subjects that interest you

from your local packet BBS. Digipeat monitoring and capture. Alert

signal to let you know when a predetermined call shows up in a

packet header on frequency. Conference and Bridge modes in

multiconnect situations. Indicator that a specific station

designated as the 'target' call connected in Packet Mode, or linked

to AMTOR Beacon/Mailbox while you were away. Automatic NET/ROM and

KA Node path set up from LAN-LINK.DIR call/path directory file.

Selective answering machine and MAILBOX using NC/L command dialogue.

Automatic Beacon Mode CQ caller. Automatic contest (DX-pedition)

mode. Will call CQ repetitively and either work the connect and keep

going after disconnect or signal you when a reply is received.

Zmodem binary file transfer capability. RTTY SELCAL.

14.4 ELMER 1.00

ELMER is an Expert System Based on a Finite State Machine. There

are two versions of ELMER supplied with LAN-LINK. One is built in

for use in packet mode communications, when connected to someone

else. The second version is a stand alone version for use in

developing the text files and the logic. The stand alone version

lets you program and debug your own personal ELMER by yourself. The

stand alone version is a separate product and requires separate

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 162

registration.

14.5 BASIC PACKET RADIO

Basic Packet Radio explains how the computer can be used to smarten

up your use of packet. Not only does the book explain packet radio,

it comes with a full feature version of LAN-LINK (for the PC) on

disk and contains the manual for the program.

The topics covered in the book are outlined below.

Getting Started in Packet Radio, What Packet Radio Is, The Shared

Radio Channel, The Virtual Channel, The Equipment Needed to Hold a

Packet QSO, The TNC, TNC Modes, The Terminal Program.

Connecting the PC to the TNC, Command Convention, Making the PC

Communicate with the TNC, Determining and Changing the State of a

Parameter, Setting the Carriage Returns and Line Feed Display

Controls, Flow Control, Connecting the TNC to the Radio, Delays in

the Transceiver.

The Local Area Network, How TNCs Communicate, Controlling the

Display of Monitored Packets, Setting Your TNC to Display or Ignore

Packets, The Wireless LAN, Use of Packet Beacons, The Alert Call,

The Target Call, LAN Answering Machine, Mail Beacon (Annunciator),

Remote Beacon Shutdown.

Connecting to Someone, Connecting to Other Stations the LAN-LINK

Way, The Point and Shoot (MH List) Way, Using the Call Menu, Loop

Backs, Names, Handles and Paths, Connecting via NET/ROM and TheNet,

Connecting via KA-Nodes, The SAREX Call and Working MIR.

Extending the Range, Digipeating, Using NET/ROM and TheNet Nodes,

Using G8BPQ Nodes, Using KA-Nodes, Using MSYS Nodes, Using ROSE

Switches, Using LAN-LINK to Communicate via Nodes, Path

Determination to a DX Station, Finding Packet Signals, Using

Communication Satellites.

The Packet Bulletin Board System, How to Connect to the PBBS,

Connected to the PBBS, Connected, But What Next ?, Telling it Your

Name, Getting Information about the PBBS Itself, The Command Prompt

Line, Changing the Prompt Line, Listing the Messages and Bulletins,

Reading Messages, Sending Messages, Sending Messages Around the

World, Sending a Bulletin, Forwarding Bulletins, Sending NTS

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 163

Traffic, The Parts of the NTS Message, Killing Messages, Using LAN-

LINK to Send NTS Traffic, Handling NTS Traffic, Files, Talk to the

SYSOP, Connecting Elsewhere via the PBBS, Obtaining On-Line Help,

Logging Off the PBBS, The F6FBB PBBS Features, Automating Message

Reading, Message Headers.

The PacketCluster, Summary of commands and features, Smartening Up

PacketCluster Access With LAN-LINK.

Internet wormholes and how to access them.

Other Applications of Packet, Data Base Servers, Using a REQFIL

Server, Using a REQDIR Server, Using a REQQTH Server, Electronic

Newspapers, Non Real-Time QSOs, Automatic Beacons, Robots and

Contests, Propagation Research, Transmission Control

Protocol/Internet Protocol (TCP/IP).

This is a book to read once, then use as a reference over and over

again. If you decide you don't like the book, send it back within 30

days for a swift refund.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 164

15.0 How Shareware Works

The Association of Shareware Professionals (ASP) has established

standards for its members and for any organization which has "ASP

Approved" status. The ASP wants to make sure the shareware principle

works for you. If you are unable to resolve a problem with an ASP

member or organization (other than technical support), the ASP may

be able to help. Please write to

The ASP Ombudsman, 545 Grover Road, Muskegon, MI. 49442-9427, USA.

You are encouraged to copy the floppy disk and share it freely with

others. You have the luxury of trying out the product at your own

pace and in the comfort of your own home or workplace.

After you have used the material for a reasonable evaluation period

(30 days), you should either discontinue use of the material or

register your copy. Your support is important and greatly

appreciated. With it, Shareware authors are encouraged to design and

distribute new products. Without it, a great deal of high quality,

low cost software will cease to be available.

Why pay at all?

You receive support from the author. You receive a CURRENT copy of

the program. Your input and ideas help shape future products. You

have a sense of pride and ownership in having honestly participated

in the Shareware revolution. You help to keep software prices down

by supporting a distribution method which doesn't depend on

expensive advertising campaigns.

Be aware of the following restrictions, designed to protect the

community of Shareware users and to prevent greedy people from

taking unfair advantage of the trust, hard work and good will of

Shareware authors.

1. No price or consideration may be charged for the material.

However, a distribution cost may be charged for the cost of the

diskettes, shipping and handling, not to exceed $6.

2. The files and programs on the disks may not be modified or

deleted.

3. The material cannot be sold as part of some other more inclusive

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 165

package.

4. The material cannot be "rented" or "leased" to others.

5. The end user must be told clearly in writing on the outside of

the package and in all advertising that the diskette(s) are

"Shareware."

6. The package must contain a written explanation that the disk is

for evaluation purposes, and that an additional "registration

fee" is expected by the author, if the material is used beyond

an initial evaluation period.

7. In the case of distribution via any telecommunications link, the

following must be done:

* An error checking protocol must be used.

* The individual files must be combined into, and transferred in

a library or archive format.

8. Shareware distribution is permitted only in the United States,

Canada, England, and Australia.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 166

Appendix 1 WHATS-UP 1.40 REGISTRATION FORM

PLEASE COPY this form and MAIL to

Software for Amateur Radio, P.O. BOX 3419, SILVER SPRING, MD 20918.

CALL ________________ TODAY'S DATE ________________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________________

CITY ______________________________________________

STATE _________ POSTCODE ____________ TNC TYPE_____

FIRMWARE REV ____ HOME BBS ______________

DISK SIZE 5.25 _____ 3.5 ____

Please register me as a user of WHATS-UP. I am currently using

WHATS-UP Version ______ which I obtained from _______. Please send

me the latest version of WHATS-UP or if a more recent one does not

exist at this time, QSL my registration and add my name to the list

to receive a free update when it becomes available.

I also enclose an additional amount for evaluation copies of ELMER

_, LAN-LINK _, PC-HAM _ and Startrek TCP _ ($5 for 1 program, $15.00

for the set). If I like them, I plan to register them in due course.

Basic Packet Radio Book (29.95)                       __.__

Shipping for book $3.50 US/Canada, $10.00 overseas    __.__

WHATS-UP Registration ($45.00)                        __.__

LAN-LINK Registration ($45.00)                        __.__

ELMER Registration ($45.00)                           __.__

Evaluation software                                   __.__

____________________________________

Maryland Residents please add 5% sales tax.           __.__

Total Enclosed                                        __.__

MasterCard/Visa Number ___________________________Expires __/__

Many additions come into WHATS-UP as a result of user suggestions,

so here's your chance to get some input in. Write them on this piece

of paper.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 167

Index

#�#, 50

AFSK, 9, 75, 108, 118, 124, 130

Alt-A, 6, 45

Alt-B, 3, 23, 24

Alt-C, 3, 23, 24, 46

Alt-D, 3, 23, 24

Alt-E, 6, 46

Alt-F, 3, 23, 24

Alt-H, 6, 46

Alt-I, 3, 23, 24

Alt-J, 3, 23, 24

Alt-P, 3, 23, 25

Alt-S, 3, 6, 23, 25, 46

Alt-U, 6, 46

Alt-X, 3, 6, 23, 25, 47

Altitude, 8, 14, 19, 21, 53, 57, 58, 108, 125, 127

AOS, 4, 5, 8, 18, 19, 20, 21, 43, 62, 129, 131, 134, 135, 138, 156

Ariane, 105, 110, 157

Arrows, 9

ARSENE, 8, 111, 115, 116

ASCII, 5, 9, 2, 3, 7, 8, 13, 28, 39, 40, 41, 43, 61, 64, 66, 69, 74,

75, 98, 102, 124, 126, 130, 135, 143, 145, 148, 150, 162

Astronomy, 105, 106

Autotrack, 3, 9, 7, 21, 22, 23, 24, 132, 134, 158

BAUDOT, 3, 40, 61, 64, 66, 135, 145

Beacon, 5, 9, 10, 2, 4, 7, 19, 24, 39, 40, 41, 48, 49, 61, 70, 76, 80,

94, 95, 97, 98, 100, 101, 104, 105, 108, 111, 115, 116, 118, 119,

123, 124, 130, 131, 132, 133, 134, 137, 138, 143, 147, 151, 162, 163

Branegan, 157

Capture-to-disk, 4, 5, 6, 13, 18, 23, 31, 32, 33, 41, 126, 127, 133,

145, 158

CNF, 6, 7, 8, 10, 16, 30, 31, 38, 39, 40, 42, 126, 127, 131, 132, 133,

147, 158

Co-axial, 11

Color, 5, 8, 9, 5, 16, 17, 18, 19, 20, 41, 43, 125, 126, 128, 129, 132,

138, 146, 147, 148

CW, 2, 4, 5, 6, 9, 4, 9, 11, 26, 27, 37, 39, 40, 43, 49, 50, 51, 76,

91, 92, 94, 95, 96, 97, 98, 100, 104, 105, 118, 119, 120, 126, 130,

132, 135, 145, 147, 158

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 168

Delay, 5, 10, 36, 37, 50, 109, 133, 137

Digipeater, 73, 94, 111, 117, 118, 137

Doppler, 3, 4, 9, 10, 2, 9, 19, 23, 24, 31, 32, 36, 126, 130, 132, 134,

150, 151, 152

DOVE-OSCAR, 2, 4, 62, 134, 156

Downconverter, 64, 68

Downlink, 7, 3, 4, 11, 61, 67, 69, 76, 84, 94, 95, 107, 108, 111, 118,

119, 122

Drag, 35, 52, 57

Earth-Sun, 54

EWT, 10, 19, 20, 21, 43, 129, 131, 137, 138, 156

Extract, 4, 8, 5, 20, 28, 125, 127, 128, 150

Fades, 3, 65, 66

FO, 4, 61, 64, 69, 92, 93, 94, 95, 102, 119, 131, 133, 139, 145, 156

Footprint, 21

Fuji-OSCAR, 7, 4, 7, 62, 92, 95, 96, 97, 98, 99, 100, 101, 102, 103,

104, 133, 134, 135, 156

Greek, 50

Hop, 21

Icom, 135, 136

Inclination, 7, 35, 53, 55, 62, 93, 111, 116, 153, 154

Index, 168

JAS, 93, 95, 98, 101, 102, 143

JBS, 94, 95, 101, 133, 143

Jupiter, 106, 107, 110, 111

Kepler, 8, 10, 54, 125, 127, 152

Keplerian, 4, 9, 15, 19, 28, 31, 33, 34, 131, 132, 134, 152

Keyboard, 6, 4, 9, 10, 15, 43, 49, 50

Keyer, 9, 51

Keystroke, 49

KPC, 161

LAN-LINK, 11, 4, 18, 42, 44, 45, 46, 161, 162, 163, 164, 167

Laptop, 9, 136

Latitude, 8, 13, 21, 93, 125, 126

Launched, 58, 59, 70, 73, 76, 84, 92, 105, 111, 116

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 169

Line-of-sight, 11

Logbook, 6, 9, 10, 4, 9, 44, 45, 46, 47, 125, 126, 129, 130, 160, 161,

162

Longitude, 8, 13, 21, 125, 127

LPT, 10, 133, 135, 136, 138

LSB, 37, 80, 81, 82, 83, 96, 97, 100, 104, 135

LUSAT, 2, 84, 85, 89, 91, 92, 131, 152

LUSAT-OSCAR, 2, 4, 62, 134, 156

Mheard, 123

MicroSat, 5, 2, 6, 14, 39, 40, 58, 84, 143, 148, 157

Milliseconds, 50, 51

MIR, 2, 6, 10, 4, 9, 15, 20, 21, 47, 48, 49, 130, 133, 137, 138, 152,

157, 158, 163

Mode-B, 78, 79, 155

Mode-BS, 155

Mode-J, 82, 83

Mode-JD, 94

Mode-JL, 79, 155

Mode-L, 79, 80, 83

Mode-LS, 155

Mode-S, 79, 80, 82, 83, 155

Morse, 5, 3, 4, 21, 39, 40, 43, 50, 51, 82, 91, 95, 129

Multi-mode, 2, 64, 128, 161

Multi-task, 50

Multi-tasking, 42, 158

Multi-year, 158

North, 19, 55, 116

Orbits, 3, 4, 6, 11, 26, 27, 33, 53, 58, 62, 63, 66

PACSAT, 2, 85, 131, 152

QTH, 9, 126, 130

RAAN, 7, 35, 55, 56

Range, 8, 3, 11, 14, 18, 19, 20, 62, 64, 65, 67, 76, 98, 107, 113, 125,

129, 163

Ranging, 82, 106

Re-entered, 59, 73

Real-time, 5, 18, 21, 23, 26, 28, 74, 138, 164

Receiver, 3, 9, 4, 5, 10, 12, 23, 24, 36, 64, 65, 66, 67, 68, 78, 79,

107, 108, 121, 133, 134, 135

COPYRIGHT Joe Kasser, G3ZCZ 1996.

WHATS-UP.DOC Release 1.40 Page 170

RS, 5, 13, 14, 15, 20, 41, 64, 66, 120, 121

SARA, 5, 7, 39, 41, 69, 105, 106, 107, 108, 110, 111, 158

SARA-OSCAR, 7, 62, 105

SAREX, 2, 4, 6, 9, 10, 4, 9, 26, 27, 42, 47, 48, 49, 125, 129, 133,

135, 137, 138, 158, 163

Segment, 138, 139, 141, 142, 143, 144, 150

Sidereal, 110

SSB, 11, 12, 45, 63, 94, 156

Stream, 9, 126, 131

Sub-satellite, 21

Sun, 4, 9, 33, 34, 35, 54, 55, 56, 59, 60, 70, 73, 81, 82, 85, 93, 106,

107, 108, 109, 110, 126, 130

Telemetry, 2, 7, 8, 10, 2, 3, 4, 5, 6, 15, 16, 17, 20, 23, 32, 40, 41,

42, 61, 65, 66, 69, 70, 71, 73, 74, 75, 76, 77, 78, 84, 85, 86, 87,

89, 91, 94, 95, 96, 98, 99, 101, 102, 103, 104, 107, 108, 111, 113,

115, 116, 117, 118, 119, 120, 121, 122, 124, 125, 127, 133, 138, 139,

140, 142, 143, 144, 145, 146, 147, 148, 149, 150, 157, 158, 162

Temporary, 51

Transceiver, 11, 12, 163

Transients, 10

Transverter, 12

Transverters, 12

TTL, 9

UoSAT-OSCAR, 4, 40, 156, 157

Uplink, 11, 94, 95, 108, 111, 118, 119, 123, 124, 135

Vector, 54, 82

Viewer, 130

WEBER-OSCAR, 2, 4, 62, 134, 156

Zap, 6, 48, 49

COPYRIGHT Joe Kasser, G3ZCZ 1996.

#####